3GPP TSG-SA WG6 Meeting #60	S6-241262

Changsha, China 15th April – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0066 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SEALDD server discovery and selection procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For current procedure of SEALDD server discovery and selection,there are some issues in incomplete description, which may casue the misunderstanding. The main change is summaried as following: - The VAL server can use the discovered/selected SEALDD server information (e.g. through CAPIF, or EES notification) to perform SEALDD data transmission subscription with the corresponding SEALDD server. The SEALDD data transmission subscription is not limited to the regualr transmission subscription, and can be other transmission subscription such as URLLC transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to correct the description in SEALDD server discovery and selection procedure to make it more complete. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause the misunderstanding about the usage of SEALDD server discovery and selection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.2.2, 9.4.3.2.1, 9.4.3.2.2, 9.4.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.4.2.2	Procedure

The SEALDD server and VAL server may support CAPIF as shown in Figure 9.4.2.2-1. When CAPIF is supported:

-	The SEALDD server shall support the CAPIF API provider domain functions (i.e. CAPIF-2/2e (SEALDD-S interface), CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e as specified in 3GPP TS 23.222 [3]);

-	The VAL server shall act as API invoker and support the API invoker functions (i.e. CAPIF-1/1e and CAPIF-2/2e (SEALDD-S interface) as specified in 3GPP TS 23.222 [3]); and

-	The SEALDD server shall act as API invoker and support the API invoker functions (i.e. CAPIF-1/1e and CAPIF-2/2e (SEALDD-E interface) as specified in 3GPP TS 23.222 [3]).

Figure 9.4.2.2-1: SEALDD adaptation in the CAPIF architecture

The VAL server can discover a proper SEALDD server from CAPIF core function with different discovery filters, e.g. expected AEF location. If a VAL server is changed during UE mobility, a new SEALDD server may be discovered and selected.This is also applicable for the VAL server acting as EAS to discover and select an SEALDD server to use SEALDD-S services, if any.

* * * Next Change * * * *

9.4.3.2.1	VAL server registered to EES with associated SEALDD server address as VAL server endpoint

Figure 9.4.3.2.1-1: VAL server registered to EES with associated SEALDD server address as VAL server endpoint

1.	The VAL server can discover and select the SEALDD server (e.g. by CAPIF functions).

2.	The VAL server decides to use SEALDD service to enhance data transmission and send a SEALDD data transmission subscription request (e.g. SEALDD enabled regular transmission request in clause 9.2.2) to the SEALDD server.

3.	Upon receiving the request, the SEALDD server performs an authorization check and responds with a SEALDD data transmission subscription response (e.g. SEALDD enabled regular transmission response in clause 9.2.2).

4.	The VAL server (as an EAS) registers to the EES as described in clause 8.4.3.2.2 of 3GPP TS 23.558 [10] with the associated SEALDD server address as EAS Endpoint in the EAS profile. The EAS ID used by VAL server in registration may indicate the application service association between the VAL service and SEALDD service implicitly or explicitly.

This procedure reuses the current procedure described in clause 8.4.3.2.2 of 3GPP TS 23.558 [10]. The VAL server (as an EAS) registers to the EES with the associated SEALDD server address as the EAS Endpoint in the EAS profile. Then the EEC can use the EAS discovery procedure defined in clause 8.5 of 3GPP TS 23.558 [10] to find the VAL server's address which is SEALDD server's address. The VAL client can initiate SEALDD service via SEALDD client with the SEALDD server's address. This procedure can be used for scenario (a).

* * * Next Change * * * *

9.4.3.2.2	EAS registered to EES with associated SEALDD server information

Figure 9.4.3.2.2-1: EAS register to EES with associated SEALDD server information

1.	The SEALDD server (as an EAS) registered to EES with SEALDD profile (i.e. EAS profile) as an API provider.

2.	The EAS (VAL) server can discover and select the SEALDD server (e.g. by CAPIF functions).

3.	The EAS (VAL server) decides to use SEALDD service to enhance data transmission and send a SEALDD data transmission subscription request (e.g. SEALDD enabled regular transmission request in clause 9.2.2) to the SEALDD server.

4.	Upon receiving the request, the SEALDD server performs an authorization check and responds with a SEALDD data transmission subscription response (e.g. SEALDD enabled regular transmission response in clause 9.2.2).

5.	The VAL server (as an EAS) registers to the EES as described in clause 8.4.3.2.2 of 3GPP TS 23.558 [10] with the associated SEALDD server information (i.e. SEALDD service and SEALDD server address) as associated EAS ID and EAS Endpoint in the EAS profile.

This procedure reuses the current procedure described in clause 8.4.3.2.2 of 3GPP TS 23.558 [10]. The VAL server (as an EAS) registers to the EES with the associated SEALDD server information (i.e. SEALDD service and SEALDD server address) as associated EAS ID and EAS Endpoint in the EAS profile. Then the EEC can use the EAS discovery procedure defined in clause 8.5 of 3GPP TS 23.558 [10] to find the VAL server's address and associated SEALDD server's address. The VAL client can initiate SEALDD service via SEALDD client with the SEALDD server's address. For scenario (b), the VAL client can also establish the connection with VAL server using the VAL server's information for application signalling transfer and only use the SEALDD connection for application data transfer.

* * * Next Change * * * *

9.4.3.2.3	VAL server and SEALDD server registered to EES

Figure 9.4.3.2.3-1: VAL server and SEALDD server registered to EES

1. The VAL server (acting as an EAS) registers to the EES.

2. The SEALDD server (acting as an EAS) registers to the EES.

3. The EEC performs service provisioning procedure as described in clause 8.3.3.2.2 of 3GPP TS 23.558 [10]. The EEC is responded with the EES which supports the SEALDD server and the VAL server. The EEC sends the EAS discovery request for EAS bundle including VAL service and SEALDD service. The EES may collect the performance of VAL server and SEALDD server (e.g. E2E latency between the SEALDD server and client and load information of SEALDD and VAL server) from the ADAE server when receiving the EAS discovery request, as specified in clause 8.2.2 (for transmission quality, e.g. E2E latency) and 8.8.2 (for edge load) of 3GPP TS 23.436 [13].

4. The EES identifies the VAL server and the associated SEALDD server based on the performance of VAL server and the performance of SEALDD server (e.g. to satisfy the AC service KPI).

5. The EES sends the EAS discovery response to EEC, including the SEALDD server address, the VAL server address. The EES may send the selected SEALDD server information (i.e. the SEALDD server address) to the VAL server, then the VAL server performs a SEALDD data transmission subscription (e.g. SEALDD enabled regular transmission request/response in clause 9.2.2) to the SEALDD server.

Upon receiving the SEALDD server address, the VAL server address, the VAL client can initiate SEALDD service via SEALDD client with the SEALDD server's address. The data transmission between the VAL client and the VAL server can be enabled via the SEALDD connection, as specified in clause 9.2.2.