3GPP TSG-SA6 Meeting #60	S6-241302

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.435 | CR | 0022 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to Slice requirement verification and alignment capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSCALE, TEI19 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure defined in clause 9.16 sends notifications of alignment to the VAL server for every situation. There is no programmability in allowing or blocking the notification. The reason to have programmability is to solve the issue of large number of notifications, false notifications arising from the dynamic behaviour of the network like varying wireless channel. For example, because of some temporary issues like blockage of signal by a truck, fading etc, the signal strength decreases and leads to increase latency. This could trigger false alignment notification. The VAL server may configure the reporting of alignment notification based on time and space. For example, a UE travelling under a tunnel will lead to large number of notifications, the VAL server can configure to restrict the notifications based on location. Another example is during night time or when the UE/network acitivty is less, the VAL server can confiure to restrict in order to reduce the number of notifications based on time. Another example is Ambulance UE, the VAL server can configure to receive notifications from specific locations on the way the Ambulance is travelling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding time and space to the alignment reporting criteria. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current procedure could lead to a wastage of resources in reporting unnecessary alignment notifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.16.2.1, 9.16.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 9.16	Slice requirements verification and alignment capability exposure

### 9.16.2	Procedure

#### 9.16.2.1	Procedures on slice requirements verification and alignment capability exposure

Figure 9.16.2.1-1 provides a possible procedure of slice requirements verification and alignment capability exposure.

Pre-conditions:

1.	The NSCE Server has subscribed to the service of network slice performance management provided by EGMF.

Figure 9.16.2.1-1: Slice requirements verification and alignment process

1.	The VAL server sends slice requirements verification and alignment subscription NSCE server to require the NSCE server to check whether the slice requirements (by configuring the attributes of serviceProfile) matches the real network slice usage status, the request may include the S-NSSAI, the ID of the VAL server, the slice requirements parameters (attributes of serviceProfile) which is requested to be aligned. Optionally, the request includes the reporting criteria indicating when the NSCE server needs to send the notification. This alignment reporting allows the VAL server to configure the reporting based on location, number of UEs, and time information in order to send/block the reporting of alignment notification. The automatic enforcement mode indicates the NSCE server should automatically enforce the updated slice requirement.

2.	The NSCE server checks if the VAL server is authorized to trigger the service of slice requirements alignment.

3.	The NSCE server response to VAL server with the result of slice requirements verification and alignment request, e.g., the slice requirements alignment request is accepted.

4.	The NSCE server may optionally subscribe the QoE data from the NSCE client. The QoE data can refer to the data defined in clause 16, TS 26.114[27]. The NSCE server subscribes UE location monitoring using SEAL LM services defined in clause 9.3.11, clause 9.3.12 defined in TS 23.434

Note:	The QoE data collection from NSCE client follows the mechanism defined in EVEX work in SA4.

5.	The NSCE server subscribes the network slice related performance data and KPIs (e.g., the average PRB usage, the distribution of the PRB usage) defined in TS 28.552[19]. The NSCE server can aslo subscribe the analytics data (both statistics and predictions) from MDAS and ADAEs.

6a-6b.	NSCE server periodically receives the subscribed data (from steps 4 and 5) and compares the slice requirement parameters of network slice (the values of attributes of serviceProfile, e.g., radioSpectrum, the maxNumberofUEs) with network slice performance statistics (e.g., active number of users, the average PRB usage, the distribution of the PRB usage of the S-NSSAI) to generate the optimal slice requirements for the required service (represented by S-NSSAI) of the vertical.

This step depends on implementation (e.g., if in implementation specific time window the VAL client experience is satisfied while the network slice resources are with low utilization, the resources required in the slice requirements could be reduced based on predictions and statistics). When the reporting criteria is met like generated slice requirement is outside the given slice requirement range, the NSCE server send the notification to inform the VAL about the result of the verification and alignment. If automatic enforcement mode is indicated by the VAL server, the NSCE server enforces the slice optimization with the generated optimal slice requirement as the procedure defined in clause 9.12.2.1.2.

7.	Based on the analysis in step 6, the NSCE server periodically sends notifications to VAL server to inform the recommended changes of the slice requirements parameters. Optionally, if VAL server configures the reporting based on the alignment criteria then the NSCE server reports the notification to the VAL server e.g. based on the mentioned number of UEs are present in the slice, time and location information of the UEs.

#### 9.16.3.2	Slice requirements verification and alignment subscription

Table 9.16.3.2-1 describes information elements for slice requirements verification and alignment subscription.

Table 9.16.3.2-1: Slice requirements verification and alignment subscription

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | The identifier of the VAL service for which the requirement request applies |
| VAL UE ID list | O | The list of VAL UE IDs for which the requirement request applies |
| Slice identifier | M | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Slice Requirements List | M | The list of the slice requirements which need to be aligned. |
| >sliceRequirement | M | The requirement which need to be aligned, this parameter refers to the attribute of serviceProfile defined in TS 28.541 [10] |
| Reporting criteria | O | Indicate the notification reporting criteria, e.g. generated slice requirement is outside the given slice requirement range |
| Automatic enforcement mode | O | Indicates the mode for automatic enforcement of the generated optimal slice requirements to meet the slice requirements. E.g OFF or ON |
| Alignment reporting criteria | O | Indicates the alignment reporting conditions to the VAL server e.g. based on count of UEs, based on temporal (time information), based on spatial (location information) |
| Alignment reporting action | O | Indicate the alignment reporting criteria action mode (e.g. allow or block) |

* * * End of Changes * * * *