3GPP TSG-SA6 Meeting #60	S6-241330

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0083 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of references to other specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-17, references were added to SA2 spec. 23.287 "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services". Many references were incorrectly indicated as reference [8] (TS 23.682) instead of TS 23.287, which is reference [19]. In Rel-18, further references were added to SA2 spec. 23.287 "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services" but with [x] and not a number. Also, the Rel-18 version of TS 23.286 has two instances of Annex D. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (Rel-18 only) One instance of Annex D (Change history) renumbered to Annex E references to TS 23.287 corrected to [19] and added where missing. (Rel-18) one missing reference number ([21]) to TS 23.222 added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec. contains two instances of Annex D, incorrect and missing references to TS 23.287, TS 23.222 (Rel-18). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2.2, 9.6.4.1, 9.6.4.2, 9.7.4.2, 9.17.2.3, 9.20.3, 9.20.4, 9.22.2.1, 9.22.3.2, Annex D, Annex E (renumbered, was Annex D) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3GPP TS 23.286 V18.4.0 (2023-12)

*** First Change ***

#### 9.6.2.2	Set PC5 parameters request

Table 9.6.2.2-1 describes the information flow set PC5 parameters request from the VAE server to the VAE client.

Table 9.6.2.2-1: Set PC5 parameters request

| Information element | Status | Description |
| --- | --- | --- |
| V2X UE ID | O | Identity of the V2X UE to which the configuration is required |
| PC5 parameters configuration data | M | The PC5 parameters configuration data as specified in 3GPP TS 23.285 [5] and 3GPP TS 23.287 [19]. For PC5 provisioning in multi-operator V2X services (as specified in clause 9.15), these parameters may be the joint provisioning policies/parameters to be used by the V2X-UEs within the V2X service. |

*** Next Change ***

### 9.6.4	PC5 parameters provisioning

#### 9.6.4.1	General

This subclause describes the procedure for the VAE server provisioning the PC5 parameter provisioning information to the V2X UE via V1-AE reference point for V2X communication on PC5. The configuration information includes the configuration parameters specified in 3GPP TS 23.285 [5], 3GPP TS 23.287 [19] and additional application related configuration parameters.

#### 9.6.4.2	Procedure

Figure 9.6.4.2-1 illustrates the procedure for provisioning PC5 parameters to the V2X UE via V1 reference point.

Pre-conditions:

1.	The V2X UE has discovered and connected to the VAE server.

2.	If multicast delivery mode is used, the MBMS bearer being used is activated by the VAE server.

Figure 9.6.4.2-1: PC5 parameters provisioning

1.	The VAE server sends the set PC5 parameters request to the VAE client with the information of the PC5 parameters. The PC5 parameters information are specified in subclause 4.4.1.1.2 in 3GPP TS 23.285 [5] and in subclause 5.1.2.1 in 3GPP TS 23.287 [19]. This message can be sent via unicast or multicast.

2.	Upon receiving the set PC5 parameters request, the VAE client stores the received PC5 parameters in the V2X UE.

3.	The VAE client provides an acknowledgement by sending the set PC5 parameters response to the VAE server.

*** Next Change ***

#### 9.7.4.2	Procedure

Figure 9.7.4.2-1 illustrates the procedure where the VAE server sends notification of the network monitoring information to the V2X UEs.

Pre-conditions:

1.	The VAE server acting as a SCS/AS is configured with the SCEF information and is authorized to exchange information with the SCEF as specified in subclause 4.9 in 3GPP TS 23.682 [8]. The VAE server has subscribed to QoS notification service from 5GS (e.g. PCF/NWDAF). The notification may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [24] or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [19];

2.	The V2X UE1 and V2X UE2 have subscribed for the network monitoring information at the VAE server.

3.	The VAE server may be aware of the RAT type the V2X UE supports from the registration of VAE client.

Figure 9.7.4.2-1: Notifications for network monitoring information

1.	The VAE server communicates with the SCEF to receive network monitoring information as per the procedure in subclause 4.9 of 3GPP TS 23.682 [8], and/or communicates with NEF or NWDAF to receive QoS monitoring information and/or data analytics information of 5GS as defined in 3GPP TS 23.288 [24] for the requesting area and/or the V2X UE(s). The VAE server may determine to receive network monitoring information from EPC or 5GC or both considerting the RAT type provided by the UE. The VAE server subscription to QoS monitoring service from 5GS (e.g. PCF/NWDAF) may be active for a certain period of time or a given geographical area and/or the V2X UE(s). The monitoring may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [24], or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [19]. The reporting may be configured by the application enabler layer for a given area, V2X UE(s), time, periodicity etc taking into account the service requirement and other parameters (e.g. expected congestion in certain area, time of the day, road conditions). Based on the subscription, as specified in 3GPP TS 23.287 [19], 5GS provides the extended QoS monitoring report, over N33 interface. This report may come either from NWDAF or SMF via PCF/NEF.

2.	The network monitoring information (e.g. uplink or downlink link degradations, congestions, etc.) available at the VAE server about the on-going V2V session(s) and the network status is based on the original network monitoring information provided by the SCEF in step 1, and/or based on QoS monitoring report or data analytics from 5GS as specified in step 1. The VAE server determines the network monitoring information to send to the UE.

3.	The monitoring information is sent to the subscribed V2X UEs via network monitoring information notification.

*** Next Change ***

#### 9.17.2.3	V2X groupcast/broadcast configuration request

Table 9.17.2.3-1 describes the information flow V2X groupcast/broadcast configuration request from the VAE server to the VAE client.

Table 9.17.2.3-1: V2X groupcast/broadcast configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAE Server ID | M | Identity of the VAE server which is requester of the V2X groupcast/broadcast configuration |
| V2X group ID | M (NOTE) | Identity of the V2X group for which the V2X groupcast/broadcast configuration is requested. |
| V2X service ID | M (NOTE) | The V2X service ID for which the groupcast/broadcast configuration is requested |
| PC5 Provisioning policies/ parameters | M | PC5 provisioning policies/parameters to be used by the V2X-UEs within the V2X service (list provided in TS 23.287 [19]) |
| Relay V2X-UE ID list | O | List of identities of the V2X UEs to serve as application layer relays. |
| Minimum number of re-transmissions | O | Minimum Number of allowed re-transmissions for the V2X message delivery. |

*** Next Change ***

### 9.20.3	Subscription Procedure

Pre-conditions:

1.	The VAE server includes an Application Function (AF) functionality, as defined in 3GPP TS 23.287 [19].

2.	The VAE server has subscriptions with SEAL servers to receive V2X service-related information (e.g. UE IDs, location, group, configuration info, etc.).

Figure 9.20.3-1: Subscription for monitoring and control of QoS for eV2X communications

1.	The V2X application specific server sends a service adaptation and QoS control subscription request to the VAE server.

2.	The VAE server, who acts as an AF, subscribes to QoS monitoring service from 5GS (e.g. PCF/NWDAF). This subscription may be active for a certain period of time or a given geographical area. The monitoring may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [24], or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [19]. The reporting may be configured by the application enabler layer for a given area, time, periodicity etc taking into account the service requirement and other parameters (e.g. expected congestion in certain area, time of the day, road conditions).

3.	The VAE server sends a service adaptation and QoS control subscription response indicating success or failure of subscription.

### 9.20.4	Notification Procedure

Pre-condition:

1.	The subscription procedure specified in clause 9.20.3 is performed.

Figure 9.20.3-1: Notification for monitoring and control of QoS for eV2X communications

1.	Based on the subscription, as specified in 3GPP TS 23.287 [19], 5GS provides the extended QoS monitoring report, over N33 interface. This report may come either from NWDAF or SMF via PCF/NEF.

2	The VAE server, based on the monitoring events (step 1) for one or multiple V2X-UEs within one service or across multiple services, may trigger a service requirement adaptation (step 3) based on the actual or expected QoS change for one or more V2X services.

3.	The VAE server sends a service requirement adaptation notification request to V2X application specific server to identify the action which is needed, based on the actual or expected QoS change. This could be the adaptation of the Level of Automation (LoA) for one or more V2X-UEs or V2X services.

4.	The V2X application specific server decides whether to adapt the service requirement based on step 3.

5.	The V2X application specific server sends a response to VAE server with a positive or negative acknowledgment. Optionally, this may include the list of a sub-set of V2X-UEs within a service for which the requirement change should be applied

6.	If no QoS adaptation is required, V2X application layer may adapt based on the adapted service requirements; and steps 7-10 are optional.

7.	If a QoS adaptation is required, the VAE server, triggers the adaptation of QoS for the affected V2X-UE(s) within the service or across multiple services in close vicinity.

8.	The VAE server performs network resource adaptation by interacting with the NRM server as specified in the 3GPP TS 23.434 [10].

9.	The VAE server also notifies about the QoS adaptation to the V2X application specific server.

10.	The V2X application layer adapts based on the adapted service and QoS requirements.

*** Next Change ***

### 9.22.2	Information flows

#### 9.22.2.1	V2P application requirement request

Table 9.22.2.1-1 describes information elements for the V2P application requirement request from the V2X Application Specific Server (VASS) to the VAE server or from the V2X application specific client to the VAE client.

Table 9.22.2.1-1: V2P application requirement request

| Information element |  | Status |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Requestor ID |  | M |  | The identifier of the service requestor (VASS, VAL server, App ID) |  |  |
| V2X service ID |  | M (see NOTE) |  | The V2X service ID for which application requirement corresponds to. |  |  |
| V2X group ID |  | M (see NOTE) |  | Identity of the V2X group for which the V2P application requirement is initiated. |  |  |
| V2P QoS requirements |  | M |  | The application QoS requirements (reliability, delay, jitter) for the V2P service |  |
| Application Traffic Pattern |  | M |  | The application transmission cycle or traffic schedule for the V2P service. This may be in form of a schedule of an expected transmission/reception of a V2X message over a pre-defined time window. |  |
| PC5 Provisioning policies/ parameters |  | O |  | PC5 provisioning policies/parameters to be used by the V2X-UEs within the V2X service (list provided in TS 23.287 [19]) |  |
| NOTE:One of these shall be present. |  |  |  |  |  |

*** Next Change ***

#### 9.22.3.2	Procedure on VAE client enabled V2P communication schedule configuration

This subclause describes the procedure for V2P communication schedule configuration and update support by the VAE client.

Figure 9.22.3.2-1 illustrates the procedure where the VAE client configures the traffic schedule for V2P communications based on application requirement; and triggers the translation to an AS layer/ QoS configurations update.

Pre-conditions:

1.	VAE client 1 has been configured by the VAE server to configure the communication schedule for V2P communications.

2.	V2X UE #1 and V2X UE#2 have discovered each other based upon V2P service and established a unicast connection using the V2X Service oriented Layer-2 link establishment procedure as specified in 3GPP TS 23.287 [19] clause 6.3.3.1.

Figure 9.22.3.2-1: VAE client - enabled V2P communication schedule configuration

1.	On V2X UE #1, one or more V2X application specific clients send V2P application requirement request to the VAE client, including application QoS requirements for the V2P applications and provisioning policies and parameters for the PC5 communication.

2. The VAE client sends a V2P application requirement response to the requestor V2X application specific client (s).

3.	V2X UE #1's VAE client consolidates requirements (from one or more application specific clients) and generates a UE level transmission schedule, so that the off-duration is maximized. The determination of the transmission schedule can be determined based on the configuration on the UE (energy efficiency target) and the service KPIs.

4.	V2X UE #1's VAE client sends a V2P communication schedule update request which includes the generated UE level transmission schedule to other VAE client 2 (in vicinity, or in service-based group), in order to negotiate the optimal transmission pattern or inform on the expected reception pattern.

5.	V2X UE #2's VAE client optionally generates or updates the communication schedule for V2X UE #2, based on step 4, or negotiates the traffic pattern.

6.	VAE client of V2X UE #2 may send a V2P communication schedule update response indicating the result or an expected/generated UE #2 transmission pattern (in case of negotiation).

7.	V2X UE #1's and V2X UE #2's VAE clients provide the updated traffic pattern as V2X Application Requirements to V2X layer, also including QoS requirements such as delay requirements, priority, etc., for the PC5 communication for both applications, as specified in 3GPP TS 23.287 [19] clause 5.4.1.1. VAE client may also indicate the transmission mode (unicast, groupcast) per application. The V2X layer of UEs #1 and #2 process the requirements from VAE client and generates a UE level DRX schedule. The AS layer of UE #1 and #2 applies/configures the DRX schedule for the corresponding V2X communication.

8.	VAE clients of V2X UEs #1 and #2 may send a V2P schedule notification to the V2X application specific client(s) to inform on the communication traffic pattern.

*** Next Change ***

Annex D (normative):
Utilization of edge computing

The application architecture for supporting edge applications are specified in 3GPP TS 23.558 [25]. The V2X application layer functional model as specified in clause 6 may be deployed in edge computing environment.

Figure D-1 illustrates a V2X application layer deployment in edge computing environments.

Figure D-1: Utilization of edge computing

Figure D-1 illustrates the utilization of edge computing for V2X application layer based on edge enabler layer specified in 3GPP TS 23.558 [25].

At UE side, V2X Application Specific client(s) and VAE client interact with the Edge Enabler Client (EEC) via EDGE-5 reference point. In an Edge Data Network (EDN), V2X Application Specific Server and VAE server assume the role of EAS (Edge Application Server) and interacts with the Edge Enabler Server (EES) via EDGE-3 reference point, for instance, to register its profile into the EES. Upon service provisioning, the EEC interacts with the EES via EDGE-1 reference point, for instance, to discover V2X Application Specific Server(s) and VAE Server and further the EEC provides the discovered V2X Application Specific Server(s) and VAE server to the V2X Application Specific client and VAE client respectively. The interactions between the entities and 5GS are not shown for the sake of simplicity.

In an EDN, there could be several EES(s) provided by the same or different ECSP. The V2X application specific server(s), VAE server(s) and SEAL servers (e.g. GMS, NRMS, ADAES) shall be able to discover and register into an appropriate EES. If CAPIF is used, this can be done by utilizing the AEF serving area and/or the AEF location as described in 3GPP TS 23.222 [21]; otherwise, local configuration of the EES endpoint may be used.

The services provided by EES over EDGE-3 are not re-exposed by the VAE server or SEAL servers to the V2X application specific server but are directly consumed by the SEAL servers, VAE server and V2X application specific server(s).

NOTE:	This clause illustrates an example edge deployment using edge enabler layer and there can be other valid edge deployments enabled for V2X application layer.

Annex E:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-10 | SA6#26 |  |  |  |  | TS skeleton | 0.0.0 |
| 2018-10 | SA6#26 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-181488, S6-181489, S6-181490, S6-181548, S6-181549, S6-181554, S6-181555, S6-181556, S6-181575, S6-181576, S6-181583, S6-181589, S6-181591 | 0.1.0 |
| 2018-12 | SA6#27 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-181693, S6-181777, S6-181779, S6-181781, S6-181783, S6-181785, S6-181836, S6-181837, S6-181838, S6-181839 | 0.2.0 |
| 2019-01 | SA6#28 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-190137, S6-190144, S6-190219, S6-190221, S6-190222, S6-190223, S6-190224, S6-190271, S6-190298 | 0.3.0 |
| 2019-03 | SA6#29 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-190364, S6-190368, S6-190372, S6-190460, S6-190461, S6-190462, S6-190463, S6-190464, S6-190465, S6-190466, S6-190467, S6-190508 | 0.4.0 |
| 2019-03 | SA#83 | SP-190062 |  |  |  | Presentation for information at SA#83 | 1.0.0 |
| 2019-04 | SA6#30 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-190641, S6-190643, S6-190644, S6-190651, S6-190652, S6-190755, S6-190756, S6-190757, S6-190758, S6-190760, S6-190762, S6-190827, S6-190828, S6-190829, S6-190830, S6-190831 | 1.1.0 |
| 2019-05 | SA6#31 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-191020, S6-191021, S6-191030, S6-191032, S6-191126, S6-191028, S6-191129, S6-191130, S6-191131, S6-191191133, S6-191136, S6-191138, S6-191139, S6-191194, S6-191195, S6-191196 | 1.2.0 |
| 2019-05 | SA#84 | SP-190472 |  |  |  | Presentation for Approval at SA#84 | 2.0.0 |
| 2019-06 | SA#84 | SP-190472 |  |  |  | MCC Editorial update for publication after TSG SA approval (SA#84) | 16.0.0 |
| 2019-09 | SA#85 | SP-190734 | 0001 | 2 | F | Remove EN GeoIDs | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0002 | 3 | F | Functionalities with SA2 dependency | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0003 |  | F | Update to functional model | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0004 | 1 | F | Update to network QoS and situation monitoring aligned with SA2 | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0005 | 2 | F | Dynamic group API | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0006 | 1 | F | Correction to push layer-2 group ID mapping information flow | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0007 | 4 | F | Corrections on notifications for network monitoring procedure | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0008 | 1 | F | Addition of missing VAE server APIs | 16.1.0 |
| 2019-09 | SA#85 | SP-190734 | 0009 | 1 | F | Update the configurations information | 16.1.0 |
| 2019-12 | SA#86 | SP-191112 | 0010 | 2 | F | CR Message Delivery | 16.2.0 |
| 2019-12 | SA#86 | SP-191112 | 0011 |  | F | Update API names | 16.2.0 |
| 2019-12 | SA#86 | SP-191112 | 0012 | 3 | F | Update to uplink message delivery procedure | 16.2.0 |
| 2020-03 | SA#87-E | SP-200115 | 0013 |  | F | Add VAE application requirement notification | 16.3.0 |
| 2020-03 | SA#87-E | SP-200115 | 0014 | 1 | F | Add VAE service continuity API | 16.3.0 |
| 2020-03 | SA#87-E | SP-200115 | 0015 | 1 | F | Correct Group Management procedure | 16.3.0 |
| 2020-03 | SA#87-E | SP-200115 | 0016 | 1 | F | Align with TS 23.434 | 16.3.0 |
| 2020-03 | SA#87-E | SP-200115 | 0017 | 1 | F | Clarifications on V2X USD | 16.3.0 |
| 2020-03 | SA#87-E | SP-200115 | 0018 | 1 | F | Correction of the usage of SEAL services by the V2X application specific server | 16.3.0 |
| 2020-09 | SA#89-E | SP-200842 | 0019 |  | F | Correct Geo id | 16.4.0 |
| 2020-09 | SA#89-E | SP-200842 | 0020 | 1 | F | Correction to de-registration procedure | 16.4.0 |
| 2020-12 | SA#90-E | SP-200992 | 0033 | 1 | F | Clarifications for network monitoring information notification | 16.5.0 |
| 2020-12 | SA#90-E | SP-200998 | 0021 | 2 | B | PC5 Provisioning in multi-operator V2X scenarios | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0022 | 1 | B | V2X application layer architecture enhancement | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0023 | 1 | B | Business relationships between V2X service providers | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0024 | 1 | B | V2X service discovery across multiple V2X service providers | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0025 | 1 | B | Support for HD map dynamic information | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0026 | 1 | B | Support CAPIF | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0027 | 2 | B | Support local MBMS | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0028 | 2 | B | UE-to-UE broadcast/groupcast configuration by VAE layer | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0030 | 1 | B | V2V communication mode switching | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0031 | 1 | B | Support for local MBMS delivery | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0032 | 2 | B | V2X UE registration enhancement | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0034 | 2 | B | Network monitoring procedure enhancement | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0036 | 1 | F | Clarifications for netwok monitoring information notification | 17.0.0 |
| 2021-04 | SA#91-E | SP-210182 | 0037 | 2 | F | Clarifications on network monitoring | 17.1.0 |
| 2021-04 | SA#91-E | SP-210182 | 0038 | 1 | B | Dynamic local service information in multiple V2X service provider | 17.1.0 |
| 2021-04 | SA#91-E | SP-210182 | 0039 | 1 | B | UE initiated session oriented service | 17.1.0 |
| 2021-04 | SA#91-E | SP-210182 | 0040 | 1 | B | Monitoring and control of QoS for eV2X communications | 17.1.0 |
| 2021-04 | SA#91-E | SP-210182 | 0041 | 2 | B | Support for enhancements to V2X group management and group communication | 17.1.0 |
| 2021-04 | SA#91-E | SP-210182 | 0042 | 1 | B | Session oriented services | 17.1.0 |
| 2021-04 | SA#91-E | SP-210182 | 0043 |  | B | Update to VAE server APIs | 17.1.0 |
| 2021-04 | SA#91-E | SP-210182 | 0044 |  | F | V2X UE identity | 17.1.0 |
| 2021-06 | SA#92-E | SP-210488 | 0046 | 2 | F | Network resource management service of SEAL for V2X application | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0049 |  | D | Editorial changes for references | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0049 |  | B | Resolving editor notes in session oriented services | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0054 | 1 | F | Editorial corrections for HDMap feature | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0055 | 1 | B | Information flows and API definitions for session-oriented service | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0056 | 1 | B | Requirements for message distribution | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0057 | 1 | B | Alignment of Get V2X UE service discovery response with request | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0058 | 1 | B | Alignment of information flows for local service information | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0060 | 1 | B | Resolve EN on V2X groupcast/broadcast configuration | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0061 | 1 | B | Resolve EN on PC5 provisioning for multi-operator V2X services | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0062 | 1 | B | Update of VAE server APIs | 17.2.0 |
| 2021-06 | SA#92-E | SP-210488 | 0064 |  | B | Updates to usage of SEAL for V2X application layer | 17.2.0 |
| 2021-09 | SA#93-E | SP-210965 | 0066 | 2 | F | Clarifications for RAT Type usage in network monitoring procedure | 17.3.0 |
| 2021-09 | SA#93-E | SP-210965 | 0067 |  | F | Clarifications for RAT Type usage in network monitoring procedure | 17.3.0 |
| 2021-09 | SA#93-E | SP-210965 | 0068 |  | F | Alignment with SEAL key managment | 17.3.0 |
| 2021-09 | SA#93-E | SP-210965 | 0069 | 1 | F | Corrections to HDMap procedures | 17.3.0 |
| 2022-06 | SA#96 | SP-220474 | 0071 | 1 | F | Add location reference in HDmap | 17.4.0 |
| 2022-12 | SA#98-e | SP-221252 | 0072 | 2 | B | V2X application layer architecture support for edge deployments | 18.0.0 |
| 2022-12 | SA#98-e | SP-221252 | 0073 | 2 | C | Enhancement to network monitoring | 18.0.0 |
| 2022-12 | SA#98-e | SP-221252 | 0074 | 2 | B | Support for VRU zone configuration and operation | 18.0.0 |
| 2023-03 | SA#99 | SP-230298 | 0075 | 1 | B | VAE support for Energy Efficient V2P communications | 18.1.0 |
| 2023-03 | SA#99 | SP-230298 | 0076 |  | B | Adding information flows and editorial corrections | 18.1.0 |
| 2023-06 | SA#100 | SP-230716 | 0077 |  | B | VAE client enabled V2P communication schedule configuration | 18.2.0 |
| 2023-09 | SA#101 | SP-231011 | 0078 |  | F | Add VAE server API in clause 10 | 18.3.0 |
| 2023-12 | SA#101 | SP-231574 | 0080 | 2 | F | Correction for VAE server APIs General | 18.4.0 |