3GPP TSG-SA WG6 Meeting #60	S6-241509

Changsha, China 15th – 19th April 2024	(revision of S6-241316)

Source:	Nokia

Title:	Solution for KI#3

Spec:	3GPP TR 23.700-21 v0.2.0

Agenda item:	8.4

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution proposes a solution for KI#3.

2. Reason for Change

In TR 23.700-21, key issue#3 is as follows:

## 4.3 Key issue #3: Digital avatars support ### 4.3.1 Description In addition to new 5GS performance requirements, support for Metaverse services have introduced new use cases and requirements related to optimal support for multi-user, highly performant applications. These use cases led to new CN functionality supporting richer digital representations of physical entities (e.g. user profiles in SA2, XR scene in SA4) As defined in 3GPP TS 22.156 [2], Avatars are digital representations of users interacting with the metaverse and with other users. The application enabler layer can enable creation, discovery, and management of avatar profiles for users to offload applications and enable Core Network functionality across services and verticals. NOTE 1: The work in SA6 focuses on providing enablement service to application specific Avatar profile and if available, can leverage the work done in SA2 and SA4. Editor's note: How application specific Avatar profile can be linked to user subscription is FFS and depends on SA2 work. Therefore, solutions addressing this key issue may also leverage work from SA2 and SA4 on user profiles and digital assets. The following use cases can be considered for study of value-added features: - A user provides configuration (or preferences) indicating which metaverse application services can access the avatar information when the user performs transition between metaverse services, to make sure that consistency and continuity of their digital representation is maintained. - SA6 can maintain a list of applications allowed to use the avatar (similar to allowed MNO list for federation in EDGEAPP) - A User creates multiple avatars and indicates his/her preferences such as: use of specific avatars based on location, use of audio-only service only when experiencing low bandwidth. - SA6 can maintain user’s choices and: provide avatar information to different application servers based on the location of the user, reject requests for avatar-based communication in poor network conditions, etc. ### 4.3.2 Open issues This key issue will study: - What SA6-defined information is required to expose the SA2-defined, SA4-defined and SA3-defined avatar information to support allowed application configuration, location-based avatar contextual information? - Whether and how application enablement layer manages and exposes information about digital avatars to the consumers (i.e. application clients and servers) or across verticals?

A solution for this key issue is proposed based on the SEAL architecture option (Option#x) where A-DAC function (SEAL) exposes the APIs (for avatar management) to the Vertical Application Layer Servers or Vertical Application Clients on UEs. The procedures for avatar management is proposed.

The avatar management API operations are organized as follows:

- Avatar profile creation;

- Avatar upload;

- Avatar linking to user/subscriber;

- Avatar download;

- Avatar update/modification;

- Avatar delete;

The VAL layer utilizes Avatar profile creation operation which enables meta-data related to Avatar being provided and authorized to be accepted by the A-DAC function (offered by MNO). Based on successful creation of Avatar profile, the media object related to Avatar profile can be uploaded by the VAL layer to the A-DAC function using Avatar upload operation. Further, VAL layer can link the user/subscribers to the Avatar by utilizing the Avatar linking operation. VAL applications which are allowed to use the avatar can download the Avatar for the application purpose utilizing the Avatar download operation.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-21 v0.2.0.

* * * First Change * * * *

## 7.x	Solution #x: Digital avatar support by A-DAC

### 7.x.1	Solution description

#### 7.x.1.1	General

This solution maps to KI#3. This solution is based on A-DAC function to support the following avatar management services offered to VAL layer (VAL client and VAL server):

-	Avatar profile creation;

-	Avatar upload;

-	Avatar linking to user/subscriber;

-	Avatar download;

-	Avatar update/modification;

-	Avatar delete;

#### 7.x.1.2	Avatar profile creation

Figure 7.x.1.2-1 illustrates the high level flow for avatar profile creation procedure.

Figure 7.x.1.2-1: Procedure for Avatar profile creation

1.	VAL server/VAL client sends the Create Avatar Profile Request to A-DAC function with the details such as Avatar ID (optional), Avatar properties (e.g. name), Avatar usage policies (e.g. how to use the avatars).

2.	The A-DAC function checks the authorization of the VAL server/client to perform the operation. If successful, then A-DAC function stores the avatar profile.

3.	The A-DAC function sends a Create Avatar Profile Response to the VAL server/VAL client indicating success or failure of the operation.

#### 7.x.1.3	Avatar profile upload

Figure 7.x.1.3-1 illustrates the high level flow for avatar upload procedure.

Pre-conditions:

1.	Avatar ID is available as per the procedure in clause 7.x.1.2

Figure 7.x.1.3-1: Procedure for Avatar upload

1.	The VAL server/VAL client sends the Upload Avatar Request to the A-DAC function with the Avatar ID and Avatar object/media information.

2.	The A-DAC function checks the authorization of the VAL server/client to perform the operation. If successful, the A-DAC function performs the storage of avatar object/media information else the storage of the avatar object/media information.

3.	The A-DAC function sends a Upload Avatar Response to VAL server/client indicating success or failure of the operation.

#### 7.x.1.4	Avatar linking to user/subscriber

Figure 7.x.1.4-1 illustrates the high level flow for avatar linking to user/subscriber procedure.

Figure 7.x.1.4-1: Procedure for Avatar linking to user/subscriber

1.	The VAL server/VAL client sends a Link Avatar Rquest to the A-DAC function with Avatar ID and GPSI/External identifier of the UE.

2.	The A-DAC function checks the authorization of the VAL server/client to perform the operation. If successful, the A-DAC function checks for the authorization (e.g. user consent) of the subscriber (as per step 1) for linking the avatar with the user/subscriber. If authorized, the A-DAC function links the Avatar ID with the subscriber/user information and stores the related information.

3.	The A-DAC function sends a Link Avatar Response to the VAL server/client indicating success or failure of the operation.

#### 7.x.1.5	Avatar download

Figure 7.x.1.5-1 illustrates the high level flow for avatar download procedure.

Figure 7.x.1.5-1: Procedure for Avatar download

1.	The VAL server/VAL client sends a Download Avatar Request to the A-DAC function with the Avatar ID, GPSI/External ID of the UE, Filter/Adaptation information (e.g.VAL server/client information, media adaptations).

2.	The A-DAC function checks the authorization of the VAL server/client to perform the operation. If successful, the A-DAC function performs the media adaptation as per the additional filters/adaptation information in the request on the avatar object/media. The adapted avatar media/object is used in the further response to the VAL server/client.

3.	The A-DAC function sends a response to the VAL server/client indicating success or failure of the operation. If success, the avatar object/media is included in the response.

### 7.x.2	Architecture Impacts

The solution utilizes the architecture specified in clause 6.x, Option#x.

### 7.x.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution addressing KI#3. The A-DAC function can support the avatar management services towards the VAL layer.

NOTE:	The security mechanisms for this solution is the responsibility of SA3.