3GPP TSG-SA WG6 Meeting #60	S6-241545

Changsha, China 15th – 19th April 2024	(revision of S6-241114)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0613 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Update services of Eees_TrafficInfluenceEAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Traffic influence information could also be updated via update service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Application traffic influence update trigger from EAS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EAS traffic influence update is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.7.2.2, 8.6.7.3.4, 8.6.7.3.1, 8.6.7.3.5, 8.6.7.3.6, 8.6.7.3.7, 8.6.7.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.6.7.2.2	Procedure of application traffic influence update trigger from EAS

Figure 8.6.7.2.2-1 illustrates the procedure of application traffic influence update trigger from EAS.

Figure 8.6.7.2.2-1: Application traffic influence update trigger from EAS

- The EAS sends an EAS traffic influence update request providing the corresponding transaction ID, and updated target UE Identifier(s).

2.	The EES performs an authorization check to verify whether the EAS has the authorization to request application traffic influence update.

3.	Upon successful authorization, the EES do the update accordingly. For those UE(s) to be deleted, the EES invokes the Nnef_TrafficInfluence_Delete or Npcf_PolicyAuthorization_Delete Request to delete the original request to delete the traffic influence for EAS. For those UE(s) to be added, the EES invokes the Nnef_TrafficInfluence_Create or Npcf_PolicyAuthorization_Create with a new transaction ID.

4.	The EES sends the EAS traffic influence update response.

8.6.7.2.3	Procedure of application traffic influence cancellation trigger from EAS

Figure 8.6.7.2.3-1 illustrates the procedure of application traffic influence cancellation trigger from EAS.

Figure 8.6.7.2.3-1: Application traffic influence cancellation trigger from EAS

1.	The EAS sends an EAS traffic influence cancellation request providing the corresponding transaction ID.

2.	The EES performs an authorization check to verify whether the EAS has the authorization to request application traffic influence cancellation.

3.	Upon successful authorization, the EES includes transaction ID in the Nnef_TrafficInfluence_Delete Request to the NEF, or Npcf_PolicyAuthorization_Delete Request to the PCF, to delete the traffic influence for EAS as described in 3GPP TS 23.501, clause 5.6.7.1.

4.	The EES sends the EAS traffic influence cancellation response.

8.6.7.3	Information flows

8.6.7.3.1	General

The following information flows are specified for application traffic influence trigger from EAS:

-	Application traffic influence trigger from EAS request and response.

-	Application traffic influence update trigger from EAS request and response.

-	Application traffic influence cancellation trigger from EAS request and response.

8.6.7.3.2	Application traffic influence trigger from EAS request

Table 8.6.7.3.2-1: application traffic influence trigger from EAS request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | Identifier of the EAS |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Target UE Identifier(s) | M | Indicates the target UE(s) or any UE. |

8.6.7.3.3	Application traffic influence trigger from EAS response

Table 8.6.7.3.3-1: application traffic influence trigger from EAS response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the traffic influence request was successful. |
| >Transaction ID | M | Identifier of the traffic influence transaction used. |
| Failure response | O | Indicates that the traffic influence request has failed. |
| > Cause | O | Indicates the cause of failure |

8.6.7.3.4	Application traffic influence update trigger from EAS request

Table 8.6.7.3.4-1: application traffic influence update trigger from EAS request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | Identifier of the EAS |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Updated target UE Identifier(s) | M | Indicates the updated target UE(s) or any UE. |
| Transaction ID | M | Indicates the target transaction that is to be updated. |

8.6.7.3.5	Application traffic influence update trigger from EAS response

Table 8.6.7.3.5-1: application traffic influence update trigger from EAS response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the traffic influence request was successful. |
| >Transaction ID | O | Present If new Application traffic influence is created with new transaction ID |
| Failure response | O | Indicates that the traffic influence request has failed. |
| > Cause | O | Indicates the cause of failure |

8.6.7.3.6	Application traffic influence cancellation trigger from EAS request

Table 8.6.7.3.6-1: application traffic influence cancellation trigger from EAS request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | Identifier of the EAS |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Transaction ID | M | Indicates the target transaction that is to be updated. |

8.6.7.3.7	Application traffic influence cancellation trigger from EAS response

Table 8.6.7.3.7-1: application traffic influence cancellation trigger from EAS response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the traffic influence request was successful. |
| Failure response | O | Indicates that the traffic influence request has failed. |
| > Cause | O | Indicates the cause of failure |

8.6.7.4.1	General

Table 8.6.7.4.1-1 illustrates the APIs for application traffic influence trigger from EAS.

Table 8.6.7.4.1-1: Eees_TrafficInfluenceEAS API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TrafficInfluenceEAS | Create | Request/Response | EAS |
|  | Update | Request/Response | EAS |
|  | Cancellation | Request/Response | EAS |

8.6.7.4.2	Eees_TrafficInfluenceEAS_Create operation

API operation name: Eees_TrafficInfluenceEAS_Create

Description: The consumer requests traffic influence for EAS via EES.

Inputs: See clause 8.6.7.3.2.

Outputs: See clause 8.6.7.3.3.

See clause 8.6.7.2.1 for details of usage of this operation.

8.6.7.4.3	Eees_TrafficInfluenceEAS_Update operation

API operation name: Eees_TrafficInfluenceEAS_Update

Description: The consumer requests to update traffic influence for EAS via EES.

Inputs: See clause 8.6.7.3.4.

Outputs: See clause 8.6.7.3.5.

See clause 8.6.7.2.2 for details of usage of this operation.

8.6.7.4.4	Eees_TrafficInfluenceEAS_Cancellation operation

API operation name: Eees_TrafficInfluenceEAS_Cancellation

Description: The consumer requests to cancel traffic influence for EAS via EES.

Inputs: See clause 8.6.7.3.6

Outputs: See clause 8.6.7.3.7

See clause 8.6.7.2.3 for details of usage of this operation.