3GPP TSG-SA WG6 Meeting #60	S6-241629

Changsha, China 15th – 19th April 2024	(revision of S6-241553, S6-241290)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0630 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AC and EEC interaction for ACR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The AC and EEC interaction over EDGE-5 is not specified for ACR triggering by the EEC via AC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following are the changes proposed: 1. EEC Notification to AC: EEC triggers AC to perform DNS query 2. ACR trigger request: to include CAS information when AC performs DNS query 3. New procedure DNS trigger status reporting between AC and EEC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AC and EEC interaction specification would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.3.10, 8.14.2.x (new), 8.14.3.x1 (new), 8.14.3.x2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.14.3.10	ACR trigger request

Table 8.14.3.10-1 describes information elements of the ACR trigger request sent by the AC to the EEC.

Table 8.14.3.10-1: ACR trigger request

| Information element | Status | Description |
| --- | --- | --- |
| AC profile | M | AC profile for EEC to determine the target EAS. |
| Security credentials | M | Security credentials of the AC |
| Requested ACR action | M | One of ACR detection or ACR initiation request actions |
| S-EAS information | M | ID of the EAS to which the ACR request relates. |
| T-EAS information (NOTE) | O | AC provided target EAS information or CAS information received from DNS query result. |
| NOTE: This IE may be included if the request is to initiate the ACR to EAS or CAS. |  |  |

* * * Next Change * * * *

#### 8.14.2.x	CAS information sharing

The procedure is for AC to share CAS information to EEC, from the result of DNS query. The DNS query is triggered by AC due to unavailablity of T-EES or T-EAS.

Pre-conditions:

1.	AC has performed DNS query due to unavailability of suitable T-EES and/or unavailability of suitable T-EAS as specified in clause 8.14.2.5.3.

Figure 8.14.2.x-1: CAS information sharing procedure

1.	The AC sends CAS information sharing report to the EEC, for providing the result of the AC performed DNS query (due to unavailability of suitable T-EES and/or unavailability of suitable T-EAS). The request includes ACID and the result of DNS trigger status as described in clause 8.14.3.x1.

2.	The EEC provides an acknowledgement back to AC as described in clause 8.14.3.x2.

* * * Next Change * * * *

#### 8.14.3.x1	CAS information sharing request

Table 8.14.3.x1-1 describes information elements of the CAS information sharing request sent by the AC to the EEC.

Table 8.14.3.x1-1: CAS information sharing request

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | ID of the AC which performed the DNS trigger request and sharing the CAS information. |
| Successful response (NOTE) | O | Indicates that the DNS query was successful. |
| > CAS information | O | AC provided target CAS information received from DNS query result. |
| Failure response (NOTE) | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * Next Change * * * *

#### 8.14.3.x2	CAS information sharing response

Table 8.14.3.x2-1 describes information elements of the CAS information sharing request sent by the EEC to the AC.

Table 8.14.3.x2-1: CAS information sharing response

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | ID of the AC for which the CAS information was shared. |
| Acknowledgement | M | Indicates receipt of the CAS information sharing request. |

* * * End of Changes * * *