3GPP TSG-SA WG6 Meeting #60	S6-241654

Changsha, China 15th – 19th April 2024	(revision of S6-241619, 1493, 1152)

Source:	Huawei, Hisilicon

Title:	Key Issue on deployment enhancement for XR application

Spec:	3GPP TR 23.700-23 v0.2.0

Agenda item:	8.5

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper proposes a new key issue deployment enhancement for XR application.

2. Reason for Change

Due to one DNAI may be mapped to multiple UPFs, thus the issue may exist that the selected UPF and selected application server may still not in same data center and the N6 delay still contains the delay of UPF – APP server involving the data transmission via router (s) which the latency is longer than the data transmission via network adapter. In this case the E2E latency may not be able to satisfy the XR service requirement.

Therefore, further study the deployment enhancement for XR application is required.

The detailed clarification is in the discussion paper S6-241153

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v0.2.0.

* * * First Change * * * *

4.x	Key issue #x: XR application server selection enhancement

As for XR service, current XR application server selection mechanism using EDGEAPP may not be able to satisify the strict latency requirement. And considering that XR application server may involve multi-model communication flow, to support multi-model communication flow synchronization, it is better to improve service exprience to provide ensure the operation on the multi-model synchrnization.

However, due to one DNAI may be mapped to multiple UPFs in different EDNs, thus only using DNAI to and application server may not be enough.

Consider the following case (as showed in figure x.1):

The UE is in the overlapping area of EDN#1 and EDN#2, when performing service provisioning, the UE may anchor to UPF#2 however the UE may connect to the EES#1 for EAS information. Then the UE may connect to EAS#1 discovered from EES#1.

The issue may happen that the user plane path is not optimal, which may not be able to satisify the XR application requirement.

Figure x.1 scneario on XR application server selection

This key issue includes the following aspects:

- Whether and how enablement layer can support XR application server to further reduce service latency?

- Whether and how enablement layer can support XR application server selection?

NOTE: For this Key issue, the coordinate with SA2 may be required.