3GPP TSG-SA WG6 Meeting #62	S6-243019

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	FirstNet Authority, Nokia, AT&T

Title:	Pseudo-CR on ongoing communications continuity when backhaul is recovered

Spec:	3GPP TR 23.700-09, v0.1.0

Agenda item:	8.8

Document for:	Approval

Contact:	Mark Lipford, mark.lipford@firstnet.gov

1. Introduction

This key issue proposes looking at the impact of ongoing communications once the backhaul has been restored

2. Reason for Change

Add a key issue on impacks of ongoing communications when backhaul is restored

3. Conclusions

n/a

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-09, v0.1.0

* * * First Change * * * *

### 4.x.x	Key issue # on ongoing communication continuity when backhaul is recovered

#### 4.x.x.1	Description

When the backhaul of a base station to Macro network is broken, the base station enters IOPS mode and connects to the Local network, and the MC users served by this base station will connect to the IOPS MC system.

After some time, the backhaul is recovered, then this base station switches back to normal mode and re-connects to the Macro network, and the users served by this base station will re-connect to the Primary MC system. This scenario is illustrated in Figure 4.x.x-1. At the transition time, there is a call ongoing in the IOPS MC system. Re-connecting to the Primary MC system will likely cause the termination of the ongoing communication in the IOPS MC system. However, it is beneficial to minimize the disruption to ongoing communication during the transition.

Figure 4.x.x-1: Ongoing communication continuity when backhaul is recovered

Further study is needed:

-	Understand how communications are re-established in the Primary MC system including how group members join group communications in progress in the Primary MC system;

-	How to handle call collisions if calls are ongoing in the same group in both the Primary MC system and IOPS MC system;

-	How the ongoing communication continuity can be achieved when transitioning from IOPS mode to Macro mode.

* * * End of Changes * * * *