3GPP TSG-SA WG6 Meeting #62	S6-243029

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	China Mobile

Title:	Sol for KI#7 EAS discovery with computing requirements to optimize XR service requirements

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#7,

- How to support the E2E KPI optimization to meet real-time XR service requirements, taking the processing delay of XR application server into account, e.g. considering the potential enhancements to EDGEAPP in XR application server discovery or instantiation.

- What available information from 5G network can be used by enable layer to optimize the real-time XR service requirements.

2. Reason for Change

Considering that XR systems are computationally intensive services, the resource requirements should be taken into consideration when performing EAS discovery. Currently, EAS discovery considers the AC Service KPI in the AC profile. However, the AC Service KPI provided by the application provider is merely statistical data, which may not be sufficiently accurate, particularly regarding computing resource requirements that can vary depending on device capabilities (e.g., different hardware for different tasks, etc.). For example, for services require split rendering, the computing resource requirements would be different based on its rendering capability and rendering content. Therefore, it is proposed that AC/EEC should provide specific resource requirements in the EAS discovery request, which could be seen as an complementary of the existing solution.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

7.x	Solution #x: Sol for KI#7 EAS discovery with computing requirements to optimize XR service requirements

7.x.1	Architecture Impacts

This solution is presented based on the existing EDGEAPP architecture in TS 23.558 [11].

7.x.2	Solution description

Figure 7.x.2-1 illustrates the procedure to support the EAS discovery with list of resources requirement for XR services. The provided resource requirement could be used in EAS discovery and also in the EAS insitantiation.

Pre-conditions:

- EAS performs EAS registration procedure, as described in clause 8.4.3.2.2 of TS 23.558.

- EES registers to ECS according to EES Registration procedure in clause 8.4.4 of TS 23.558 [11].

EAS

2. EAS discovery request

3. EES discovers the EAS(s)

EAS

1. Application access request

4. EAS discovery response

EES

EEC

AC

Figure 7.x.2-1: procedure of supporting EAS discovery with specific computing requirements

- AC sends application access request to EEC, including list of resources requirement for a specific XR application (if needed).

NOTE 1: the EAS discovery could be considered to be reused as application access request.

NOTE 2: the List of resources requirement could be considered to be added in the EAS discovery filters.

- The EEC sends the EAS discovery request to the EES, including list of resources requirement for XR application.

- Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). If authorized, the EES takes the provided list of resources requirement into consideration during the EAS discovery and/or the EAS insitantiation.

- The EES sends the EAS discovery response to EEC with the EAS endpoint information.

7.x.3	Solution evaluation

This solution enables E2E KPI (processing delay) optimization with EAS discovery request carrying list of resources requirement. This solution is updated based on EAS discovery procedure in clause 8.5.2.2 of TS 23.558. The solution has no architectural impacts.

* * * End Change * * * *