3GPP TSG-SA WG6 Meeting #62	S6-243032

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	China Mobile

Title:	Update overall evaluation and conclusion for KI#8

Spec:	3GPP TR 23.700-23

Agenda item:	8.5

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This paper proposes to update overall evaluation and conclusion for KI#8.

- Reason for Change

Update overall evaluation and conclusion for KI#8 is needed to capture the PDU Set Information utilization in solution#6.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23.

* * * First Change * * * *

9.3.8	Key issue #8 solutions evaluation

The open issue of KI#8 include:

-	Whether and how SEALDD or other application enabler may be enhanced to facilitate XR communication flows between application clients and application servers involved in the same application service in relation to PDU set (e.g., support for PDU set handling, QoS measurement)?

In the existing solutions, solution#4 addresses QoS measurement for XR flows including PDU set on crossflow RTT. The SEALDD measurement function is enhanced to identify the 1st UL or last DL packet in a PDU set, e.g. for SEALDD client triggered measurement, the SEALDD client needs to identify the last DL packet in a PDU set to complete the measurement for a DL video frame. The crossflow QoS measurement provides added value for the XR service, for example, Motion to Photon can be estimated as crossflow RTT measurement (e.g. {[traffic descriptor 1 for joystick input, UL]; [traffic descriptor 2 for video frame, DL]}).

Editor's note: Whether identifying packet in PDU set is necessary in crossflow RTT measurement is FFS.

In the existing solutions, solution#7 addresses PDU set handling support. The SEALDD layer is enhanced with RTP packetilization and PDU set inclusion.

Solution#6 utilizes the PDU Set Information for Multi-modal flows alignment. The PDU Set Information in RTP Header Extension, including the PDU Set Sequence Number, Indication of End PDU of the PDU Set could be used to detect the target flows for Multi-modal flows alignment.

* * * Next Change * * * *

10.8	Conclusions of key issue #8

Solution#4 can be considered for normative work with the following principles:

-	Solution #4 can be used to measure and expose crossflow QoS measurement for XR flows including PDU set.

-	For the target flow identification, traffic descriptor could be used to uniquely identifies the flow while the exact information element needs to be defined during normative work.

Solution #7 can be considered for PDU set handling support for normative work.

Solution#6 can be used to do Multi-modal flows alignment based on PDU Set Information.

* * * Next Change * * * *

<Proposed change in revision marks>