3GPP TSG-SA WG6 Meeting #62	S6-243037

Maastricth, Netherlands, 19-23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0567 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional architecture for MCX Service Logging, Recording and Replay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Logging, Recording and Replay are not part of MCX Service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Logging, Recording and Replay related functional entities and reference points. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCX Logging, Recording and Replay will not be realized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 7.3.1, 7.4.2.x (new), 7.4.2.x.1 (new), 7.4.2.x.2 (new), 7.5.2.2, 7.5.2.y (new)…7.5.2.y+6 (new), 8.1.1, 8.1.8 (new), A.n (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1]. Not all definitions are used in this document.

Accuracy: Reflects the uncertainty of the location at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

ACM: Administrative Configuration Management, which enables the exchange of administrative configuration data between interconnected MC systems.

ACMC: Administrative Configuration Management Client, client entity which initiates administrative configuration exchange request to an interconnected partner MC system and which could make decision on such request received from an ACMC of a partner MC system.

ACMS: Administrative Configuration Management Server, server entity which receives administrative configuration exchange requests from an ACMC belonging to the same primary MC system or via an ACMS of an interconnected partner MC system.

Active MC service user profile: The MC service user profile that is currently used by an MC service client of an MC service user while receiving MC service.

Ad hoc Group Communication: The combining of a multiplicity of MC service users into a group for the duration of a communication. When the communication is released, the group no longer exists. If the communication is associated with an alert, then the group continues to exist until the alert is also canceled.

Ad hoc Group emergency alert: The combining of a multiplicity of MC service users into a group for sending an emergency alert. When the alert is cancelled, the group no longer exists. If the alert is associated with a communication, then the group continues to exist until the communication is also canceled.

Altitude: Third dimension for the geographical coordinates at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Bearing: Direction at the moment of location measurement, e.g. see 3GPP TS 25.305 [30].

Chat group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Group members must join the pre-estabslihed group call to participate.

ECGI: E-UTRAN Cell Global Identifier, which is used to identify cells globally, where the ECGI is constructed from the Mobile Country Code (MCC), Mobile Network Code (MNC) and the E-UTRAN Cell Identifier (ECI).

Interconnection: A means of communication between MC systems whereby MC service users obtaining MC service from one MC system can communicate with MC service users who are obtaining MC service from one or more other MC systems.

Interconnection group: An MC service group that is configured to allow inclusion of MC service group members who are MC service users from partner MC system(s).

LCS network: The 3GPP network that provides location service as defined in 3GPP TS 23.271 [29].

Location: The current physical location of the MC service UE.

MBMS SAI: Multimedia Broadcast Multicast Service Area Identity which is mapped to the MBMS service area.

MC gateway server: A server providing topology hiding for MC service interconnection with a partner MC system, where that partner MC system is in a different trust domain.

MC Recording Server: A server that is able to log the metadata and record the media of MCPTT, MCData, and MCVideo Group Communications and Private Communications. This server is also able to retrieve the logged/recorded data when requested by an authorized user.

MC Replay UE: A user equipment that can be used to fetch and replay recorded MCPTT, MCData, and MCVideo metadata and media of MCX Service Group Communications and MCX Service Private Communications under the UE user’s authority. MC Replay UE can be a mobile device or a (computer) workstation.

MC Replay Client: A client application that is able to fetch and replay recorded metadata and media from a Recording Server.

MC service: A generic name for any one of the three mission critical services: either MCPTT, or MCVideo, or MCData.

MC service affiliated group member: An MC service user who has indicated an interest in a particular MC service group and has been accepted to participate in MC service group communication for that MC service group.

MC service client: A generic name for the client application function of a specific MC service. MC service client could be replaced by MCPTT client, or MCVideo client, or MCData client depending on the context.

MC service group: A defined set of MC service users with associated communication dispositions (e.g. media restrictions, default priority and commencement directions) configured for the use with one or more MC services.

MC service group affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is determined.

MC service group call: A mechanism by which an MC service user can make a one-to-many MC service(s) transmission to other users that are members of MC service group(s).

MC service group de-affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is removed.

MC service group home system: The MC system where the MC service group is defined.

MC service group host MC service server: The MC service server within an MC system which provides centralised support for a particular MC service of an MC service group defined in a MC service group home system.

MC service group member: An MC service user, whose MC service ID is listed in a particular MC service group.

MC service ID: A generic name for the user ID of a mission critical user within a specific MC service. MC service ID could be replaced by MCPTT ID, or MCVideo ID, or MCData ID depending on the context.

MC service server: A generic name for the server application function of a specific MC service. MC service server could be replaced by MCPTT server, MCVideo server, or MCData server depending on the context.

MC service user: An authorized user, who can use an MC service UE to participate in one or more MC services.

MC service user profile: The set of information associated to an MC service user that allows that user to employ one or more MC services in a given role and from a given MC service UE.

MC service UE: A UE that can be used to participate in one or more MC services.

MC service UE label: A generic name for identification of a specific MC service UE.

MC system: The collection of applications, services, and enabling capabilities required to provide a single mission critical service or multiple mission critical services to one or more mission critical organizations.

MC user: A user, identified by an MC ID, who, after authorization, obtains mission critical service(s).

Migration: A means for an MC Service user to obtain MC service directly from a partner MC system.

Partner MC system: Allied MC system that provides MC services to an MC service user based on the MC service user profiles that are defined in the primary MC system of that MC service user.

Preconfigured MC service group: an MC service group used only for regrouping that has been configured in advance of a group or user regrouping operation to serve as the source of regroup group configuration.

Pre-arranged group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Affiliated group members are invited when the group communication is setup.

Pre-selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile through configuration, and applicable for an authenticated MC service user upon MC service authorization.

Primary MC system: MC system where the MC service user profiles of an MC service user are defined.

Requested Priority: A value for use in a MC service group or MC private communication that, if accepted, is used by the MCX service server to temporarily replace the priority level that is predefined in the MC service group or MC service user profile. This value is used in combination with other factors to determine the application priority for the requested communication.

Selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile for an MC service upon request by an MC service user.

Serving MC service server: The MC service server which is providing MC service to an MC service client.

NOTE 1:	There is one serving MC service server for each MC service, which can be the primary MC service server of the MC service user of the MC service client, or can be a partner MC service server to which the MC service user has migrated.

Serving MC system: The MC system which is providing MC service to an MC user.

NOTE 2:	The MC system can be the primary MC system of the MC service user, or can be a partner MC system to which the MC service user has migrated.

Speed: Movement at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Time of measurement: Date and time expressed with a certain precision to reflect the moment of the location measurement.

For the purposes of the present document, the following terms given in 3GPP TS 22.280 [3] apply

Mission Critical

Mission Critical Applications

Mission Critical Organization

Mission Critical Service

Functional alias

For the purposes of the present document, the following terms given in 3GPP TS 22.179 [2] apply

Multi-talker control

Group-broadcast group

For the purposes of the present document, the following terms related to a MC gateway UE function apply

MC gateway UE:  A functional entity that enables simultaneous access to the MC system for multiple MC clients.

MC client: Aggregates a set of clients (i.e. Group management client, Configuration management client, Identity management client, Key management client, Location management client and MC service client).

MC server: Aggregates a set of servers (i.e. Group management server, Configuration management server, Identity management server, Key management server, Location management server and MC service server) which serves the MC client accordingly.

Non-3GPP device: A device that enables connectivity towards an MC gateway UE using an access method not specified by 3GPP. A subset of these devices can host an MC client specified by 3GPP.

MC gateway client: A client that enables the authorized binding with one or more MC GW UEs in order to be able to handle MC services (only one MC gateway UE per MC service).

MC gateway UE server: A server on an MC gateway UE that controls authorized binding with multiple MC gateway clients.

MC gateway UE function: Functional block as part of the MC service server that authorises and manages the association between MC client and MC gateway UE.

# * * * * Second change * * * *

### 7.3.1	On-network functional model

Each MC service can be represented by an application plane functional model. The functional model across MC services may be similar but is described by the individual functional entities and reference points that belong to that MC service. Within the application plane for an MC service there is a common set of functions and reference points. The common set is shared across services. This common set of functions and reference points is known as the common services core.

Figure 7.3.1-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1-1: Functional model for application plane for an MC system

Editor's note:	If a Location management client is needed in MC Replay UE is ffs.

The common services core functions and reference points shown in figure 7.3.1-1 are shared across each MC service. The description of the functions and reference points specific to an MC service is contained in the corresponding MC service TS.

NOTE 1:	The ACM client and ACM server are not essential entities to enable MC services. The ACM client and ACM server are optional and only required if the administrative configuration management feature is deployed. In administrative configuration management case, only the UE of an authorized MC service user requires such an ACM client.

In the model shown in figure 7.3.1-1, the following apply:

-	A specific MC service server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [18].

-	The functional alias management client is an integrated functional entity of the configuration management client. The functional alias management client is described in subclause 7.4.2.2.12.

-	The functional alias management server is an integrated functional entity of the configuration management server. The functional alias management server is described in subclause 7.4.2.2.13.

Figure 7.3.1-2 shows the functional model for the signalling control plane.

Figure 7.3.1-2: Functional model for signalling control plane

Figure 7.3.1-3 shows the relationships between the reference points of the application plane of an MC service server and the signalling plane.

Figure 7.3.1-3: Relationships between reference points of MC service application plane and signalling control planes

NOTE 2:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 3:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 7.3.1-3.

NOTE 4:	CSC-5, CSC-9, CSC-15, RCS-2, RCS-4, RCS-5 and RCS-6 make use of SIP-1 and SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 7.3.1-3.

# * * * * Third change * * * *

#### 7.4.2.x	Recording and Replay

##### 7.4.2.x.1	MC Replay client

The MC Replay client is a functional entity that can be used by an authorized user to retrieve recorded metadata and media from an MC recording server and replay it.

A MC Replay client may be used in an MC Replay UE. It may also be included in an MC Service UE.

##### 7.4.2.x.2	MC Recording server

The MC Recording server is a functional entity that can receive and record communications metadata and media relating to target users and target groups from MC Service Servers. The MC recording server securely stores the recorded information into mass storage(s) and allows controlled access for replay/retrieve functions.

The MC Recording server functional entity is supported by the SIP AS and HTTP server functional entities of the signalling control plane.

The target users and/or groups for recording are set in the MC service user profile configuration data (A.3) and in the Group configuration data (A.4). This can be done by an authorized user utilizing the CSC-4 reference point.

Editor's note:	Whether the existing configuration tables (A3, A.4) can be utilized or a new configuration table is needed is FFS.

# * * * * Fourth change * * * *

#### 7.5.2.2	Reference point CSC-1 (between the identity management client and the identity management server)

The CSC-1 reference point, which exists between the identity management client and the identity management server, provides for the authentication of the common services core to the MC service client and MC replay client and subsequent authentication of the user to the common services core on behalf of applications within the application plane.

CSC-1 is specified in 3GPP TS 33.180 [25].

# * * * * Fifth change * * * *

#### 7.5.2.y	Reference point RCS-1 (between the replay client and the recording server)

The RCS-1 reference point, which exists between replay client and recording server, is used by the replay client to request and receive logs/recordings of metadata and media from the replay server.

The RCS-1 reference point shall use the HTTP-1 and HTTP-2 reference points.

The details of RCS-1 reference point are out of the scope of the current specification.

#### 7.5.2.y+1	Reference point RCS-2 (between recording server and location management server)

The RCS-2 reference point, which exists between recording server and location management server, is used by the recording server to request and receive location information from the location management server.

The RCS-2 reference point uses the HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling. The RCS-2 reference point uses SIP-2 reference point for transport and routing of subscription/notification related signalling.

#### 7.5.2.y+2	Reference point RCS-3 (between recording server and key management server)

The RCS-3 reference point, which exists between recording server and key management server, provides a means for the key management server to provide security related information (e.g. encryption keys) to the MC recording server.

The RCS-3 reference point shall use the HTTP-1 and HTTP-2 reference points for transport and routing of security related information to the recording server.

Editor’s note: RCS-3 shall be specified in 3GPP TS 33.180 [25].

#### 7.5.2.y+3	Reference point RCS-4 (between recording server and configuration management server)

The RCS-4 reference point, which exists between recording server and configuration management server, is used by the recording server to request and receive information related to target users for the recordings.

The RCS-4 reference point shall use HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling. The RCS-4 reference point shall use SIP-2 reference point for transport and routing of subscription/notification related signalling.

#### 7.5.2.y+4	Reference point RCS-5 (between recording server and group management server)

The RCS-5 reference point, which exists between recording server and group management server, is used by the recording server to request and receive information related to target groups for the recordings.

The RCS-5 reference point shall use HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling. The RCS-5 reference point shall use SIP-2 reference point for transport and routing of subscription/notification related signalling.

#### 7.5.2.y+5	Reference point RCS-6 (between recording server and MC service servers)

The RCS-6 reference point, which exists between recording server and MC service servers, is used for transmitting metadata and media of the target user’s and target groups.

The RCS-6 reference point shall use the HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling. The RCS-6 reference point shall use SIP-2 reference point for transport and routing of subscription/notification related signalling.

#### 7.5.2.y+6	Reference point RCS-7 (between recording server and mass storage(s))

The RCS-7 reference point, which exists between recording server and mass storage(s), is used for storing the logged MCPTT, MCData, and MCVideo metadata as well as the recorded media of MCX Service Group Communications and MCX Service Private Communications. RCS-7 is used also to retrieve the recorded data and media when requested by an authorized user.

The details of RCS-7 are out of the scope of the current specification.

# * * * * 6th change * * * *

### 8.1.1	Mission Critical user identity (MC ID)

The mission critical user identity is also known as the MC ID. The MC ID is the identity that an MC service user or MC Replay client user presents to the identity management server during a user authentication transaction. In general, since identity management is a common service it uses an identity which is linked to a set of credentials (e.g. biometrics, secureID, username/password) that may not necessarily be tied to a single mission critical service. The MC ID and the MC service ID may be the same. The MC ID uniquely identifies the MC service user or MC Replay client user to the identity management server. The MC ID is used by the identity management server to provide the identity management client a means for mission critical service authentication.

NOTE:	The specific security and authentication mechanisms required in order to use the MC user identity is specified in 3GPP TS 33.180 [25].

# * * * * 7th change * * * *

### 8.1.z	MC replay UE label

The optional MC replay UE label allows to distinguish between different MC replay UEs in use by the same MC service ID. The MC replay UE label may be added to location information reports.

The non-routable MC replay UE label may include human readable information, such as an incident or MC service user specific ID, manufacturer name, brand, model, serial number, etc.

NOTE:	The MC replay UE label may be provided during initial MC replay UE configuration, see clause A.n.

Editor's note:	Whether MC replay UE label is needed is FFS.

# * * * * 8th change * * * *

# A.n	Initial MC replay UE configuration data

Editor`s Note:	The content of this clause is FFS.

# * * * * End of changes * * * *