3GPP TSG-SA WG6 Meeting #62	S6-243042

# Maastricht, The Netherlands, 19-23 August 2024

Source:	Kontron Transportation France, Nokia, Huawei

Title:	Discussion paper on Interworking ad hoc group calls with GSM-R

Agenda item:	9.3

Document for:	DISCUSSION

- Discussion

1.1	Information regarding GSM-R

The goal of the interworking with GSM-R is to support interworking between MCPTT and GSM-R Voice Group Call Service (VGCS). VGCS is a GSM-R specific type of group call that has includes users in the group call defined with the following attributes:

- Group call area. In GSM-R this is a list of cell IDs that form the group call area.

- List of dispatchers that are included in the group call

- List of dispatchers that are allowed to initiate the group call

- List of dispatchers that are allowed to terminate the group call

Besides the group call area, the group call ID (GID) is used.

The Group call reference (GCR) is the concatenation of the group call ID (as the least significant part) and the group call area ID (as the most significant part).  Each voice group call in one network is uniquely identified by its Group call reference.

The attributes of all the group calls are stored in the group call register (GCR) on the GSM-R call server.

VGCS GSM-R group call is defined by help of a specific called number:

50SSSSSXXX

With 50	VGCS

SSSSS	Service Area

XXX	GID

The most important GIDs are:

200	Train groups default group

299	Railway emergency call

Group call initiated by a mobile user (service subscriber)

A service subscriber initiating a voice group call has to call the wanted group ID. The MSC in which the call is originated shall accumulate from the BSS the called group ID and the originating cell ID.

In case of dispatcher originated call the identity used by the MSC to interrogate the GCR shall consist of the group call reference.

Example of a service subscriber originated GSM-R group call:

Called number is: 299	The service subscriber is in the service area 12345. The MSC will determine the GCR based on the originating cell ID. Then the MSC will include all the drivers of the trains that are in this service area and all the dispatchers as defined in the GCR for this group call area.

Example of a dispatcher originated GSM-R group call:

Called number is: 5012345299	This is a railway emergency call in the service area 12345. It will include all the drivers of the trains that are in this service area and all the dispatchers as defined in the GCR for this group call area.

In both cases the terminating users receive the CT5 FN that uniquely identifies the group call in the service area in the incoming group call request.

As one of the main conditions for the interworking solution is to avoid changes on the GSM-R side, the solution is based on using the VGCS group call number on the GSM-R side.

1.2	Information regarding MCPTT

For FRMCS it was decided to use the ad-hoc group call model based on criteria to identify the participants of the group calls. From 3GPP perspective the criteria-based call is very flexible (see 3GPP TS 23.379), i.e. the content is mostly left to implementation or controlled by the operator.

Nevertheless, some criteria were specified by FRMCS. The most important ones are:

- FRMCS_CallType:	For REC the FRMCS_CallType shall be FRMCS_CT_REC

- FRMCS_UseInitatingArea		Boolean

- FRMCS_UseAdressedArea	Boolean

- FRMCS_ListOfAdressedArea	For the first release of FRMCS only one area is supported. In later release multiple areas might be possible.

In later releases additional criteria might be added.

1.3	Interworking principle

1.3.1	A group is call initiated by a GSM-R user

Pre-conditions:

1.	The group call register (GCR) of the GSM-R system contains an additional entry under terminating dispatchers for a user on the IWF that represents the location defined group on the MCPTT system (a CT5 number).

Figure 1.3.1: Group call initiated by GSM-R user on an interworking group defined in the GSM-R system

- Triggered by a group call in GSM-R the GSM-R system sends a GSM-R IWF group call request to the IWF.

2.	Based on the CT5 FN (see NOTE) received in step 1 the IWF determines the criteria to be used in the ad-hoc group call.

NOTE:	An international EIRENE CT 5 number has the following format: IC+50/1+SSSSS+GID. The IWF generates the criteria based on the service area (SSSSS) and the GID part of the EIRENE CT5 FN. For REC for example, GID 299 is mapped to criteria with the value of group call type that represents REC, and the FRMCS_ListOfAdressedArea derived from the. Service Area. GID 200 is mapped to criteria with the value of group call type that represents “Train groups default group” and FRMCS_ListOfAdressedArea derived from the. Service Area.

3.	The IWF sends an IWF MCPTT ad hoc group call request containing the criteria determined in step 2 to the MCPTT server.

4.	The MCPTT server determines the list of users to be included based on the criteria received in step 3.

5.	The MCPTT server sends individual MCPTT ad hoc group call requests to all selected MCPTT users.

6.	The MCPTT server sends an IWF MCPTT ad hoc group call response back to the IWF.

7.	The IWF sends an IWF GSM-R group call response to the GSM-R system.

8.	The media plane is established between the group participants in the MCPTT system and in the GSM-R system.

1.3.2	A group is call initiated by an MCPTT user

Pre-conditions:

1.	The configuration on the MCPTT server for the criteria used in the ad hoc group call contains an additional entry with an MCPTT user that represents the corresponding CT5 number in GSM-R.

2.	The IWF contains an individual MCPTT user that is mapped to the CT5 number in GSM-R.

3.	The group call register (GCR) of the GSM-R system contains an additional entry under dispatchers for a user on the IWF that represents the location defined group on the MCPTT system.

NOTE 1:	This entry is required to authenticate the Dispatcher Leg on GSM-R GCR and allow group call establishment.

Figure 1.3.2: Group call setup initiated by MCPTT user on an interworking group defined in MCPTT system

1.	The user at MCPTT client 1 would like to initiate an ad hoc group call to a group that is involving a GSM-R system.

2.	The MCPTT client 1 sends an ad hoc group call request containing the details of the criteria to be applied by the MCPTT server.

3.	The MCPTT server accepts the ad hoc group call request checks if the ad hoc group call is supported and authorized.

4.	The MCPTT server shall send an ad hoc group call request return message to MCPTT client 1 containing the following information elements:

i.	The MCPTT ad hoc group ID generated by the MCPTT server;

ii.	The group ID of the pre-configured group to be used for the ad hoc group call (only included when the ad hoc group call is authorized); and

iii.	Result of whether the ad hoc group call is authorized or not

If the ad hoc group call request is not authorized, the MCPTT server and client 1 shall not proceed with the rest of the steps.

5.	The MCPTT server determines the list of users to be included in the ad hoc group call.

NOTE 2:	Based on the criteria provided in step 2 and based on configuration within the MCPTT server, the MCPTT server determines an MCPTT user representing the GSM-R group to be included in the ad hoc group call. Additionally, the list of users to be included normally contains MCPTT users matching the criteria.

6.	The MCPTT server sends individual MCPTT ad hoc group call requests to all MCPTT users determined in step 5.

7.	The MCPTT server sends an IWF MCPTT ad hoc group call requests to the IWF inviting the MCPTT user representing the GSM-R group determined in step 5.

8.	Based on the criteria received in step 7 and based on configuration within the IWF, the IWF determines the CT5 FN to be used in the subsequent group call request towards GSM-R.

9.	The IWF sends an IWF GSM-R group call request to the GSM-R system.

10.	The GSM-R system sends an IWF GSM-R group call response to the IWF.

11.	The IWF sends an IWF ad hoc group call response to the MCPTT server.

12.	The media plane is established between the group participants in the MCPTT system and in the GSM-R system.

- Conclusion

The solution principles are

- The IWF translates the functional number into criteria when the call is initiated from a GSM-R user, and

- The IWF translates the criteria into a functional number when the call is initiated by an MCPTT user.

It is recommended to specify solutions based on the above principles.