3GPP TSG-SA WG6 Meeting #62	S6-243046

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on Spatial anchor management and discovery

Spec:	3GPP TS 23.437 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes normative text on spatial anchor creation and discovery.

2. Reason for Change

The following has been concluded in TR 23.700-21 regarding spatial anchor management and discovery.

i.	for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.

3. Conclusions

This pCR provides normative text for KI#1 from Solution #1 and Solution #2.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r23434]		3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[r23502]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

* * * Next Change * * * *

## 8.3	Spatial anchors management procedures

### 8.3.a	Spatial anchor creation

#### 8.3.a.1	General

Spatial anchor creation enables a spatial anchor management client or VAL server to create spatial anchor(s) with the spatial anchor management server.

#### 8.3.a.2	Procedure

Figure 8.3.a.2-1 illustrates the spatial anchor creation procedure.

Pre-conditions:

1.	The spatial anchors management client or VAL server has received information (e.g. URI, IP address) related to the spatial anchors management server;

2.	The spatial anchors management client or VAL server has received security credentials authorizing it to communicate with the spatial anchors management server.

Figure 8.3.a.2-1: Spatial anchor creation

1.	The requestor (e.g., spatial anchors management client or VAL server) sends a spatial anchor create request to the spatial anchors management server. The request includes a requestor identifier, security credentials, spatial anchor position, spatial anchor service area, and an application service identifier. The request may include access control rules defining which entities are permitted to discover and access the spatial anchor, and discoverability information (e.g., indicating whether the spatial anchor is discoverable by any user). The request may also include application layer specific spatial anchor information (e.g., application layer information about a product associated with a spatial anchor) that remains transparent to the spatial anchors management service and that can be used during spatial anchor discovery. The request may include information to create multiple spatial anchors.

2.	Upon receiving the request, the spatial anchors management server validates if the requestor is authorized for the request. If the requestor is authorized, the spatial anchors management server creates the spatial anchor(s), assigns spatial anchor identifier to the newly created spatial anchor(s) and stores the spatial anchor information.

3.	The spatial anchors management server sends a spatial anchor create response to the requestor. If the spatial anchors management server created a spatial anchor, the response includes an indication of success and spatial anchor identifier(s) of newly created spatial anchor(s). Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.3.a.3	Information flows

Editor’s Note: The definition of information flows spatial anchor creation is FFS.

### 8.3.b	Spatial anchor update

#### 8.3.b.1	General

Spatial anchor update enables a spatial anchors management client or VAL server to update spatial anchor(s) with the  anchors management server.

#### 8.3.b.2	Procedure

Figure 8.3.b.2-1 illustrates the spatial anchor update procedure.

Pre-conditions:

1.	The spatial anchors management client or VAL server has received information (e.g. URI, IP address) related to the spatial anchors management server; and

2.	The spatial anchors management client or VAL server has received security credentials authorizing it to communicate with the spatial anchors management server.

Figure 8.3.b.2-1: Spatial anchor update

1.	The requestor (e.g., spatial anchors management client or VAL server) sends a spatial anchor update request to the spatial anchors management server. The request includes a requestor identifier, security credentials, the identifier of a spatial anchor, and spatial anchor information to update. The request may include information to update multiple spatial anchors.

2.	Upon receiving the request, the spatial anchors management server validates if the requestor is authorized for the request. If the requestor is authorized and the spatial anchor identifier provided in the request is found, the spatial anchors management server updates the spatial anchor information based on information provided in the request.

3.	The spatial anchors management server sends an spatial anchor update response to the requestor. If the spatial anchors management server updated the spatial anchor, the response includes an indication of success and the identifier of updated spatial anchor(s). Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.3.b.3	Information flows

Editor’s Note: The definition of information flows for spatial anchor update is FFS.

### 8.3.c	Spatial anchor deletion

#### 8.3.c.1	General

Spatial anchor deletion enables a spatial anchors management client or VAL server to delete spatial anchor(s) with the spatial anchors management server.

#### 8.3.c.2	Procedure

Figure 8.3.c.2-1 illustrates the spatial anchor deletion procedure.

Pre-conditions:

1.	The spatial anchors management client or VAL server has received information (e.g. URI, IP address) related to the spatial anchors management server; and

2.	The spatial anchors management client or VAL server has received security credentials authorizing it to communicate with the spatial anchors management server.

Figure 8.3.c.2-1: Spatial anchor deletion

1.	The requestor (e.g., spatial anchors management client or VAL server) sends a spatial anchor delete request to the spatial anchors management server. The request includes a requestor identifier, security credentials, the identifier of the spatial anchor to delete. The request may include information to delete multiple spatial anchors.

2.	Upon receiving the request, the spatial anchors management server validates if the requestor is authorized for the request. If the requestor is authorized and the spatial anchor identifier provided in the request is found, the spatial anchors management server deletes the spatial anchor information.

3.	The spatial anchors management server sends a spatial anchor delete response to the requestor. If the spatial anchors management server deleted the spatial anchor(s), the response includes an indication of success and the identifier of deleted spatial anchor(s). Otherwise, the response includes an indication of failure and may include a reason for failure.

#### 8.3.c.3	Information flows

Editor’s Note: The definition of information flows for spatial anchor deletion is FFS.

### 8.3.d	Spatial anchor discovery

#### 8.3.d.1	General

Spatial anchor discovery enables a spatial anchors management client or VAL server to discover spatial anchor(s) with the  anchors management server. The discovery of spatial anchor(s) is based on matching spatial anchor discovery filters provided in the request.

#### 8.3.d.2	Procedure

Figure 8.3.d.2-1 illustrates the procedure for spatial anchor discovery following the request-response model.

Pre-conditions:

1.	The spatial anchors management client or VAL server has received information (e.g. URI, IP address) related to the spatial anchors management server; and

2.	The spatial anchors management client or VAL server has received security credentials authorizing it to communicate with the spatial anchors management server.

Figure 8.3.d.2-1: Spatial anchor discovery

1.	The requestor (e.g., spatial anchors management client or VAL server) sends a spatial anchor discovery request message to the spatial anchors management server to obtain information about spatial anchors of interest. The request includes a requestor identifier, security credentials, and may include spatial anchor discovery filters. Spatial anchor discovery filters may include the location(s) of interest, information about the application service(s) of interest (e.g., service identifier, service type, service provider identifier, time of day when service is requested), and application specific user preferences. If the discovery request was triggered by a VAL client on a UE, the spatial anchor discovery request may additionally include a VAL client identifier and VAL client type.

2.	Upon receiving the request, the spatial anchors management server validates if the requestor is authorized for the request. If the requestor is authorized, the spatial anchors management server identifies spatial anchor(s) that satisfies anchor discovery filters and application specific user preferences.

NOTE 1:	Application specific user preferences can be compared with application specific spatial anchor information.

For each matching spatial anchor, the spatial anchors management server checks whether the requestor is authorized to obtain the spatial anchor based on the identity and location of the requestor, and based on the access control rules and service area characteristics of the spatial anchor. To determine the location of interest of the requestor, the spatial anchors management server may use location information provided in the request. The spatial anchors management server may obtain location information by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [r23273] and 3GPP TS 23.502 [r23502], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [r23434]. Additionally, the spatial anchors management server may obtain and consider data analytics, such as UE mobility prediction, during spatial anchor determination. The spatial anchors management server may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [r23288] and 3GPP TS 23.502 [r23502].

3.	The spatial anchors management server sends a spatial anchor discovery response to the requestor. If the spatial anchors management server has identified spatial anchor(s), the response includes an indication of success and the identified spatial anchors and may includes the recommended list of spatial anchors. Otherwise, the response includes an indication of failure and may include a reason for failure.

NOTE 2:	The logic to generate a recommended list of spatial anchors is out of scope and implementation specific.

4.	Upon receiving the spatial anchor discovery response, if the response indicates success, the requestor can store the spatial anchor information in a spatial anchor cache. The requestor evaluates the validity conditions associated with the discovered spatial anchors prior to using the spatial anchor. If the requestor is the spatial anchors management client, the spatial anchors management client can provide spatial anchor information to the VAL client(s). Upon receiving valid spatial anchor information, VAL client(s) can access the service(s) associated with each spatial anchor.

#### 8.3.d.3	Information flows

Editor’s Note: The definition of information flows for spatial anchor discovery is FFS.