3GPP TSG-SA WG6 Meeting #62	S6-243047

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on Spatial anchor subscription

Spec:	3GPP TS 23.437 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes normative text on Spatial anchor subscriptions.

2. Reason for Change

The following has been concluded in TR 23.700-21 regarding spatial anchor subscriptions.

i.	for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.

3. Conclusions

This pCR provides normative text for KI#1 Solution #3.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.1.0.

* * * First Change * * * *

## 8.3	Spatial anchors management procedures

### 8.3.y	Spatial anchor subscription

#### 8.3.y.1	General

Spatial anchor subscription enables a spatial anchors management client or VAL server to subscribe with the spatial anchors management server to receive notifications related to spatial anchors.

#### 8.3.y.2	Procedures

##### 8.3.y.2.1	Subscribe

Figure 8.3.y.2.1-1 illustrates the spatial anchor subscription procedure.

Pre-conditions:

1.	The spatial anchors management client or VAL server has received information (e.g. URI, IP address) related to the spatial anchors management server; and

2.	The spatial anchors management client or VAL server has received security credentials authorizing it to communicate with the spatial anchors management server.

Figure 8.3.y.2.1-1: Spatial anchor subscription

1.	The requestor (e.g., spatial anchors management client or VAL server) sends a spatial anchor subscribe request to the spatial anchors management server. The request includes a requestor identifier, security credentials, a notification endpoint, and event information for which the requestor is subscribing. The request may include an expiration time.

a.	The requestor may include the "discovery" event to indicate the spatial anchors management server to notify the requestor when the spatial anchors management server detects spatial anchors according to discovery filters provided in the request. Spatial anchor discovery filters may include the location(s) of interest, range to detect spatial anchors within a range, an indication to enable persistent search to notify spatial anchors within range of UE’s location, the current pose of the user along with direction of the movement, information about the application service(s) of interest (e.g., service identifier, service type, service provider identifier, time of day when service is requested), and application layer specific user preferences. If the requestor is a spatial anchors management client, the spatial anchor subscribe request may additionally include a VAL client identifier, VAL client type, and an indication whether the event is for spatial anchors becoming in range, out of range, or both of the UE.

b.	The VAL server may include the "UE with spatial anchor interest" event to indicate the spatial anchors management server to notify the VAL server when the spatial anchors management server detects UE(s) with interest in a specific spatial anchor according to the spatial anchor of interest detection filters provided by the VAL server. The detection filters may include spatial anchor identifiers to detect UEs that have discovered specific spatial anchors, spatial anchor associated service identifiers to detect UEs that have discovered spatial anchors associated with a service identifier, location area to detect UEs that have discovered spatial anchors within a location area, spatial anchor range to detect UEs that have discovered and are within range or become out of range of spatial anchors. The event request may also indicate whether the event is for a UE entering the range, leaving the range or both.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the spatial anchors management server creates the subscription and stores the subscription information.

a.	If the subscribed event is for "discovery" and the requestor is a spatial anchors management client, the spatial anchors management server starts monitoring the corresponding UE location and pose information. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [r23273] and 3GPP TS 23.502 [r23502], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [r23434]. Additionally, the spatial anchors management server may consider data analytics, such as UE mobility prediction. The spatial anchors management server may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [r23288] and 3GPP TS 23.502 [r23502]. The spatial anchors management server may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs becomes in-range or out-of-range of the spatial anchors of interest.

b.	If the subscribed event is for "UE with spatial anchor interest", the spatial anchors management server considers if the spatial anchors of interest have been discovered by a UE and obtains the corresponding UE location information. The UE location information may be obtained by using capabilities of the 5GS by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [r23273] and 3GPP TS 23.502 [r23502], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [r23434]. Additionally, the spatial anchors management server may consider data analytics, such as UE mobility prediction. The spatial anchors management server may obtain data analytics by invoking NEF APIs as described in 3GPP TS 23.288 [r23288] and 3GPP TS 23.502 [r23502]. The spatial anchors management server may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

3.	The spatial anchors management server sends a spatial anchor subscribe response to the requestor. If the spatial anchors management server has created the subscription, the response includes an indication of success, the subscription identifier and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the spatial anchors management server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.3.y.2.2	Notify

Figure 8.3.y.2.2-1 illustrates the spatial anchor notify operation between the spatial anchors management server and a spatial anchors management client or VAL server.

Pre-conditions:

- The spatial anchors management client or VAL server has subscribed for spatial anchor notifications with the spatial anchors management server;

Figure 8.3.y.2.2-1: Spatial anchor notification

1.	An event occurs at the spatial anchors management server that satisfies trigger conditions for notifying a subscriber (e.g., spatial anchors management client or VAL server) according to subscribed events.

a.	If the subscribed event is for "discovery" and the spatial anchors management server detects a change (e.g., creation, update, deletion, in-range, out-of-range) related to a spatial anchor, the spatial anchors management server determines the spatial anchors that satisfy the discovery filters of the subscription and notifies the subscriber accordingly;

b.	If the subscribed event is for "UE with spatial anchor interest" and the spatial anchors management server detects UE(s) with interest in spatial anchors (e.g., UE that discovers the monitored spatial anchor), the spatial anchors management server identifies whether the UE enters or exits the monitored spatial anchor range and notifies the subscriber accordingly.

2.	The spatial anchors management server sends a spatial anchor notification to the requestor indicating the event. The notification includes a subscription identity and includes event-specific information.

a.	If the subscribed event is for "discovery", the notification includes the list of determined spatial anchors and may include information about the detection of spatial anchors of interest (e.g., spatial anchor identifier(s), range from the UE, etc.). The notification may include the recommended list of spatial anchors based on the discovery filter and spatial anchor information.

NOTE:	Determining the recommended list of spatial anchors is out of scope and implementation specific.

b.	If the subscribed event is for "UE with spatial anchor interest", the notification includes information about UE with spatial anchor interest (e.g., UE identifier, spatial anchor identifier(s), range from the spatial anchor(s), entering or exiting the range, etc.).

##### 8.3.y.2.3	Subscription update

Figure 8.3.y.2.3-1 illustrates the procedure for a spatial anchors management client or VAL server to update a subscription with the spatial anchors management server.

Pre-conditions:

- The spatial anchors management client or VAL server has subscribed for spatial anchor notifications with the spatial anchors management server;

Figure 8.3.y.2.3-1: Spatial anchor subscription update

1.	The requestor (e.g., spatial anchors management client or VAL server) sends a spatial anchor subscription update request to the spatial anchors management server. The request includes the requestor identifier, security credentials and the subscription identifier and may include subscribed event information and expiration time.

2.	Upon receiving the request from the requestor, the spatial anchors management server validates if the requestor is authorized for the request. If the requestor is authorized, the spatial anchors management server updates the subscription information.

3.	The spatial anchors management server sends a spatial anchor subscription update response to the requestor. If the spatial anchors management server has updated the subscription, the response includes an indication of success and may include an expiration time. To maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the spatial anchors management server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.3.y.2.4	Unsubscribe

Figure 8.3.y.2.4-1 illustrates the procedure for a spatial anchors management client or VAL server to unsubscribe with the spatial anchors management server.

Pre-conditions:

- The spatial anchors management client or VAL server has subscribed for spatial anchor notifications with the spatial anchors management server;

Figure 8.3.y.2.4-1: Spatial anchor unsubscribe

1.	The requestor (e.g., spatial anchors management client or VAL server) sends a spatial anchor unsubscribe request to the spatial anchors management server. The request includes the requestor identifier, security credentials and the subscription identifier.

2.	Upon receiving the request from the requestor, the spatial anchors management server validates if the requestor is authorized for the request. If the requestor is authorized, the spatial anchors management server cancels the subscription associated with the subscription identifier.

3.	The spatial anchors management server sends a spatial anchor unsubscribe response to the requestor. If the spatial anchors management server has canceled the subscription, the response includes an indication of success. If the spatial anchors management server has not canceled the subscription, the response includes an indication of failure and may include a reason for failure.

#### 8.3.y.3	Information flows

Editor’s Note: The definition of information flows for spatial anchor subscription is FFS.