3GPP TSG-SA WG6 Meeting #62	S6-243049

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on AIML model distribution

Spec:	3GPP TS 23.482 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This CR brings normative text on AIML model distribution.

2. Reason for Change

The following agreement has been approved in TR 23.700-82 on transfer of AI/ML models.

The following principles will be considered in the normative phase for KI #5.

Transfer of AI/ML models:

-	Solution #24 will be considered for distribution of AIML models (request/response).

-	Solution #25 will be considered for distribution of AIML models (subscribe/notify).

NOTE 2: Possible alignment of the request/response and subscribe/notify models in Solution #24 and Solution #25 will be considered in normative phase.

Solution #24 and Solution #25 have a complementary relationship.

3. Conclusions

This pCR provides normative text for KI#5 Solution #24 and solution #25.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.1.0.

* * * First Change * * * *

## 8.x	Model Distribution

### 8.x.1	General

Model distribution enable applications consuming services from the AI/ML application enablement layer to discover and obtain AI/ML models available with the enablement layer.

Model distribution enables an AIMLE client or VAL server to obtain AI/ML models of interest. The distribution of AI/ML models is based on matching model distribution filters provided in the request.

Model distribution subscription enables an AIMLE client or VAL server to receive notifications about AI/ML models of interest. The subscription is based on matching model distribution filters provided in the subscription request.

### 8.x.2	Procedure

#### 8.x.2.1	General

Following procedures are supported for model distribution:

-	Model distribution request-response procedure;

-	Model subscribe-notify procedures for distribution of models, comprising:

-	Subscription procedure;

-	Subscription update procedure; and

-	Unsubscribe procedure;

#### 8.x.2.2	Model distribution

Figure 8.x.2.2-1 illustrates the procedure for an AIMLE client or VAL server to discover and obtain AI/ML models from the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has  received information (e.g. URI, IP address) related to the AIMLE Server;

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.x.2.2-1: Model distribution

1.	The requestor (e.g., AIMLE client or VAL server) sends a model distribution request to the AIMLE server. The request includes the requestor identifier along with security credentials and may include model distribution filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

If the requestor is authorized, the AIMLE server may determine if the requested model is available with the AIMLE server based on model distribution filters. Otherwise, the AIMLE server sends a request to the ML model repository to obtain the model; the request sent to the ML repository includes the model distribution filters.

Editor’s Note: Whether details of AIMLE server interactions with the ML repository are defined in this clause or a different subclause is FFS.

Editor’s Note: Whether details of client member selection and ML training request (e.g., solution 24) is defined in this clause or a different subclause is FFS.

3.	The AIMLE server sends a model distribution response to the requestor. If the AIMLE server has determined AI/ML models (e.g., locally, or from the ML repository), the response includes an indication of success and may include the AI/ML models. Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including AI/ML models, the requestor can store the AI/ML models and may provide the AI/ML model to the VAL client(s) if the requestor is the AIMLE client.

#### 8.x.2.3	Model distribution subscription

##### 8.x.2.3.1	General

Clause 8.x.2.3.2 and clause 8.x.2.3.3 together illustrate the model distribution subscribe/notify model.

Clause 8.x.2.3.4 illustrates the model distribution subscription update procedure.

Clause 8.x.2.3.5 illustrates the model distribution unsubscribe procedure.

##### 8.x.2.3.2	Subscribe

Figure 8.x.2.3.2-1 illustrates the procedure for an AIMLE client or VAL server to subscribe with the AIMLE server to be notified of distribution of AI/ML models.

Pre-conditions:

- The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

- The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.x.2.3.2-1: Model distribution subscription

1.	The requestor (e.g., AIMLE client or VAL server) sends a model distribution subscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and may include model distribution filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server creates the subscription and stores the subscription information.

3.	The AIMLE server sends a model distribution subscribe response to the requestor. If the AIMLE server has created the subscription, the response includes an indication of success, the subscription identity and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the model distribution subscription expires. If the AIMLE server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.3.3	Notify

Figure 8.x.2.3.3-1 illustrates the model distribution  notify operation between the AIMLE server and an AIMLE client or VAL server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for model distribution with the AIMLE Server;

Figure 8.x.2.3.3-1: Model distribution notification

1.	An event occurs at the AIMLE server that satisfies trigger conditions for notifying a subscriber (e.g., AIMLE client or VAL server). The event may be the detection of availability of new AI/ML model(s) (e.g., new model or new revision of a model) that satisfies the model distribution filters of the subscription.

2.	The AIMLE server sends a model distribution  notification to the requestor indicating newly available AI/ML models. The notification includes a subscription identity and newly available AI/ML model(s).

##### 8.x.2.3.4	Subscription update

Figure 8.x.2.3.4-1 illustrates the procedure for an AIMLE client or VAL server to update a subscription with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for model distribution with the AIMLE Server;

Figure 8.x.2.3.4-1: Model distribution subscription update

1.	The requestor (e.g., AIMLE client or VAL server) sends a model distribution update request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier and may include model distribution filters and expiration time.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the subscription information.

3.	The AIMLE server sends a model distribution subscription update response to the requestor. If the AIMLE server has updated the subscription, the response includes an indication of success and may include an expiration time. To maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the model distribution subscription expires. If the AIMLE server has not updated the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.x.2.3.5	Unsubscribe

Figure 8.x.2.3.5-1 illustrates the procedure for an AIMLE client or VAL server to unsubscribe with the AIMLE server.

Pre-conditions:

- The AIMLE client or VAL server has subscribed for model distribution with the AIMLE Server;

Figure 8.x.2.3.4-1: Model distribution unsubscribe

1.	The requestor (e.g., AIMLE client or VAL server) sends a model distribution unsubscribe request to the AIMLE server. The request includes the requestor identifier, security credentials and the subscription identifier.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server cancels the subscription.

3.	The AIMLE server sends a model distribution unsubscribe response to the requestor. If the AIMLE server has canceled the subscription, the response includes an indication of success. If the AIMLE server has not canceled the subscription, the response includes an indication of failure and may include a reason for failure.

### 8.x.3	Information flows

Editor’s Note: The definition of model distribution information flows is FFS.

* * * End of Changes * * * *