3GPP TSG-SA WG6 Meeting #62	S6-243062

Maastricht, Netherlands, 19th – 23th Aug 2024	(revision of S6-243xxx)

Source:	NTT DOCOMO

Title:	Add RNAA functions to the Functional Entities

Spec:	3GPP TR 23.946 v0.5.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1. Introduction

This pCR presents the addition the RNAA fuctions to the Functional Entities.

2. Reason for Change

There are no descriptions of RNAA functions in subclause 4.3. Therefore, the additional descriptions of RNAA functions are needed.

This pCR will be helpful for the developers in understanding the RNAA and CAPIF.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.946.

* * * First Change * * * *

4.3	Functional Entities

API invoker: Typically provided by a 3rd party application provider who has service agreement with PLMN operator

The API invoker supports several CAPIF capabilities such as supporting authentication, obtaining authorization, and discovering service APIs and invoking service APIs.

CAPIF core function (CCF): Supports capabilities used by other functional entities, for example the following:

-	For the API invoker: authenticating the API invoker, providing authorization information, and service API discovery.

-	For the API exposing function: (AEF) providing the service API access policy, providing API routing information, and charging of service API invocations.

-	For the API publishing function: publishing and storing the service APIs information.

-	For the API management function: providing the service API invocation log for auditing, storing configurations of the API provider policies, and updating registration information of API provider domain functions.

A CAPIF core function can also interact with another CAPIF core function for API publishing and discovery.

API exposing function (AEF): The API exposing function (AEF) is the provider of the Service APIs and is also the service communication entry point of the Service API to API invokers.

API publishing function (APF): The API publishing function (APF) enables the API provider to publish the Service APIs information to enable the discovery of Service APIs by the API invoker.

API management function (AMF): The API management function (AMF) enables the API provider to manage service APIs such as querying the Service API invocation log for auditing, monitoring the events, and configuring the API provider policies.

Authorization function: The authorization function enables receiving authorization from the resourse owner function and providing the API invoker with the authorization information which is needed to access the resource owner’s resources. This function is used in RNAA scenarios.

Resource owner function (ROF): The resource owner function enables the authorization for resource access and managing and revoking authorization for resource access. This function is used in RNAA scenarios.