3GPP TSG-SA WG6 Meeting #62	S6-243080

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.700-82 | CR | 0019 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solution update in subclause 8.3.4 of TR 23.700-82 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AIMLAPP |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As stated in subclause 8.3.4 of TR 23.700-82, it is necessary to include the details of the corresponding API as part of the documentation. Currently, this information is not provided. Therefore, we will refer to subclause 8.8.3 of TR 23.700-82 and include the relevant information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is necessary to fill in the corresponding API for subclause 8.3.4, as it is currently not specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The details of the corresponding API are unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.3.4	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #3.

-	ML client provisioning request API (request/response model; API provider: AIMLE server; known consumer: AIML consumer (like VAL server, VAL UE); corresponding to step 1and step 2 of clause 8.3.2).

Table 8.3.4-1 shows the request sent by an AIML enabler service consumer to an AIML enabler service producer for the AIML enabler service consumer registration procedure.

Table 8.3.4-1: Request for AIML enabler service consumer identifier registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| AIML enabler service consumer | M | The identifier of the AI/ML enabler service consumer. |
| Security credentials | M | Security credentials to authenticate and authorize the AIML enabler service consumer. |
| AIML enabler service consumer profile | M | Information about the capability of the AIML enabler service consumer to support AIML operations as detailed in table 8.3.4-2. |
| List of supported services | O | Supported service information. |
| >List of VAL service ID(s) | O | List of VAL service ID(s) that the consumer supports |
| >Corresponding service permission level(s) | O | Service permission level(s) (e.g., raw data, processed data, premium resource usage standard resource usage, limited resource usage |

Table 8.3.4-2: AIML enabler service consumer profile

| Information element | Status | Description |
| --- | --- | --- |
| AIML consumer profile ID | M | Profile identifier. |
| Supported ML models | O | ML models supported by the AIML enablement service consumer |
| Supported AIML roles | M | AIML roles supported by the AIML enablement service consumer such as data collection, data preparation, training, or inferencing |
| Availability schedule | O | Availability schedule of the the AIML enablement client |
| Application layer AI/ML member capabilities | M | Application layer AI/ML member capability information (e.g. ML application type, client availability to support AIML operations at the UE). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of data descriptors |
| Data capabilities | O | Data capabilities such as data collection and supported data processing capabilities |

Table 8.3.4-3 shows the response sent by the AIML enabler service producer to the AIML enabler service consumer for the AIML enabler service consumer registration procedure.

Table 8.3.4-3: Response for AIML enabler service consumer registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIML consumer profile ID | M | An identifier for the AIML consumer profile |

* * * End of Changes * * * *