3GPP TSG-SA WG6 Meeting #62	S6-243088

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	UIC

Title:	Mission Critical Control and User Plane Separation (MCCUPS)

Agenda Item:	9.3 (FRMCS_Ph5)

Contact:	Guillaume Gach (gach@uic.org)

Abstract: FRMCS builds its capabilities on top of three ingredients: 5GS, SIP/IMS, and MCx. 5GS provides with Control and User Plane Separation (CUPS) already means to keep user traffic local and avoid unnecessary transport of traffic to a central location, while keeping the control at the central location. Unfortunately, the centralized approach of MCPTT breaks the advantageous approach of 5GS, since all MCx traffic still is required to run through a central location.

In this discussion paper we argue that it will be advantageous also for MCx to introduce Mission Critical Control and User Plane Separation (MCCUPS) and thus introduce the same benefits as 5GS CUPS, the avoidance of data transport forth to and back from a central location. Benefits are energy savings, associated costs savings, CO2e reduction, smaller failure units, quicker recovery, and more. In this discussion paper we want to introduce you to the potential workings of MCCUPS.

# Current Situation

Mission Critical Push-To-Talk (MCPTT) defines a single MCPTT Server, usually the one associated with the MC Client originating a (group) call, to carry the functional units for Signalling and Call Control, Floor Control and Media Distribution Function. Between those functional units no standardized interfaces do exist, such that they are all three either part of one physical unit or a proprietary solution will permit a separation of those units.

Centralization onto a single physical unit will force all traffic through that one centralized instance. As a consequence, traffic that could be kept local will need to be transported to the central unit and back, leading to waste of resources and capacities that would not be necessary. Figure 1 does illustrate that case.

Figure 1

Note that connectivity demand can become dramatic, if larger geographic areas are being covered, while at the same time terminations are actually concentrated in a relatively small area. Typical situations larger countries like Spain, France, Italy, Germany and Poland will face.

# FRMCS Specifics

In FRMCS the entities communicating with each other are persons on board of trains with persons along the trackside, supervising railway traffic in all kinds of combinations. In very few instances that kind of communication is not local. That leads in FRMCS to situations illustrated by Figure 2.

Figure 2: Communication of Person On-Board with Operations

A train has a need to communicate for some operational purpose. Most of the roles relevant for that purpose will be in close vicinity of the train. As a consequence, in this example all persons but one are local, all the traffic is transported back to the central MCPTT server and then returns to the MC Clients again. This figure exacerbates massively, if multi-talker capability is enabled, but media mixing only at the MC Clients. When that happens all information from originating talkers (m) needs to be replicated to all listeners, leading to n*m simultaneous streams, instead of n streams when there is strictly only one person admitted to the floor.

Such an approach will lead to waste of transport resources, which can be expressed in terms of energy costs and CO2e produced. In an ideal situation, there would only be a central control for setting up, tearing down and administering (group) calls, but otherwise keeping the traffic local. In order to achieve that, all tasks related to speech processing would gain most benefits when they are local too.

Also, traffic traveling along the illustrated paths will accrue latency. Depending on the number of routing or switching elements encountered and the distance travelled on physical medium. That may become relevant in the years after deployment and in particular when additional new maintenance applications will be introduced (e.g., real-time supervision of trains). Note that Blue-light Organizations share the similar long system life cycles, therefore looking out for the future is a important task.

A model that would help the railway infrastructure managers in supporting their operations structure, is to follow the CUPS approach of 5GS and introduce separate localized user plane handling functions for MCx. Once that has been done, rail communication will remain local.

# Mission Critical CUPS

Currently the interactions of a MCPTT Server are illustrated through the Figure 3. MCPTT Ctrl is the Application and Signalling Plane (see 3GPP TS24.380) that governs the set-up, modification and tear down of MCPTT (group) calls.

Figure 3: MCPTT Architecture acc. to 3GPP TS 23.289

All handling of the speech occurs at the media distribution function, with the floor control determining who has access to the speaker’s floor and can speak at that moment in time. Figure 4 shows the example of a possible internal architecture of the controlling MCPTT Server, with the Application and Signalling Plane on the top, the floor control and the media distribution function below.

Figure 4: Schematic view of a possible internal structure of an MCPTT Server (3GPP TS24.380)

MCCUPS would now introduce a cut in Figure 4 between the call handling and administrative functions of the MCPTT Server – i.e. Application and Signalling Plane – and the functional parts responsible for handling speech: floor control and media distribution function.

Figure 3 will change to Figure 5. Note in particular the split of the MCPTT Server into a pure control part MCPTT Control and the speech handling part, comprising the Media Distribution Function and the Floor Control. The little Cloud in between indicates that the MCMDF can be positioned anywhere in the network. Best position would be as close as possible to producers and consumers of the speech. That architecture is now similar to the 5GS CUPS architecture.

Figure 5: MC CUPS MCPTT Architecture acc. to 3GPP TS 24.380

In order to separate MCPTT control from floor control and media distribution function, an interface needs to be defined to permit the interaction between what is now called Mission Critical Media Distribution Function (MCMDF) and comprises Floor Control and Media Distribution Function. Note that reference point MCPTT-8 and MCPTT-9 have been left out, since FRMCS is currently not supporting MBS.

Note, for the MC Client a similar SW structure is being depicted, as we believe that reusing successful patterns will be able to reduce the efforts of implementation.

When applying MCCUPS to Figure 2 one can observe in Figure 6 how much simpler the picture becomes by having local MCMDFs. Clearly in order to realize that architecture the interface between the MCPTT Control and the MCMDF will need to be defined in the Standards. It is that Standardisation of the MCMCI interface (see Figure 5) that we are proposing with that discussion paper in order to realize MCCUPS.

Figure 6: Communication of Person On-Board with Operations localized by MCCUPS

# Conclusion

In this short discussion paper, we tried to introduce the concept of MCCUPS. We have been guided by 5GS CUPS, which is very successful helping to save energy and costs for MNOs. The introduction of MCCUPS in FRMCS will require some CR into the 3GPP standards that we will need to drive through SA6.

We understand that railway operations are very much local and that 80% of all communication does happen locally. As a consequence, there are also benefits in scalability with an MCCUPS approach, as local traffic can grow more smoothly than centralized traffic backhauled to a single location.

Besides energy savings, which we could verify for FRMCS, however at a smaller size than 5GS CUPS savings of MNOs, we do see advantages in business resiliency (smaller failure units, faster failover times) and would also therefore recommend standardizing the MCMCI interface of MCCUPS.

An alternative proposition of using distributed MCPTT servers was evaluated yet showed us that there is higher risk in Standardisation, as there is no prior understanding of how MC Clients within the same administrative domain, can be migrated between different subregions of that domain – in order to achieve the mobility of the data traffic. Such capability would also need to be standardized if it shall work across vendors and is believed a higher effort than the work for MCCUPS.

We therefore recommend continuing working on MCCUPS and standardize the necessary new interface MCMCI. We hope that many of you find the proposition helpful for their own business and would welcome a common cooperation on that effort.