3GPP TSG-SA WG6 Meeting #62	S6- 243090

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Add new solution for KI#1 about resource owner consent check

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solution for KI#1 about resource owner consent check.

2. Reason for Change

This contribution provides solutions about resource owner checking the resource owner consent to the CAPIF core function.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.44.0.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |

* * * Second Change * * * *

6.15	Solution #14: Resource Owner Consent check

6.15.1	Solution description

6.15.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

This solution enables the resource owner to check its resource owner consent stored in the authorization function. The CAPIF core function sends a resource owner consent check request to the resource owner client with its CAPIF core function ID when it determines the check whether the resource owner consent has changed. For example, when the time period is expired, the CAPIF core function sends a resource owner consent check request to the resource owner client. The resource owner can check the resource owner consent and determind whether to change it. If it is, the resource owner consent update procedure is performed and otherwise do nothing.

6.15.1.2		Procedure for resource owner consent check

Pre-condition:

1.	The CAPIF core function determines the check whether the resource owner consent has changed.

Figure 6.15.1.2-1: Procedure for resource owner consent check

1. The CAPIF core function sends a resource owner consent check request to the resource owner client with its CAPIF core function ID when it determines the check whether the resource owner consent has changed. For example, when the time period is expired, the CAPIF core function sends a resource owner consent check request to the resource owner client.

2. The resource owner client checks the resource owner consent and responds to the CAPIF core function with resource owner ID and status which indicates whether the resource owner consent has changed. If it changed, the resource owner client sends resource owner consent and/or resource reference to the CAPIF core function.

3. If the status received in step 2 is “changed”, the CAPIF core function updates the resource owner client profile based on the information received in step 2.

4. After updating, the CAPIF core function sends a resource owner consent update respond to the resource owner client.

Note: If the status received in step 2 is “changed” and the resource owned by the resource owner can be published to other CAPIF core functions, the CAPIF core function shall send event notification to other CAPIF core functions with its CAPIF core function identity, resource owner identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.15.2	Architecture Impacts

6.15.3	Corresponding APIs

Table 6.15.3-1: Resource owner consent check request

| Information element | Status | Description |
| --- | --- | --- |
| CAPIF core function identity information | M | The information that uniquely identifies the CAPIF core function. |
| Resource owner consent | O | Resource owner consent used by the CAPIF core function to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Resource reference | O | The reference (set) provided by the CAPIF core function about the resource of resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |

Table 6.15.3-2: Resource owner consent check respond

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |
| Status | M | The status indicates whether the resource owner consent has changed. The value is “changed” or “not changed”. |
| Resource owner consent | O (see NOTE1) | Resource owner consent used by the CAPIF core function to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Resource reference | O (see NOTE2) | The reference (set) provided by the CAPIF core function about the resource of resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |
| NOTE1: Information element shall be present when status is “changed”. NOTE2: Information element shall be present when the resource owner client changes the resource owner consent in resource owner client profile. |  |  |