3GPP TSG-SA WG6 Meeting #62	S6- 243091

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Add new solution for KI#1 about resource owner consent provision

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solution for KI#1 about resource owner consent provision.

2. Reason for Change

This contribution provides solutions about resource owner providing, updating and checking the resource owner consent to the CAPIF core function.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |

* * * Second Change * * * *

6.13	Solution #12: Resource Owner Consent Provision

6.13.1	Solution description

6.13.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

This solution enables the resource owner to provide the resource owner consent to the authorization function in CAPIF core function so that the CAPIF core function and the AEF can use it to authorize the resource owner-aware northbound API invocation.

6.13.1.2		Procedure for resource owner consent provision

Pre-condition:

1.	The service API access requires obtaining consent from resource owner.

Figure 6.13.1.2-1: Procedure for resource owner consent provision

1. The resource owner client sends a resource owner consent provision request to the CAPIF core function/Authorization function with resource owner ID and resource owner consent.

2. The CAPIF core function/Authorization function generates the resource reference for each resource and updates the resource owner client profile based on the request.

3. The CAPIF core function/Authorization function responds the resource owner client.

Note: If the resource owned by the resource owner can be published to other CCFs, the CCF shall send event notification to other CCFs with its CCF identity, RO identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.13.2	Architecture Impacts

6.13.3	Corresponding APIs

Table 6.13.3-1: Resource owner consent provision request

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |
| Resource owner consent | M | Resource owner consent used by the CCF to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |

Table 6.13.3-3: Resource owner consent provision respond

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of creating or updating the resource owner consent |
| Resource reference | M | The reference (set) provided by the CAPIF core function about the resource of Resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |