3GPP TSG-SA WG6 Meeting #62	S6- 243092

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Add new solution for KI#1 about resource owner consent update

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solution for KI#1 about resource owner consent update.

2. Reason for Change

This contribution provides solutions about resource owner updating the resource owner consent to the CAPIF core function.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |

* * * Second Change * * * *

6.14	Solution #13: Resource Owner Consent Update

6.14.1	Solution description

6.14.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

This solution enables the resource owner to update the consent related to its resource to the authorization function in CAPIF core function. The consent is already stored on the authorization function in the provisioning procedure.

If there is an API invocation authorization associated with the consent, the revocation procedure may be performed. Otherwise, only update the consent.

6.14.1.2		Procedure for resource owner consent update

Pre-condition:

1.	The service API access requires obtaining consent from resource owner.

2.	The consent is already stored on the authorization function in the provisioning procedure.

Figure 6.14.1.2-1: Procedure for resource owner consent update

1. The resource owner client sends a Resource owner consent update request to the CAPIF core function/Authorization function with resource owner ID, the resource reference and the new resource owner consent referred to the resource reference.

2. If there is an API invocation authorization associated with the consent, the revocation procedure may be performed. Otherwise, the CAPIF core function/Authorization function updates the resource owner client profile based on the request.

3. The CAPIF core function/Authorization function responds the resource owner client.

Note: If the resource owned by the resource owner can be published to other CAPIF core function, the CAPIF core function shall send event notification to other CAPIF core function with its CAPIF core function identity, resource owner identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.14.2	Architecture Impacts

6.14.3	Corresponding APIs

Table 6.14.3-1: Resource owner consent update request

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |
| Resource owner consent | M | Resource owner consent used by the CAPIF core function to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Resource reference | M | The reference (set) provided by the CAPIF core function about the resource of resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |

Table 6.14.3-2: Resource owner consent create/update respond

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of creating or updating the resource owner consent |