3GPP TSG-SA WG6 Meeting #62	S6-243093

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Add new solution for KI#1 about resource owner deregistration

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solution for KI#1 about resource owner deregisteration.

2. Reason for Change

This contribution provides solution for KI#1 to enable the resource owner to deregister from the CAPIF when it no longer communicates with the CAPIF.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v 0.4.0

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |
| Sol #15 | X |  |  |  |  |  |  |
| Sol #16 | X |  |  |  |  |  |  |

* * * Second Change * * * *

6.17	Solution #16: Resource Owner deregistration

6.17.1	Solution description

6.17.1.1	General

This solution is for Key issue #1 about resource owner deregisteration.

This solution enables the resource owner to deregister from the CAPIF when it no longer communicates with the CAPIF. The resource owner client initializes the deregistration procedure with its resource owner ID. The CAPIF core function executes the deregistration process including deleting the resource owner client profile.

6.17.1.2		Procedure for resource owner deregistration

Figure 6.17.1.2-1: Procedure for resource owner deregistration

1. The resource owner client triggers resource owner client deregistration request towards the CAPIF core function with its resource owner identity.

2. The CAPIF core function cancels the deregistration of the API invoker from CAPIF and delete information related to the resource owner. This includes deletion of the resource owner consent and resource reference. Depending on the operator policy, the CAPIF core function may retain the information of the deregistered resource owner client.

3. The CAPIF core function sends resource owner client deregistration response message, indicating the successful deregistration of the resource owner client.

Note: If the resource owned by this resource owner has published to other CAPIF core functions, the CAPIF core function may send Event notification message to other CAPIF core functions to indicate that the resource owner client deregisteres.

6.17.2	Architecture Impacts

6.17.3	Corresponding APIs

Table 6.17.3-1: Resource owner client deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| CAPIF core function identity information | M | The information that uniquely identifies the CAPIF core function. |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |

Table 6.17.3-2: Resource owner client deregistration respond

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the resource owner deregistration |