3GPP TSG-SA WG6 Meeting #62	S6-243094

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Add new solution for KI#1 about resource owner registration

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solution for KI#1 about resource owner registeration.

2. Reason for Change

This contribution provides solution for KI#1 to enable the resource owner to register with the CAPIF in order that the CAPIF core function can identify and interact with the resource owner.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v 0.4.0

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |
| Sol #15 | X |  |  |  |  |  |  |

* * * Second Change * * * *

6.16	Solution #15: Resource Owner registration

6.16.1	Solution description

6.16.1.1	General

This solution is for Key issue #1 about resource owner registeration.

This solution enables that the resource owner registers with and provides information to the CAPIF in order that the CAPIF core function can identify it before it provides resource owner consent.

6.16.1.2		Procedure for resource owner registeration

Figure 6.16.1.2-1: Procedure for resource owner registration

1.	For supporting resource owner-aware northbound API invocation, the resource owner client triggers resource owner client registration request towards the CAPIF core function.

2.	The CAPIF core function begins the registration process by verifying whether all the necessary information has been provided to the resource owner client. Successful registration results in provisioning resource owner client profile which includes resource owner ID. If the resource owner client provides Resource owner consent in step 1, the CAPIF core function adds it in resource owner client profile and generates Resource reference for each resource.

3.	If the resource owner client has registered with the CAPIF core function, the CAPIF core function responds the resource owner client with registration successful message which may include the resource reference if the resource owner client provides resource owner consent in step 1.

4.	As a result of successful registration process, the resource owner client is able to interact with the CAPIF core function and provide, manage and revoke resource owner consent.

Note: If the resource owner client provides resource owner consent in step 1 and the resource owned by the resource owner can be published to other CAPIF core functions, the CAPIF core function shall send event notification to other CAPIF core functions with its CAPIF core function identity, resource owner identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.16.2	Architecture Impacts

6.16.3	Corresponding APIs

Table 6.16.3-1: Resource owner client registration request

| Information element | Status | Description |
| --- | --- | --- |
| registration information | M | The information of the resource owner client including registration details, required for registration, e.g., resource owner identity information |
| Resource owner consent | O | Resource owner consent used by the CAPIF core function to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Proposed expiration time | O | Proposed expiration time for the registration. |

Table 6.13.3-2: Resource owner client registration response

| Information element | Status | Description |
| --- | --- | --- |
| status | M | The result of registration request i.e., success indication is included if the resource owner client is registered with the CAPIF core function otherwise failure. |
| Enrolled information | O (see NOTE 1) | Information from the provisioned resource owner client profile which may include resource owner identity information and security information used for identify the resource owner. |
| Resource reference | O (see NOTE 2) | The reference (set) provided by the CAPIF core function about the resource of resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |
| Reason | O (see NOTE 3) | This element indicates the reason when status is failure. |
| Expiration time | O | Indicates the expiration time of the registration. At expiration, CAPIF core function cancels the registration of the resource owner client from CAPIF. If omitted, it indicates the registration does not expire. |
| NOTE 1: Information element shall be present when the status is successful. NOTE 2: Information element may be present when the status is successful and the resource owner client provides resource owner consent in resource owner client registration request. NOTE 3: Information element shall be present when the status is failure. |  |  |