3GPP TSG-SA WG6 Meeting #62	S6-243095

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Add new solutions for KI#1

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solutions for KI#1, including resource owner consent provision, update and check.

2. Reason for Change

This contribution provides solutions about resource owner providing, updating and checking the resource owner consent to the CAPIF core function.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0.

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |

* * * Next Change * * * *

6.13	Solution #12: Resource Owner Consent Provision

6.13.1	Solution description

6.13.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

This solution enables the resource owner to provide the resource owner consent to the authorization function in CAPIF core function so that the CAPIF core function and the AEF can use it to authorize the resource owner-aware northbound API invocation.

6.13.1.2		Procedure for resource owner consent provision

Pre-condition:

1.	The service API access requires obtaining consent from resource owner.

Figure 6.13.1.2-1: Procedure for resource owner consent provision

1. The resource owner client sends a Resource owner consent provision request to the CAPIF core function/Authorization function with resource owner ID and resource owner consent.

2. The CAPIF core function/Authorization function generates the resource reference for each resource and updates the resource owner client profile based on the request.

3. The CAPIF core function/Authorization function responds the resource owner client.

Note: If the resource owned by the resource owner can be published to other CCFs, the CCF shall send event notification to other CCFs with its CCF identity, RO identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.13.2	Architecture Impacts

6.13.3	Corresponding APIs

Table 6.13.3-1: Resource owner consent provision request

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |
| Resource owner consent | M | Resource owner consent used by the CCF to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |

The Resource owner consent is defined as below:

Table 6.13.3-2: Resource owner consent

| Information element | Status | Description |
| --- | --- | --- |
| Resource information | M | The resource information includes resource name, resource description, serving area information (optional), allowed operation, allowed API invoker lists (optional). |
| Time period | O | The time period indicates when the resource information will expire. |
| Shareable information | O (see NOTE) | Indicates whether the resource owned by the resource owner can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the resource can be published is contained. |
| NOTE: If the shareable information is not present, the resource owned by the resource owner is not allowed to be shared. |  |  |

Table 6.13.3-3: Resource owner consent provision respond

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of creating or updating the resource owner consent |
| Resource reference | M | The reference (set) provided by the CAPIF core function about the resource of Resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |

6.14	Solution #13: Resource Owner Consent Update

6.14.1	Solution description

6.14.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

This solution enables the resource owner to update the consent related to its resource to the authorization function in CAPIF core function. The consent is already stored on the authorization function in the provisioning procedure.

If there is an API invocation authorization associated with the consent, the revocation procedure may be performed. Otherwise, only update the consent.

6.14.1.2		Procedure for resource owner consent update

Pre-condition:

1.	The service API access requires obtaining consent from resource owner.

2.	The consent is already stored on the authorization function in the provisioning procedure.

Figure 6.14.1.2-1: Procedure for resource owner consent update

1. The resource owner client sends a Resource owner consent update request to the CAPIF core function/Authorization function with resource owner ID, the resource reference and the new resource owner consent referred to the resource reference.

2. If there is an API invocation authorization associated with the consent, the revocation procedure may be performed. Otherwise, the CAPIF core function/Authorization function updates the resource owner client profile based on the request.

3. The CAPIF core function/Authorization function responds the resource owner client.

Note: If the resource owned by the resource owner can be published to other CCFs, the CCF shall send event notification to other CCFs with its CCF identity, resource owner identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.14.2	Architecture Impacts

6.14.3	Corresponding APIs

Table 6.14.3-1: Resource owner consent update request

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |
| Resource owner consent | M | Resource owner consent used by the CCF to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Resource reference | M | The reference (set) provided by the CAPIF core function about the resource of Resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |

Table 6.14.3-2: Resource owner consent create/update respond

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of creating or updating the resource owner consent |

6.15	Solution #14: Resource Owner Consent check

6.15.1	Solution description

6.15.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

This solution enables the resource owner to check its resource owner consent stored in the authorization function. The CCF sends a Resource owner consent check request to the resource owner client with its CCF ID when it determines the check whether the resource owner consent has changed. For example, when the time period is expired, the CCF sends a Resource owner consent check request to the resource owner client. The resource owner can check the resource owner consent and determind whether to change it. If it is, the resource owner consent update procedure is performed and otherwise do nothing.

6.15.1.2		Procedure for resource owner consent check

Pre-condition:

1.	The CAPIF core function determines the check whether the resource owner consent has changed.

Figure 6.15.1.2-1: Procedure for resource owner consent check

1. The CAPIF core function sends a Resource owner consent check request to the resource owner client with its CCF ID when it determines the check whether the resource owner consent has changed. For example, when the time period is expired, the CAPIF core function sends a Resource owner consent check request to the resource owner client.

2. The resource owner client checks the resource owner consent and responds to the CAPIF core function with resource owner ID and status which indicates whether the resource owner consent has changed. If it changed, the resource owner client sends resource owner consent and/or resource reference to the CAPIF core function.

3. If the status received in step 2 is “changed”, the CAPIF core function updates the resource owner client profile based on the information received in step 2.

4. After updating, the CAPIF core function sends a resource owner consent update respond to the resource owner client.

Note: If the status received in step 2 is “changed” and the resource owned by the resource owner can be published to other CAPIF core functions, the CAPIF core function shall send event notification to other CAPIF core functions with its CCF identity, resource owner identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.15.2	Architecture Impacts

6.15.3	Corresponding APIs

Table 6.15.3-1: Resource owner consent check request

| Information element | Status | Description |
| --- | --- | --- |
| CCF identity information | M | The information that uniquely identifies the CCF. |
| Resource owner consent | O | Resource owner consent used by the CCF to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Resource reference | O | The reference (set) provided by the CAPIF core function about the resource of Resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |

Table 6.15.3-2: Resource owner consent check respond

| Information element | Status | Description |
| --- | --- | --- |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |
| Status | M | The status indicates whether the resource owner consent has changed. The value is “changed” or “not changed”. |
| Resource owner consent | O (see NOTE1) | Resource owner consent used by the CCF to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Resource reference | O (see NOTE2) | The reference (set) provided by the CAPIF core function about the resource of Resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |
| NOTE1: Information element shall be present when status is “changed”. NOTE2: Information element shall be present when the resource owner client changes the resource owner consent in resource owner client profile. |  |  |