3GPP TSG-SA WG6 Meeting #62	S6-243096

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Add new solutions for KI#8 in TR 23.700-22

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to add new solutions for KI#8, including onboarding resource owner to CAPIF and offboarding resource owner from CAPIF.

2. Reason for Change

This contribution provides solutions for KI#8 to enable the CAPIF core function to recognize a resource owner and maintain related information.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v 0.4.0

* * * First Change * * * *

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |  |
| Sol #6 |  |  |  |  |  |  | X |  |
| Sol #7 |  |  |  |  |  |  | X |  |
| Sol #8 |  |  |  |  |  |  | X |  |
| Sol #9 | X |  |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  |  | X |
| Sol #16 |  |  |  |  |  |  |  | X |

* * * Next Change * * * *

6.16	Solution #15: Onboarding Resource Owner to CAPIF

6.16.1	Solution description

6.16.1.1	General

This solution is for Key issue #8 to onboard the resource owner.

This solution enables that the resource owner is onboarded and provides information to the CAPIF so that the CAPIF core function can recognize it before it provides resource owner consent.

6.16.1.2		Procedure for onboarding resource owner to CAPIF

Figure 6.16.1.2-1: Procedure for onboarding resource owner

1. For supporting resource owner-aware northbound API invocation, the resource owner client triggers onboard resource owner client request towards the CAPIF core function.

2. The CAPIF core function begins the onboarding process by verifying whether all the necessary information has been provided to onboard the resource owner client. Successful onboarding results in provisioning resource owner client profile which includes resource owner ID. If the resource owner client provides Resource owner consent in step 1, the CAPIF core function adds it in resource owner client profile and generates Resource reference for each resource.

3. If the resource owner client has onboarded in the CAPIF core function, the CAPIF core function responds the resource owner client with onboard successful message which may include the resource reference if the resource owner client provides Resource owner consent in step 1.

4. As a result of successful onboarding process, the resource owner client is able to interact with the CAPIF core function and provide, manage and revoke resource owner consent.

Note: If the resource owner client provides resource owner consent in step 1 and the resource owned by the resource owner can be published to other CAPIF core functions, the CAPIF core function shall send event notification to other CAPIF core functions with its CAPIF core function identity, resource owner identity information, resource owner consent of the shareable resource, resource reference of the shareable resource.

6.16.2	Architecture Impacts

6.16.3	Corresponding APIs

Table 6.16.3-1: Onboard resource owner client request

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding information | M(see NOTE) | The information of the resource owner client including enrolment details, required for onboarding, e.g., resource owner identity information |
| Resource owner consent | O | Resource owner consent used by the CAPIF core function to authorize the API invoker prior to accessing the resource owner-aware northbound service API. |
| Proposed expiration time | O | Proposed expiration time for the onboarding. |
| NOTE: If the resource owner is UE, the resource owner ID is specified as the GPSI of the corresponding UE. If the resource owner is a MNO subscriber, the resource owner ID may be the subscriber ID. |  |  |

Table 6.13.3-2: Onboard resource owner client response

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding status | M | The result of onboarding request i.e., success indication is included if the resource owner client is onboarded in the CAPIF core function otherwise failure. |
| Enrolled information | O (see NOTE 1) | Information from the provisioned resource owner client profile which may include RO identity information and security information used for mutual authentication with the CAPIF core function. |
| Resource reference | O (see NOTE 2) | The reference (set) provided by the CAPIF core function about the resource of Resource owner consent in the resource owner client profile which can be used for reference by the resource owner client. |
| Reason | O (see NOTE 3) | This element indicates the reason when onboarding status is failure. |
| Expiration time | O | Indicates the expiration time of the onboarding. At expiration, CAPIF core function cancels the enrollment of the resource owner client from CAPIF. If omitted, it indicates the onboarding does not expire. |
| NOTE 1: Information element shall be present when onboarding status is successful. NOTE 2: Information element may be present when onboarding status is successful and the resource owner client provides resource owner consent in Onboard resource owner client request. NOTE 3: Information element shall be present when onboarding status is failure. |  |  |

6.17	Solution #16: Offboarding Resource Owner from CAPIF

6.17.1	Solution description

6.17.1.1	General

This solution is for Key issue #8 to offboard the resource owner.

This solution enables the resource owner offboard from the CAPIF. The resource owner client initializes the offboard procedure with its resource owner ID. The CAPIF core function executes the offboard process including deleting the resource owner client profile and revoking API invoker authorization related to the resource owner consent.

6.17.1.2		Procedure for offboarding resource owner from CAPIF

Figure 6.17.1.2-1: Procedure for offboarding resource owner

1. The resource owner client triggers offboard resource owner client request towards the CAPIF core function with its resource owner identity.

2. The CAPIF core function cancels the enrollment of the API invoker from CAPIF and delete information related to the resource owner. This includes deletion of the resource owner consent and resource reference. Depending on the operator policy, the CAPIF core function may retain the information of the offboarded resource owner client.

3. The CAPIF core function sends offboard resource owner client response message, indicating the successful offboarding of the resource owner client.

Note: If the resource owned by this resource owner has published to other CAPIF core functions, the CAPIF core function may send Event notification message to other CAPIF core functions to indicate that the resource owner client offboards.

6.17.2	Architecture Impacts

6.17.3	Corresponding APIs

Table 6.17.3-1: Offboard resource owner client request

| Information element | Status | Description |
| --- | --- | --- |
| CAPIF core function identity information | M | The information that uniquely identifies the CAPIF core function. |
| Resource owner identity information | M | The information that uniquely identifies the resource owner. |

Table 6.17.3-2: Offboard resource owner client respond

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of offboarding the resource owner |