3GPP TSG-SA WG6 Meeting #62	S6-243098

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Update to Solution #4

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution proposes to update solution #4 to address the open issue of KI #4. This contribution adds the enhancing API invoker onboarding procedure and the enhancing API invoker offboarding procedure to support CAPIF interconnection.

2. Reason for Change

In CAPIF interconnection, the CCF-A that provides the service APIs may subscribes events related to API invokers from the CCF-B that the API invoker onboard to. When the API invoker is onboard to and offboard from the CCF-B, the CCF-B notifies the CCF-A.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v 0.4.0

* * * First Change * * * *

6.5.1.3.5	Procedure for enhanced API invoker onboarding

Pre-conditions:

1.  The API invoker and the CCF-B are in the same trust domain.

2.  The CCF-A and the CCF-B are connected to each other to support CAPIF interconnection.

Figure 6.5.1.3.5-1: Procedure for enhanced API invoker onboarding

1.	For enrollment of the API invoker to be a recognized user of the CAPIF, the API invoker triggers onboard API invoker request towards the CAPIF core function, providing the information as required for the API management.

2.	The CAPIF core function begins the onboarding process by verifying whether all the necessary information has been provided to onboard the API invoker, and further initiates a grant process. Successful onboarding results in provisioning API invoker profile which includes identity for the API invoker. The authorization information and the list of APIs and the types of APIs that the API invoker can access subsequent to successful onboarding may also be created. The CAPIF core function may create access control policy (see Table E-1) for the onboarded API invoker considering the network slice information.

NOTE 1:	Completion of onboarding process can require explicit grant by the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the grant process internally without the need of explicit grant by the CAPIF administrator.

NOTE 2:	The API invoker profile consists of at least the identity information for the API invoker, information required for the authentication and authorization by the CAPIF and the CAPIF identity information.

NOTE 3:  In order to support CAPIF interconnection, the API invoker ID indicates which CCF the API invoker is onboard to. The API invoker ID can be used by CCF and AEF of other CAPIF provider domain to identify the API invoker and obtain information of the API invoker profile from the CCF that the API invoker is onboard to.

3.	If the API invoker has triggered the onboard API invoker request and is granted permission, the onboard API invoker response provides success indication including information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs.

4.	As a result of successful onboarding process, the CAPIF core function is able to authenticate and authorize the API invoker.

5.  If the CCF-A subscribes to the CCF-B, the CCF-B notifies the CCF-A with its identity information, the API invoker ID and the API invoker status (onboarded).

6. The CCF-A sends an event notification acknowledge to the CCF-B.

* * * Next Change * * * *

6.5.1.3.6	Procedure for enhanced API invoker offboarding

Pre-conditions:

1.  The API invoker has been onboarded as a recognized user of the CAPIF provider domain B.

2.  The CCF-A and the CCF-B are connected to each other to support CAPIF interconnection.

Figure 6.5.1.3.6-1: Procedure for enhanced API invoker offboarding

1.  The API invoker are offboard from the CCF-B as specified in 3GPP TS 23.222 clause 8.2.

2.  If the CCF-A subscribes to the CCF-B, the CCF-B notifies the CCF-A with its identity information, the API invoker ID and the API invoker status (offboarded).

3.  The CCF-A cancels the stored information related to the API invoker. If there are service API invocation authorizations related to its connected AEF, the CCF-A may initiate the revoking API invoker authorization procedure specified in TS 23.222 clause 8.24.

4. The CCF-A sends an event notification acknowledge to the CCF-B.