3GPP TSG-SA WG6 Meeting #62	S6-243099

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Update to Solution #9

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution propose to update solution #9 to address the 4th open issue of KI #1.

2. Reason for Change

This contribution propose to update solution #9 to address the 4th open issue of KI #1, i.e. How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0.

* * * First Change * * * *

6.10	Solution #9: Support more API invoker info in RNAA

6.10.1	Solution description

6.10.1.1	General

This solution provides more API invoker info related to RNAA so that the resource owner can know better about which user and/or which service (and its provider) is asking for sensitive information (e.g. location) of the resource owner.

6.10.1.3	Procedures

6.10.1.3.1	Authorization

Figure 6.10.1.3.1-1: Procedure for the API invoker obtaining authorization for service API access

The procedure is similar to clause 8.11.3 of 3GPP TS 23.222 [2] with the following difference:

-	In step 1, resource owner ID and more information about API invoker (application service info like ASP id, application id) and purpose of data processing (e.g. service operation level, resource level) can be sent from API invoker to CCF during service API authorization procedure.

NOTE:	Application service info are applicable for both AF side API invoker and UE side API invoker.

-	In step 2, if received, the CCF additionally verifies application service info, and perform resource owner consent check to determine whether the resource owners allow this authorization request.

-    In step 3, Based on the API invoker's subscription information and resource owner consent, the authorization information to access the service APIs is sent to the API invoker in the obtain service API authorization response.

6.10.2	Architecture Impacts

Existing CAPIF architecture in 3GPP TS 23.222 [2] is used to accommodate new impact to CAPIF-1/1e reference point.

The CAPIF architecture shall support CAPIF-8 reference point between CCF and resource owner.

6.10.3	Corresponding APIs

Existing CAPIF service API authorization API is enhanced.

6.10.4	Solution evaluation

This solution addresses KI#1 with more granular information about API invoker and resource owner consent in the context of RNAA. It also addresses the purpose of data processing in KI#1. The impacted CAPIF procedure includes service API authorization.

* * * Next Change * * * *