3GPP TSG-SA WG6 Meeting #62	S6-243100

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Update to Solution #10

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution propose to update solution #10 to address the 4th open issue of KI #1.

2. Reason for Change

This contribution propose to update solution #10 to address the 4th open issue of KI #1, i.e. How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0.

* * * First Change * * * *

6.11	Solution #10: Access Control Management of Service API

6.11.1	Solution description

6.11.1.1	General

This solution is for Key issue #1 to give a description of the service API invocation procedure based on authorization for granular service access.

The API invoker performs the authorization based on granular service access (e.g., per service API or per service operation or per service resource). The subsequent service API invocation over CAPIF-2/2e by API invoker on the AEF should comply with the received authorization.

Figure x: Procedure for API invoker authorization to access service APIs

1.	The API invoker triggers service API invocation request to the AEF, including the service API to be invoked.

NOTE 1:	Authentication can also be performed if not authenticated previously.

NOTE 2:	The API invoker can trigger several service API invocations asynchronously.

2.	Upon receiving the service API invocation request, the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information and resource owner consent parameters.

2a.	If the AEF does not have information required to authorize service API invocation, the AEF obtains the authorization information and resource owner consent parameters from the CAPIF core function.

3.	The AEF executes the service logic for the invoked service API.

4.	The API invoker receives the service API invocation response as a result of the service API invocation.

6.11.2	Architecture Impacts

This solution is based on the existing CAPIF architecture in 3GPP TS 23.222 [2].

This solution is based on the existing procedure in clause 8.16.3 of 3GPP TS 23.222 [2].

The CAPIF core function/authorization function stores the resource owner consent information received from resourece owner via CAPIF-8 reference point.

6.11.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

6.11.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *