3GPP TSG-SA WG6 Meeting #62	S6-243101

Maastricht, Netherlands, 19th – 23rd August 2024

Source:	ChinaTelecom

Title:	Update to Solution #11

Spec:	3GPP TR 23.700-22 v0.4.0

Agenda item:	8.7

Document for:	Approval

Contact:	zhouz11@chinatelecom.cn

1. Introduction

This contribution propose to update solution #10 to address the 4th open issue of KI #1.

2. Reason for Change

This contribution propose to update solution #10 to address the 4th open issue of KI #1, i.e. How to align and manage access control that is more granular than simply granted/denied for service API (e.g., service operation level, resource level, service API originator/requestor details) with the provided resource owner consent to ensure appropriate usage of resource owner consent at the enabler layer.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22 v0.4.0.

* * * First Change * * * *

6.10	Solution #11: Resource Owner Consent Revocation

6.12.1	Solution description

6.12.1.1	General

This solution is for Key issue #1 to manage the resource owner consent.

In current specification, the consent of resource assess will only be requested by API invoker before service API invocation procedure. However, the scenario where the resource owner's consent is revoked after the API invoker obtains it has not been considered (e.g., for a certain API invoker, the location permission may change from allowed to not allowed). To support the resource owner consent revocation a procedure of resource owner consent revocation is needed.

6.12.1.2		Procedure for resource owner consent revocation

Pre-condition:

1.	The resource owner can communicate with the API invoker.

2.	The service API access requires obtaining consent from resource owner.

3.	The resource owner function has provided the user consent for the API invoker.

Figure 6.12.1.2-1:	Procedure for resource owner consent revocation

1.	The resource owner function sends a resource owner consent revocation request to CAPIF core function/Authorization function via CAPIF-8 reference point in order to provide resource owner revocation information.

Note: The request includes resource owner ID, the resource reference and the new resource owner consent referred to the resource reference.

2.	The CAPIF core function/Authorization function processes the request and sends a revocation response to the resource owner function via CAPIF-8 reference point. Then triggers the procedure for revoking API invoker authorization as described in subclause 8.23.4 of 3GPP TS 23.222 [2], i.e. the CAPIF core function/Authorization function determine which service API and API invoker are required to be revoked based on resource owner revocation information and authorization information of API invoker to access the service APIs.

Note: The CAPIF core function/Authorization function updates resource owner client profile based on the request.

6.12.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

The CAPIF architecture shall support CAPIF-8 reference point.

6.12.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

6.12.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *