3GPP TSG-SA WG6 Meeting #61	S6-243116

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0126 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location reporting information obtained from the PLMN operator (5G) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Location management procedues such as clauses10.9.3.6.1 and 10.9.3.6.2 in 3GPP TS 23.280 includes the possibility of obtaining location information from the PLMN network, but it is not explained how this is possible. For this purpose, this CR introduces a new reference point in the functional model in clause 5.3.1. Furthermore, two new clauses are added to indicate how the MC system obtains location information from the 5G core. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - adding a reference to 3GPP TS 23.273 - updating the abbreviations in clause 3.3 - adding a new reference point in the functional model in clause 5.3.1 - adding a new clause related to the location management server, and how it obtains the location information from the 5GS. The information is aligned with TS 23.280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not described how would the LMS obtain location information of a certain MC service UE from the 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.3.1, the following new clauses 5.3.2.2.x, 5.3.3.2.x and 5.3.3.2.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[15]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.304: "Proximity based Service (ProSe) in the 5G System (5GS); Stage 2".

[18]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[19]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[20]	3GPP TS 23.479: "UE MBMS APIs for Mission Critical Services".

[21]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[22]	3GPP TS 22.289: "Mobile communication system for railways".

[23]	3GPP TS 23.283: "Mission Critical Communication Interworking with Land Mobile Radio Systems".

[23.273]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

# * * * * Second change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GMLC	Gateway Mobile Location Centre

NPN	Non-Public Network

PNI-NPN	Public Network Integrated Non-Public Network

SNPN	Stand-alone Non-Public Network

NID	Network Identifier

# * * * * Third change * * * *

### 5.3.1	On-network functional model

Each MC service can be represented by an application plane functional model. The corresponding functional model across MC services may be similar but is described by the individual functional entities and reference points that belong to that MC service. Within the application plane for an MC service, a common set of functions as well as reference points is shared across MC services and is referred as the common services core.

Figure 5.3.1-1 shows the common functional model for the application plane for an MC system using 5GS.

Figure 5.3.1-1: Common functional model for application plane for an MC system

The common services core functions and reference points shown in figure 5.3.1-1 are shared across each MC service.

Figure 5.3.1-2 shows the common functional model for the signalling control plane using 5GS.

Figure 5.3.1-2: Common functional model for signalling control plane

In the model shown in figure 5.3.1-2, the SIP core may interact directly with 5GS via the N5 reference point or Rx reference point to control QoS on a per communication flow in accordance with 3GPP TS 23.501 [7].

NOTE 1:	Indirect interaction between SIP core and 5GS Network Exposure Function using N33 reference point is not supported by 3GPP TS 23.002 [8].

Figure 5.3.1-3 shows the relationships between the reference points of the common application plane of an MC service server and the common signalling plane.

Figure 5.3.1-3: Relationships between reference points of the common MC service application plane and signalling control planes

NOTE 2:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 3:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 5.3.1-3.

NOTE 4:	CSC-5, CSC-9, and CSC-15 make use of SIP-1 and SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 5.3.1-3.

# * * * * Fourth change * * * *

##### 5.3.2.2.x	Location management server (LMS)

The location management server is a functional entity that receives and stores and provides user location information to the MC service server. The location management server may also acquire location information from the 5G core network, i.e., from the LCS network (in specific from the GMLC). For the purpose of obtaining location information via the LCS network, the location management server may or may not collocate a LCS client as described in 3GPP TS 23.273 [23.273].

# * * * * Fifth change * * * *

##### 5.3.3.2.x	Reference point CSC-25 (between the Location management server and the LCS network in the PLMN operator)

The CSC-25 reference point, which exists between the location management server (or the collocated LCS client) and the LCS server in the 5GS, allows location information reporting acquired from the PLMN operator according to the procedures defined in 3GPP TS 23.273 [23.273]. The reference point CSC-25 is based on the reference point Le as defined in 3GPP TS 23.273 [23.273].

The identities of the target MC service UE, which its location is requested from the LCS network, may be a GPSI or a SUPI.

# * * * * End of changes * * * *