3GPP TSG-SA WG6 Meeting #62	S6-243118

Maastricht, Netherlands 19th – 23rd August 2024	(revision of S6-242346)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0563 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Generic ad hoc group communication towards migrated MC service user |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR describes generic procedure for ad hoc group communication towards migrated MC service user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a generic procedure for ad hoc group communication towards an MC service user which has migrated to a partner MC system. The ad hoc group communication is created based on a list of participants provided by an authorised MC service user. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clarities about ad hoc group communication towards migrated MC service users. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | The following new clauses: 10.16.x, 10.16.x.1, 10.16.x.2, and 10.16.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 10.16.x	Generic ad hoc group communication towards migrated MC service user

#### 10.16.x.1	General

This subclause describes a generic procedure for ad hoc group communication towards migrated MC service user. The ad hoc group communication is created based on participant list provided by an authorised MC service user.

This procedure needs to be read in conjunction with related procedures in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282[13].

Clause 10.16.x.2 presents the information elements in the ad hoc group communication redirection message sent from MC service server of one MC system to MC service server in another.

Clause 10.16.x.3 describes a generic procedure for ad hoc group communication towards migrated MC service user, where the participant list is provided by the initiated MC service user.

#### 10.16.x.2	Ad hoc group communication redirection

Table 10.16.x.2-1 describes the information flow of an ad hoc group communication redirection, which is sent from the MC service server in one MC system to an MC service server in another MC system.

Table 10.16.x.2-1: Ad hoc group communication redirection

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user to be included in an ad hoc group communication. This MC service ID is provided by its primary MC system before migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| MC service ID | M | The MC service ID of the MC service user to be included in an ad hoc group communication This MC service ID is provided by a partner MC system after migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| KMS URI (NOTE) | O | The KMS URI associated with the MC service ID of the migrated MC service user. |
| Redirection reason | O | The MC service server informs the MC service server in a partner MC system that the target MC service user has migrated. |
| NOTE: If the KMS URI is not configured in the MC service user profile and is not included in the ad hoc group communication redirection message, then the MC service server at the partner MC system shall obtain KMS URI using the KMS lookup procedure defined in 3GPP TS 33.180 [19]. |  |  |

#### 10.16.x.3	Procedure for participant list is determined by the initiated MC service user

Figure 10.16.x.3-1 represents a generic ad hoc group communication towards an MC service user which has migrated to a partner MC system where the participant list is determined by an initiated MC service user.

Pre-conditions:

-	MC service user 1 receives MC services from MC system A.

-	MC system C is the primary MC system of MC service user 1.

-	MC service user 1 was participating in an ad hoc group communication prior to performing migration to MC system B.

Figure 10.16.x.3-1: Generic ad hoc group communication towards migrated MC service user where the participant list is determined by MC service user

1.	MC service server A determines that MC service user 1 is no longer within MC system A, i.e., has logged out from MC system A upon performing the procedure in clause 10.6.3.3.2.1.

2.	Since MC service server C is the primary MC system of MC service user 1, MC service server A sends an ad hoc group communication request to MC service server C. The ad hoc group communication request includes the MC service ID of the MC service user 1 provided by the primary MC system C. The ad hoc group communication request message is defined in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282[13].

3.	The MC service server C determines that the MC service user 1 has migrated to MC system B.

4.	MC service server C sends an ad hoc group communication redirection to MC service server A. The message indicates the MC service ID associated to MC system B.

5.	MC service server A sends MC service user 1 an ad hoc group communication request according to 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282[13]. The request includes the MC service ID of MC service user 1 provided by MC system B, which was provided to MC service server A in the previous step.

6.	The MC service user 1 is notified to join the ad hoc group communication as defined in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282[13].

7	. MC service user 1 sends MC service server A an ad hoc group communication request according to the defined messages in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282[13].

8.	 MC service server A sends ad hoc group communication notify message to participants according to 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282[13].

9.		The ad hoc group communication is changed including the MC service user 1.

# * * * * End of changes * * * *