3GPP TSG-SA WG6 Meeting #62	S6-243119

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	ETRI, Convida Wireless LLC

Title:	Solution #8 Delete spatial map procedure segregated from Update procedure

Spec:	3GPP TR 23.700-21 v.1.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	Byung Jun AHN (bjahn@etri.re.kr)

1. Introduction

This pCR proposes to add Delete spatial map procedure segregated from Update procedure.

2. Reason for Change

Solution #8 of 3GPP TR 23.700-21 v1.1.0 does not have a separate Delete spatial map procedure which is one of the CRUD operations for spatial map management. Instead, the solution provides the delete spatial map operation as a part of Update procedure. It is not commonly used best practice.

3. Conclusions

It is required to add Delete spatial map procedure segregated from Update procedure.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.1.0.

* * * First Change * * * *

7.8.3.2	Update spatial map

Figure 7.8.3.2-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL LM client), SEAL LM server updates the spatial map as requested.

Figure 7.8.3.2-1: Update spatial map

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update a spatial map. The request includes requestor ID, spatial map ID, information on what to update such as updated access control rules and updated spatial map coverage area, etc.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client) and validates the request. The SEAL LM server updates the spatial map as requested.

3)	Then it sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID.

* * * Next Change * * * *

7.8.3.x	Delete spatial map

Figure 7.8.3.x-1 depicts the procedure to delete the spatial map(s). For the request from the spatial map consumer (VAL server or SEAL LM client), SEAL LM server deletes the spatial map(s) as requested.

Figure 7.8.3.x-1: Delete spatial map procedure

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to delete the spatial map(s). The request includes requestor ID, a list of spatial map ID(s) to delete and optionally the reason to delete.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client) and validates the request. The SEAL LM server deletes the spatial map(s) as requested.

3)	Then it sends response message to the requestor with result of the request.

* * * Next Change * * * *

7.8.4.2	Update spatial map

Table 7.8.4.2-1 describes the information flow from VAL server (or SEAL LM client) to SEAL LM server for the update spatial map request.

Table 7.8.4.2-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | ID of the spatial map to update |
| Requested update information | O | Information on what to update, e.g. access control rules or spatial map coverage area, etc. |

NOTE :	The Requested update information will be further defined during the normative work phase.

Table 7.8.4.2-2 describes the information flow from the SEAL LM server to VAL server (or SEAL LM client) for the update spatial map response.

Table 7.8.4.2-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the update spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | M | ID of the spatial maprequested to update. |
| Spatial map information | O | If Result is success, updated spatial map information is included. |

* * * Next Change * * * *

7.8.4.x	Delete spatial map

Table 7.8.4.x-1 descibes the information flow from the VAL server (or SEAL LM client) to the SEAL LM server for the delete spatial map request.

Table 7.8.4.x-1: Delete spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL LM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| List of Spatial map ID(s) | M | Spatial map ID(s) to delete. Each element includes the information described below. |
| > Spatial map ID | M | ID of the spatial map to delete |
| > Reason to delete | O | Reason to delete |

Table 7.8.4.x-2 descibes the information flow from the SEAL LM server to the VAL server (or SEAL LM client) for the delete spatial map response.

Table 7.8.4.x-2: Delete spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result list | M | A list of result(s) for requested spatial map ID(s) to delete. Each element includes the information described below. |
| > Spatial map ID | M | ID of the spatial map requested to delete |
| > Result | M | The result of the delete operation per spatial map (e.g., success or fail), including cause of failure if needed. |

* * * End of Change * * * *