3GPP TSG-SA WG6 Meeting #62	S6-243120

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	ETRI, Convida Wireless LLC

Title:	New text for clause 9 Spatial Map (SM) management

Spec:	3GPP TS 23.437 v.0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Byung Jun AHN (bjahn@etri.re.kr)

1. Introduction

This pCR proposes new text for 3GPP TS 23.437 clause 9 Spatial Map (SM) management.

2. Reason for Change

Skeleton of the 3GPP TS 23.437 (S6a240151) has been approved in the previous meeting (3GPP SA6#62 Ad-Hoc). Body text for the approved skeleton is yet to be developed.

3. Conclusions

New text for the skeleton, including clause 9, are required to be provided.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.1.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.434: " Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

* * * Next Change * * * *

4.3	Spatial map management

[AR-4.3-a]	The spatial map management service shall provide mechanisms to produce, get, update, delete and subscribe spatial maps.

[AR-5.4-b]	The spatial map management service shall provide mechanisms to expose spatial map information to the authorized third parties.

[AR-5.4-c]	The spatial map management service shall allow multiple spatial maps in a given three-dimensional space for different vertical applications and consumer’s usage.

[AR-5.4-d]	The spatial map management service shall be able to interact with other SEAL servers and 5GS to support vertical applications.

[AR-5.4-e]	The spatial map management service shall support spatial localization service.

[AR-5.4-f]	The spatial map management service shall enable creation of augmented spatial maps with information to support vertical applications.

[AR-5.4-e]	The spatial map management service shall enable VAL server(s) to provide spatial mapping service and to get proper spatial mapping information provider(s).

* * * Next Change * * * *

9	Spatial Map (SM) management

9.1	General

The Spatial Map (SM) management is a SEAL service that offers the spatial map related capabilities to vertical application(s).

9.2	Functional model for spatial map management

9.2.1	General

The functional model for the spatial map management is based on the generic functional model specified in clause 6 of 3GPP TS 23.434 [x]. It is organized into functional entities to describe a functional architecture which addresses the support for spatial map management aspects for vertical applications. The on-network functional model is specified in this clause.

9.2.2	On-network functional model description

Figure 9.2.2-1 illustrates the generic on-network functional model for spatial map management.

Figure 9.2.2-1: On-network functional model for spatial map management

The spatial map management client communicates with the spatial map management server over the SM-UU reference point. The spatial map management client provides the support for spatial map management functions to the VAL client(s) over SM-C reference point. The VAL server(s) communicate with the spatial map management server over the SM-S reference point. The VAL client communicates with the VAL server over the VAL-UU reference point, which is out of this document’s scope.

Editor’s Note: This functional model will be adapted to the conclusion of the architecture option.

The spatial map management server interacts with other SEAL server, e.g., spatial anchors management server and location management server etc., for inter-service communication over SEAL-X reference point.

NOTE:	The specific SEAL server interactions corresponding to SEAL-X will be provided.

Figure 9.2.2-2 illustrates the service-based interfaces for providing and consuming spatial map management service. The spatial map management server could provide service to VAL server and spatial map management client through interface Ssm.

Figure 9.2.2-2: Architecture for spatial map management – Service based representation

9.3	Spatial map management procedures

9.3.1	Produce spatial map

Figure 9.3.1-1 depicts the procedure for producing a spatial map. For the request from the spatial map requestor (VAL server or SEAL SM client), the service is provided by SEAL SM server.

Editor’s Note: Functional entities of this procedure will be adapted to the conclusion of the architecture option.

Figure 9.3.1-1: Produce spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to produce the spatial map. The request includes requestor ID, three-dimensional area of interest, information to be included in the spatial map such as access control rules defining which entities are permitted to discover and access the spatial map.

NOTE 1:	The information to be included in the spatial map will be further defined.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. The SEAL SM server produces a requested spatial map using the processed sensor data which is required to estimate the number of objects, type of the object, position, direction and speed of the object, etc.

NOTE 2:	For current release, the processed sensor data is stored at VAL layer (e.g., in database at VAL layer) and the SEAL SM server gets access to such database to produce spatial map via application specific way which is out of SA6 scope.

3)	The SEAL SM server sends response message to the requestor with assigned spatial map ID and information which includes three-dimensional space defined by the spatial map, stationary or moving objects with attributes of them.

NOTE 3:	The spatial map information in the response message will be further defined.

Editor's Note:	The necessity of supporting third party triggered spatial map producing, in addition to spatial map service provider self-determined of map producing, needs further discussion.

9.3.1.1	Information flows for Produce spatial map

Table 9.3.1.1-1 descibes the information flow from the VAL server (or SEAL SM client) to the SEAL SM server for the produce spatial map request.

Table 9.3.1.1-1: Produce spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Requested information | O | Information to be included in the spatial map (e.g., access control rules) |

NOTE 1:	The requested information will be further defined.

Table 9.3.1.1-2 descibes the information flow from the SEAL SM server to the VAL server (or SEAL SM client) for the produce spatial map response.

Table 9.3.1.1-2: Produce spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | O | ID of the produced spatial map |
| Spatial map information | O | If Spatial map ID is valid, a collection of information is included. The information corresponds to the produced spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them |

NOTE 2:	Details about Spatial map information is FFS and will be considered during the normative work phase.

9.3.2	Get spatial map

Figure 9.3.2-1 depicts the procedure for an authorized spatial map consumer (VAL server or SEAL SM client) to get the spatial map. The service is provided by SEAL SM server.

Editor’s Note: Functional entities of this procedure will be adapted to the conclusion of the architecture option.

Figure 9.3.2-1: Get spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to get the spatial map. The request includes requestor ID, spatial map ID to get the information on it, three-dimensional area of interest or localization information to discover spatial maps in the area, optionally discovery filters, i.e., a set of characteristics to search the matching spatial maps when multiple spatial maps are allowed in the area of interest.

NOTE:	How the consumer identifies which three-dimensional area of interest to get the spatial map is not in the scope of this study.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then the server collects the requested spatial map information. The SEAL SM server sends the get spatial map response message to the authorized third party consumer with spatial map ID and spatial map information when matching spatial map is found, otherwise it sends failure indication with reason.

9.3.2.1	Information flows for Get spatial map

Table 9.3.2.1-1 descibes the information flow from the VAL server (or SEAL SM client) to the SEAL SM server for the get spatial map request.

Table 9.3.2.1-1: Get spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Filtering criteria | M | Criteria to get spatial map(s) |
| > Spatial map ID(s) | O (NOTE) | ID(s) of the spatial map to get information of it |
| > Area of interest | O (NOTE) | Three-dimensional area information to discover spatial maps |
| > Localization information | O (NOTE) | A known location to discover spatial maps in the three-dimensional space, including an orientation |
| > Discovery filters | O | A set of characteristics to search the matching spatial maps |
| NOTE: At least one of the information elements shall be provided. |  |  |

NOTE:	The Discovery filters will be further defined.

Table 9.3.2.1-2 descibes the information flow from the SEAL SM server to the VAL server (or SEAL SM client) for the get spatial map response.

Table 9.3.2.1-2: Get spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the producing spatial map (e.g., success or fail), including cause of failure if needed |
| List of discovered spatial maps | O | List of discovered spatial anchor(s). Each element includes the information described below |
| > Spatial map ID | M | ID of a spatial map discovered |
| > Spatial map information | M | A collection of information that corresponds to the discovered spatial map including dimensional information, stationary or moving objects with attributes, e.g., object type, appearance, position, direction and speed of them |

9.3.3	Update spatial map

Figure 9.3.3-1 depicts the procedure to update the spatial map. For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server updates the spatial map as requested.

Editor’s Note: Functional entities of this procedure will be adapted to the conclusion of the architecture option.

Figure 9.3.3-1: Update spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to update a spatial map. The request includes requestor ID, spatial map ID, information on what to update such as updated access control rules and updated spatial map coverage area, etc.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SM server updates the spatial map as requested.

3)	Then it sends response message to the requestor with result of the request and updated spatial map information with the spatial map ID.

9.3.3.1	Information flows for Update spatial map

Table 9.3.3.1-1 descibes the information flow from the VAL server (or SEAL SM client) to the SEAL SM server for the update spatial map request.

Table 9.3.3.1-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | ID of the spatial map to update |
| Requested update information | O | Information on what to update, e.g., access control rules or spatial map coverage area, etc. |

NOTE:	The Requested update information will be further defined.

Table 9.3.3.1-2 descibes the information flow from the SEAL SM server to the VAL server (or SEAL SM client) for the update spatial map response.

Table 9.3.3.1-2: Update spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the update spatial map (e.g., success or fail), including cause of failure if needed |
| Spatial map ID | M | ID of the spatial map requested to update. |
| Spatial map information | O | If Result is success, updated spatial map information is included. |

9.3.4	Delete spatial map

Figure 9.3.4-1 depicts the procedure to delete the spatial map(s). For the request from the spatial map consumer (VAL server or SEAL SM client), SEAL SM server deletes the spatial map(s) as requested.

Editor’s Note: Functional entities of this procedure will be adapted to the conclusion of the architecture option.

Figure 9.3.4-1: Delete spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to delete the spatial map(s). The request includes requestor ID, a list of spatial map ID(s) to delete and optionally the reason to delete.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client) and validates the request. The SEAL SM server deletes the spatial map(s) as requested.

3)	Then it sends response message to the requestor with result of the request.

9.3.4.1	Information flows for Delete spatial map

Table 9.3.4.1-1 descibes the information flow from the VAL server (or SEAL SM client) to the SEAL SM server for the delete spatial map request.

Table 9.3.4.1-1: Delete spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| List of Spatial map ID(s) | M | Spatial map ID(s) to delete. Each element includes the information described below. |
| > Spatial map ID | M | ID of the spatial map to delete |
| > Reason to delete | O | Reason to delete |

Table 9.3.4.1-2 descibes the information flow from the SEAL SM server to the VAL server (or SEAL SM client) for the delete spatial map response.

Table 9.3.4.1-2: Delete spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| Result list | M | A list of result(s) for requested spatial map ID(s) to delete. Each element includes the information described below. |
| > Spatial map ID | M | ID of the spatial map requested to delete |
| > Result | M | The result of the delete operation per spatial map (e.g., success or fail), including cause of failure if needed. |

9.3.5	Subscribe/unsubscribe spatial map

Figure 9.3.5-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL LM client) to subscribe/unsubscribe the spatial map(s). The service is provided by SEAL SM server.

Editor’s Note: Functional entities of this procedure will be adapted to the conclusion of the architecture option.

Figure 9.3.5-1: Subscribe/unsubscribe spatial map procedure

1)	The VAL server (or SEAL SM client) sends a request message to the SEAL SM server to subscribe/unsubscribe spatial map(s). The request includes requestor ID, spatial map ID(s) or three-dimensional area of interest (or localization information) to identify the applicable spatial map(s), a list of events to subscribe/unsubscribe which trigger the notification, with triggering criteria, minimum time between consecutive notifications, etc. For the unsubscribe spatial map request, the subscription ID is included in the message.

2)	The SEAL SM server authorizes VAL server (or SEAL SM client), and validates the request. Then server add/delete the requestor into/from the subscriber list and event list of the spatial map, and sends response message with the result. For the subscribe spatial map request, the subscription ID is included in the message as well.

NOTE 1:	For the VAL server (or SEAL SM client) which unsubscribed all the events of the spatial map, the SEAL SM server removes the VAL server (or SEAL SM client) completely from the subscriber list of the spatial map. In this case, step 3) will not be initiated.

3)	When an event is detected, the SEAL SM server notifies the VAL servers (or SEAL SM clients) which subscribed the corresponding event. Spatial map notification message includes the information on the detected triggering event, e.g. added or removed objects, position or direction changes of the objects in the spatial map etc.

NOTE 2:	Further triggering events and triggering criteria etc. will be defined.

9.3.5.1	Information flows for Subscribe/unsubscribe spatial map

Table 9.3.5.1-1 descibes the information flow from the VAL server (or SEAL SM client) to the SEAL SM server for the subscribe spatial map request.

Table 9.3.5.1-1: Subscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to subscribe |
| > List of spatial map IDs | O (NOTE) | ID(s) of the spatial map(s) to subscribe |
| > Area of interest | O (NOTE) | Three-dimensional area information to discover spatial map(s) to subscribe |
| > Localization information | O (NOTE) | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3-dimensional space. |
| List of events for discovered target spatial map(s) | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O (NOTE) | Type of event to subscribe |
| > Event ID | O (NOTE) | ID of the event to subscribe |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred, e.g., added or removed objects, position or direction changes of the objects in the spatial map etc. |
| > Notification intervals | O | Specifies when to notify the detected event, e.g., promptly or periodically with minimum time between consecutive notifications, etc. |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

NOTE 1:	Details of how to define events, including by whom and what, will be specified during the normative work phase.

Table 9.3.5.1-2 descibes the information flow from the SEAL SM server to the VAL server (or SEAL SM client) for the subscribe spatial map response.

Table 9.3.5.1-2: Subscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the subscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list subscribed | M | List of the events subscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event subscribed |
| >>> Event ID | M | ID of the event subscribed |
| >>> Result | M | Result of the subscribe request for the event (e.g., success or fail), including cause of failure if needed. |
| >>> Subscription ID | O | Allocated ID for the successful subscription to the event |

Table 9.3.5.1-3 descibes the information flow from the VAL server (or SEAL SM client) to the SEAL SM server for the unsubscribe spatial map request.

Table 9.3.5.1-3: Unsubscribe spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL SM client) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Target spatial map(s) | M | Spatial map(s) to unsubscribe |
| > List of spatial map IDs | O (NOTE) | ID(s) of the spatial map(s) to unsubscribe |
| > Area of interest | O (NOTE) | Three-dimensional area information to discover spatial map(s) to unsubscribe |
| > Localization information | O (NOTE) | Localization information of the UE/User to discover spatial map(s) which include the localized location in 3-dimensional space. |
| List of events to unsubscribe for discovered target spatial map(s) | M | Events to unsubscribe. Each element includes the information described below. |
| > Event type | O | Type of event to unsubscribe |
| > Event ID | O | ID of the event to unsubscribe |
| > Subscription ID | M | Subscription ID to unsubscribe, which was allocated by the SEAL server when the requestor subscribed the event |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

Table 9.3.5.1-4 descibes the information flow from the SEAL SM server to the VAL server (or SEAL SM client) for the unsubscribe spatial map response.

Table 9.3.5.1-4: Unsubscribe spatial map response

| Information element | Status | Description |
| --- | --- | --- |
| List of results | M | List of the results for the unsubscribed events requested. Each element includes the information described below. |
| > Spatial map ID | M | ID of a spatial map to which requested events are belong |
| >> Event list unsubscribed | M | List of the events unsubscribed in a spatial map. Each element includes the information described below. |
| >>> Event type | O | Type of event unsubscribed |
| >>> Event ID | M | ID of the event unsubscribed |
| >>> Result | M | Result of the unsubscribe request for the event (e.g., success or fail), including cause of failure if needed. |

Table 9.3.5.1-5 describes the information flow form SEAL SM server to VAL server (or SEAL SM client) for the notification when a subscribed triggering event is detected.

Table 9.3.5.1-5: Notification

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | ID of the spatial map that detected the triggering event |
| Event ID | M | ID of the detected triggering event |
| Subscription ID | M | Subscription ID for the event |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |

NOTE 2:	The Notification information will be further defined during the normative work phase.

9.x	SEAL APIs for spatial map management

9.x.1	General

Table 9.x.1-1 illustrates the SEAL APIs for spatial map management.

Table 9.x.1-1: List of SEAL APIs for spatial map management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SmManagement | Produce_SpatialMap | VAL server, SEAL SM client | Request /Response |
|  | Get_SpatialMap_Info | VAL server, SEAL SM client | Request /Response |
|  | Update_SpatialMap_Info | VAL server, SEAL SM client | Request /Response |
|  | Delete_SpatialMap | VAL server, SEAL SM client | Request /Response |
| SS_SmEventInfo | Subscribe_ SpatialMap_Event_Info | VAL server, SEAL SM client | Subscribe/Notify |
|  | Unsubscribe_ SpatialMap_Event_Info | VAL server, SEAL SM client |  |
|  | Notify_SpatialMap_Event_Info | VAL server, SEAL SM client |  |

9.x.2	SS_SmManagement API

9.x.2.1	General

API description: This API enables the VAL server or SEAL SM client to communicate with the spatial map management server to produce, get, update and delete spatial map over SM-S or SM-UU.

9.x.2.2	Produce_SpatialMap operation

API operation name: Produce_SpatialMap

Description: Produce a spatial map in the given three-dimensional area of interest.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer subclause 9.3.1.1

Outputs: Refer subclause 9.3.1.1

See subclause 9.3.1 for the details of usage of this API operation.

9.x.2.3	Get_SpatialMap_Info operation

API operation name: Get_SpatialMap_Info

Description: Provide requested spatial map information to the authorized consumers.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer subclause 9.3.2.1

Outputs: Refer subclause 9.3.2.1

See subclause 9.3.2 for the details of usage of this API operation.

9.x.2.4	Update_SpatialMap_Info operation

API operation name: Update_SpatialMap_Info

Description: Update the information on a spatial map as requested by the authorized consumers.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer subclause 9.3.3.1

Outputs: Refer subclause 9.3.3.1

See subclause 9.3.3 for the details of usage of this API operation.

9.x.2.5	Delete_SpatialMap operation

API operation name: Delete_SpatialMap

Description: Delete spatial map(s) as requested by the authorized consumers.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer subclause 9.3.4.1

Outputs: Refer subclause 9.3.4.1

See subclause 9.3.4 for the details of usage of this API operation.

9.x.3	SS_SmEventInfo API

9.x.3.1	General

API description: This API enables the VAL server or SEAL SM client to communicate with the spatial map management server to subscribe/unsubscribe the spatial map events and get notifications over SM-S or SM-UU.

9.x.3.2	Subscribe_SpatialMap_Event_Info operation

API operation name: Subscribe_SpatialMap_Event_Info

Description: Process the authorized consumer’s subscription to the spatial map events.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer subclause 9.3.5.1

Outputs: Refer subclause 9.3.5.1

See subclause 9.3.5 for the details of usage of this API operation.

9.x.3.3	Unsubscribe_SpatialMap_Event_Info operation

API operation name: Unsubscribe_SpatialMap_Event_Info

Description: Revoke the authorized consumer’s subscription to the spatial map events.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer subclause 9.3.5.1

Outputs: Refer subclause 9.3.5.1

See subclause 9.3.5 for the details of usage of this API operation.

9.x.3.4	Notify_SpatialMap_Event_Info operation

API operation name: Notify_SpatialMap_Event_Info

Description: Notify the information on the detected triggering events to which the authorized consumers are subscribed.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer subclause 9.3.5.1

Outputs: Refer subclause 9.3.5.1

See subclause 9.3.5 for the details of usage of this API operation.

* * * End of Change * * * *