3GPP TSG-SA WG6 Meeting #62	S6-243123

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0570 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location reporting support via MCGWUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR introduces enhancements and improvements in the existing specification related to the support of location reporting for MC clients in non-3GPP devices behind MC gateway UE. The MC client in non-3GP device sends the identity of the MC gateway UE, which is connected to, and which is responsible of reporting 3GPP access network location parameters. For this purpose, the MC gateway UE hosts a location management client that reports 3GPP access network related parameters on behalf of the MC clients behind it. The introduced solution is aligned with the security aspects defined by SA3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The MC client hosted in non-3GPP device includes the identity of the MC gateway UE in the location information report. - Clause 11.5.2.1 describes the support of location reporting for MC clients behind MC gateway UE. - Clauses 11.5.2.2 and 11.5.2.3 and their respective subclauses are deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No location reporting support of 3GPP access network related information to the MC clients behind an MC gateway UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.9.2.2, 11.5.2.1, 11.5.2.2 and 11.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 10.9.2.2	Location information report

Table 10.9.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting.

Table 10.9.2.2-1: Location information report (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID (see NOTE 4) | O | Identity of the requesting MC service user. |
| MC service ID (see NOTE 6) | O | The identity of the MC gateway UE responsible of reporting 3GPP access network location parameters |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| Functional alias | O | Functional alias that corresponds to the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service UE label (see NOTE 5) | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: In case of an on-demand request of an MC service user the MC service ID shall be provided. In case of an MC service server request or an event-triggered report, no MC service ID is provided. NOTE 5: This information element shall be present in case the “MC service UE label” parameter in the initial MC service UE configuration data has a non-null value. NOTE 6: This information element is optional and shall be present in case the location report is sent from a LMC hosted in non-3GPP device. |  |  |

Table 10.9.2.2-2 describes the information flow from the location management server to the location management client for location information reporting.

Table 10.9.2.2-2: Location information report (LMS – LMC)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID | M | Identity of the requesting MC service user. |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| Functional alias | O | Functional alias that corresponds to the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service UE label (see NOTE 4) | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: This information element shall be present in case the “MC service UE label” parameter in the initial MC service UE configuration data has a non-null value. |  |  |

Table 10.9.2.2-3 describes the information flow from the location management server to the MC service server for location information reporting.

Table 10.9.2.2-3: Location information report (LMS – MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the MC service ID. |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. |  |  |

Table 10.9.2.2-4 describe the information flow to support the handling of location information across MC systems.

Table 10.9.2.2-4: Location information report (LMS – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the reporting MC service user at partner MC system |
| Functional alias | O (see NOTE 1) | Functional alias of the reporting MC service user at partner MC system |
| MC service ID | M (see NOTE 2) | Identity of the requesting MC service user at primary MC system |
| Functional alias | O (see NOTE 2) | Functional alias of the requesting MC service user at primary MC system |
| MC service UE label (see NOTE 5) | O | Generic name of the reporting MC service UE at partner MC system |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Location Information (see NOTE 4) | M | Location information of the individual MC service user at partner MC system |
| NOTE 1: It shall be present if used with the Functional alias associated request. NOTE 2: Only present, if provided with the associated request. NOTE 3: An on-demand request may be the triggering event. NOTE 4: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 5: This information element shall be present in case the “MC service UE label” parameter in the initial MC service UE configuration data has a non-null value. |  |  |

# * * * * Third change * * * *

### 11.5.2	3GPP access network related location information management

#### 11.5.2.1	General

The MC gateway UE is responsible of reporting the 3GPP access network related location parameters on behalf of the MC client(s) hosted in non-3GPP device(s). For this matter, it hosts a location management client which sends location report to the location management sever.

The location management client hosted in non-3GPP device receives location reporting configuration from the location management server according to the existing procedures in clause 10.9 and sends the location report (excluding any 3GPP related location parameters) accordingly. Furthermore, the location report sent from the location management client in the non-3GPP device shall include the MC service ID of the MC gateway UE it is connected to, hence is responsible of reporting 3GPP access network related location information on behalf of the non-3GPP device. As a result, the MC service server stores the mapping between the MC client in the non-3GPP device and the respective MC gateway UE it is connected to. The location management client in the MC gateway UE sends location report, which includes 3GPP access network related parameters according to the existing procedures in clause 10.9.

NOTE:	To support location reporting via MC gateway UE, the MC gateway UE has an MC service ID and provided as described in this document.

# * * * * End of changes * * * *