3GPP TSG-SA WG6 Meeting #62	S6-243128

Maastricht, Netherlands, 19th – 23th August 2024	(revision of S6-242xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0573 | rev | - | Current version: | 19.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating MC gateway UE related location and MBMS procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Recent agreements with SA3 on removing the connection authorisation procedures requires to remove to remove the term “GW MC service ID” also from location management and MBMS procedures. This CR relies on using the existing MC service ID and removes the GW MC service ID from location management and MBMS message flows and related procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The term “GW MC service ID” is replaced by “MC service ID”. Annex A.7 on providing initial MC gateway UE configuration data is voided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.0, 11.5.2.2.1, 11.5.2.2.2, 11.5.2.2.3, 11.5.3.2.3, 11.5.3.2.7, 11.5.3.3.2, 11.5.3.3.2A, Annex A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 11.5.0	General

This clause describes the procedures related to providing MC services, e.g., location reporting and MBMS support, to MC clients hosted in non-3GPP device.

# * * * * 2nd change * * * *

##### 11.5.2.2.1	MC GW location reporting configuration

Table 11.5.2.2.1-1 describes the information flow from the MC client, which resides on a non-3GPP device, to the MC gateway UE for the location reporting configuration.

Table 11.5.2.2.1-1: MC GW location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report (see NOTE 2) |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent |
| NOTE 1: If none of the information elements is present, this represents a cancellation for location reporting, if configured. NOTE 2: The triggering criteria contains only the events related to the 3GPP access network. |  |  |

##### 11.5.2.2.2	MC GW location information report

Table 11.5.2.2.2-1 describes the information flow from the MC gateway UE to the MC client residing on a non-3GPP device for the location information reporting.

Table 11.5.2.2.2-1: MC GW location information report

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Triggering event | O | Identity of the event that triggered the sending of the report |
| Location information (see NOTE) | O | Location information of the MC gateway UE |
| NOTE: The following location information elements which are related to 3GPP access network shall be present (configurable): Serving and neighbouring ECGI, MBMS SAIs, MBMSfnArea, PLMN ID. |  |  |

##### 11.5.2.2.3	MC GW location information request

Table 11.5.2.2.3-1 describes the information flow from the MC client residing on a non-3GPP device to the MC gateway UE for requesting an immediate location information report.

Table 11.5.2.2.3-1: MC GW location information request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |

# * * * * 3rd change * * * *

##### 11.5.3.2.3	MC GW MapGroupToBearer request

Table 11.5.3.2.3-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of MapGroupToBearer message received from the MC service server.

Table 11.5.3.2.3-1: MC GW MapGroupToBearer request

| Information element | Status | Description |
| --- | --- | --- |
| GW service ID | M | The MC service ID of the MC service user. |
| MCPTT group ID | M | This element identifies the MCPTT group, in which the call is started. |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group call (e.g. MBMS subchannel). |
| TMGI | M | The MBMS bearer identifier. |

# * * * * 4th change * * * *

##### 11.5.3.2.7	MC GW UnMapGroupToBearer request

Table 11.5.3.2.7-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of UnMapGroupToBearer message received from the MC service server.

Table 11.5.3.2.7-1: MC GW UnMapGroupToBearer request

| Information element | Status | Description |
| --- | --- | --- |
| GW service ID | M | The MC service ID of the MC service user. |
| MCPTT group ID | M | This element identifies the MC service group related to a group call to be dissociated over the MBS session. |
| Media stream identifier | M | This element identifies the media stream of the SDP, which is no longer used for the group call within the MBMS session. |
| TMGI | M | The MBMS bearer identifier. |

# * * * * 5th change * * * *

##### 11.5.3.3.2	Procedure for handling MapGroupToBearer message

Whenever the MC client detects that traffic received from MC service server is MapGroupToBearer message and if the MC client participates in the group session or communication identified by the MapGroupToBearer message then it should inform the details contained in the MapGroupToBearer message to MC gateway UE. When the association of group call, MBMS bearer and the MC GW service ID of the MC client is known to the MC gateway UE, it can forward the traffic received over MBMS bearer accordingly.

Figure 11.5.3.3.2-1 illustrates the procedure for handling the MapGroupToBearer message by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.2-1: Handling of MapGroupToBearer message

1.	The MC service server sends a MapGroupToBearer message over a previously activated MBMS bearer to all users that will receive the call over an MBMS bearer. The MapGroupToBearer message includes association information between the group call and MBMS bearer. The MapGroupToBearer message includes MC service group ID and information about the media stream identifier of the activated MBMS bearer and may include the identifier (i.e. the TMGI) of the MBMS bearer broadcasting the call.

2.	If the MC client is participating in the MC group communication identified by the MapGroupToBearer message, it sends the details contained in the MapGroupToBearer message to the MC gateway UE through MC GW MapGroupToBearer request message.

3.	The MC gateway UE on receiving the MC GW MapGroupToBearer Request message from the MC client it maintains the association between the MC Service ID and the corresponding MBMS sub channel.

4.	The MC gateway UE sends the MC GW MapGroupToBearer response message to the MC client which contains the details of the non-3GPP transport resources related parameters. The MC gateway UE may choose to send the details of existing communication channel information as part of non-3GPP transport resources establishment parameters IE if existing transport resources can be reused.

5.	The MC client establishes the communication channel with the MC gateway UE based on the parameters received in step 4 to receive the MC service group communication data from the MC gateway UE, if these parameters are not referring to any of the already established transport resources. The MC Gateway UE forwards the MC service group communication data it received over the MBMS bearer from the MC service server to the MC client over this transport resources.

6.	The MC service server sends the downlink media for the group communication session over the MBMS bearer.

7.	The MC gateway UE checks which MC clients should receive the media of the MC group communication based on Step 3.

8.	The MC gateway UE forwards the downlink media to the intended MC clients over the transport resources established as in step 5.

##### 11.5.3.3.2A	Procedure for handling UnmapGroupFromBearer message

Whenever the MC client receives UnmapGroupFromBearer message and if the MC client participates in the group session or communication identified by the UnmapGroupFromBearer message then it should inform the details contained in the UnmapGroupFromBearer message to MC gateway UE. When the association of group call, MBMS bearer and the MC GW service ID of the MC client is known to the MC gateway UE, it can remove the association and stop forwarding of the traffic received over MBMS bearer accordingly.

Figure 11.5.3.3.2A-1 illustrates the procedure for handling the UnmapGroupFromBearer message by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.2A-1: Handling of UnmapGroupFromBearer message

1.	An MCPTT group call is ongoing; the MC gateway UE forwards the downlink media to the intended MC client over the transport resources established.

2.	MCPTT server has determined to disconnect the call over the MBMS bearer for the MC client.

3.	An UnmapGroupFromBearer message is sent by the MCPTT server to MC client (if in MBMS coverage area) on MBMS bearer(s).

4.	If the MC client is participating in the MC group communication identified by the UnmapGroupFromBearer message, it sends the details contained in the UnmapGroupFromBearer message to the MC gateway UE through MC GW UnMapGroupToBearer request message.

5.	The MC gateway UE on receiving the MC GW UnMapGroupToBearer Request message from the MC client it removes the association maintained between the MC Service ID and the corresponding MBMS sub channel.

6.	The MC gateway UE sends the MC GW MapGroupToBearer response message to the MC client.

# * * * * 6th change * * * *

# A.7	Void

# * * * * End of changes * * * *