3GPP TSG-SA WG6 Meeting #62	S6-243131

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-24xxxx)

Source:	Ericsson

Title:	Pseudo-CR on information on NGSO handover

Spec:	3GPP TR 23.700-01 v 1.0.0

Agenda item:	8.6

Document for:	Approval

Contact:	Rana Alhalaseh, rana.alhalaseh@ericsson.com

1. Introduction

This pCR adding a new annex that provides information about handover for NGSO. The provided information concluded that the frequent handover experienced by the UE in LEO NTN is similar to the UE in TN traveling in a high-speed train.

2. Reason for Change

Provides information related to time to handover for varying UE speed.

3. Conclusions

Provides information related to time to handover for varying UE speed, and it concluded that the frequent handover experienced by the UE in LEO NTN is similar to the UE in TN traveling in a high-speed train.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v 1.0.0

* * * First Change * * * *

# X	Handover for NGSO

## X.1		General

As NGSO satellites move relatively faster in comparable to a fixed point in the earth, the UEs (both stationary and moving) have to perform handover while connected to the NGSO, e.g., LEO satellites.

## X.2		Time to handover

3GPP TR 38.821 [12] has investigated the time a UE (both stationary and moving with constant speed and one direction) needs to perform handover from one satellite to another. For the calculations, two scenarios where considered: one where the NTN cell size is 50 km diameter beamer assuming as the worst-case scenario for frequent handover. Another where the NTN cell size is 100 km diameter beamer as the best-case scenario. Table X.1-1represents the maximum time a UE can remain in an NTN cell before it performs handover. For the calculations, the satellite speed is assumed to be 7.56 km/s.

Table X.1-1: Time to handover for varying UE speed

| Cell Diameter Size (km) | UE Speed (km/hr) | Time to handover (s) |
| --- | --- | --- |
| 50 (lower bound) | +500 | 6.49 |
|  | -500 | 6.74 |
|  | +1200 | 6.33 |
|  | - 1200 | 6.92 |
|  | Neglected | 6.61 |
| 1000 (upper bound) | +500 | 129.89 |
|  | -500 | 134.75 |
|  | +1200 | 126.69 |
|  | - 1200 | 138.38 |
|  | Neglected | 132.28 |

For stationary UEs, i.e., with neglected speed, the UE is served for 6.61 s and 132.28 s by an NTN cell of 50 km diameter beamer and 100 km diameter beamer, respectively.

The case the UE is moving with a speed of 500 km/hr is equivalent to a UE served by NT cell diameter range of approximately 0.918 – 18.39 km.

Based on the analysis provided in 3GPP TR 38.821 [12] it can be concluded that the frequent handover experienced by the UE in LEO NTN is similar to the UE in TN traveling in a high-speed train. Furthermore, the larger the satellite beamer is, the more time until the UE performs handover to another satellite.

* * * End of Changes * * * *