3GPP TSG-SA WG6 Meeting #62	S6-243145

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6a240xxx)

Source:	Samsung

Title:	Pseudo-CR on update to option 2 architecture

Spec:	3GPP TR 23.700-21 (v1.1.0)

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR provides updates to the option#2 architecture.

2. Reason for Change

Following EN needs to be resolved:

Editor's Note: How VAL client(s) access the services of A-DACM function is FFS.

- Provided solution updates to align with SEAL architecture

- Provided mapping with CAPIF architecture

- Provided deployment option how VAL client can access SEAL services as an API invoker.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 (v1.1.0).

* * * First Change * * * *

## 6.2	Option #2: A-DACM architecture to support Metaverse services

### 6.2.1	Application enablement architecture

Figure 6.2.1-1 illustrates the A-DACM architecture to support Metaverse services using SEAL architecture specified in 3GPP TS 23.434 [8].

Figure 6.2.1-1: A-DACM architecture to support Metaverse services

The SEAL architecture includes A-DACM client and server which supports the VAL layer. The A-DACM server interacts with the VAL server(s) over the SEAL-S reference point. The A-DACM client interacts with the VAL client(s) over the SEAL-C reference point. The interactions between A-DACM client and A-DACM server is over the SEAL-UU reference point. The A-DACM server may utilize the 5GC services over N33 reference point.

Figure 6.2.1-2 illustrates the A-DACM architecture to support Metaverse services using CAPIF architecture specified in 3GPP TS 23.222 [14].

Figure 6.2.1-2: A-DACM architecture to support Metaverse services using CAPIF architecture

The SEAL architecture includes A-DACM server which supports the VAL layer. The VAL server(s) (acting as CAPIF’s API invokers) interact with the A-DACM server (acting as CAPIF’s AEF) over the SEAL-S reference point (compliant with CAPIF-2/2e). The SEAL client(s) (acting as CAPIF’s API invoker) interact with the A-DACM server (acting as CAPIF’s AEF) over the SEAL-UU reference point (compliant with CAPIF-2/2e). The service APIs offered by A-DACM server on SEAL-UU and SEAL-S to the VAL layer functions follow the service-based architecture as shown in the Figure 6.2.1-4. The procedures of CAPIF as specified in 3GPP TS 23.222 [14] apply for the interactions between VAL layer and A-DACM server. The A-DACM server may utilize the 5GC services over N33 reference point.

Figure 6.2.1-3 illustrates possible deployment where VAL client interacts with VAL server over SEAL-UU interface. It is assumed that all VAL clients requiring direct interaction with SEAL server has implemented SEAL client functionalities.

Figure 6.2.1-3: VAL client and SEAL client combined deployment

NOTE 1:	When A-DACM client is provided by OEM vendors, deployment as shown in Figure 6.2.1-3 is not feasible.

Figure 6.2.1-4 illustrates the service-based SEAL architecture to support Metaverse services.

Figure 6.2.1-2: Service-based SEAL architecture to support Metaverse services

The A-DACM function exposes the APIs over the Sdacm interface to the VAL layer. The VAL functions (VAL servers) consume the services by interacting on the Sdacm interface of the A-DACM function. The CAPIF can be used by VAL functions to discover the services of the A-DACM function.

NOTE:	The A-DACM function can be realized as a new SEAL service or part of existing SEAL service.

### 6.2.2	Functional Elements

#### 6.2.2.1	A-DACM function

The A-DACM function expose the services/APIs to the VAL layer services over Sdacm interface for the following avatar management capabilities:

-	Avatar profile creation;

-	Avatar upload;

-	Avatar linking to user/subscriber;

-	Avatar download;

-	Avatar update/modification;

-	Avatar delete;

### 6.2.3	Reference Points

#### 6.2.3.1	SEAL-S

This reference point supports the interactions between VAL server(s) and A-DACM function. The VAL server consumes the avatar management services supported by the A-DACM function as specified in clause 6.2.2.1.

### 6.2.4	Service-based interface

#### 6.2.4.1	Sdacm

This interface supports the interactions between VAL function (VAL servers) and A-DACM function. The VAL function consumes the avatar management services supported by the A-DACM function as specified in clause 6.2.2.1.

* * * End of Change * * * *