3GPP TSG-SA WG6 Meeting #62	S6-243148

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Samsung

Title:	Pseudo-CR on spatial anchors management architecture

Spec:	3GPP TS 23.437 (v0.1.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR provides architecture for Spatial anchors management.

2. Reason for Change

In FS_Metaverse_App study, KI#1 was studied to provide support for managing spatial anchors.

Based on the architecutre conclusion: “For Key issue #1 Enabler support for managing spatial anchors, it is concluded to define new SEAL server to support spatial anchors management. The new SEAL server will consider to re-use the architecture as specified in option#1.”

- A new SEAL server is required to support mangement of spatial anchors.

- It is required to define architecture for the new SEAL server which is based on SEAL archtiecture defined in TS 23.434.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (v0.1.0)

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r23434]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to TR 21.905 be the first in the list.

* * * Next Change * * * *

## 8.2	Functional model for spatial anchors management

### 8.2.1	General

This clause provides architecture for spatial anchors management (SAnM) service. The on-network functional model is specified in clause 8.2.2, and service based architecture is specified in clause 8.2.3.

### 8.2.2	On-network functional model description

Figure 8.2.2-1 illustrates an on-network functional model for Spatial Anchors (SAn) Management service.

Figure 8.2.2-1: On-network Spatial Anchors management functional model

The Spatial Anchors management functional entities are grouped into the vertical application layer and the Service Enabler Architecture Layer (SEAL) layer. The Spatial Anchors management offers the capabilities to manage spatial anchors to the vertical application layer. The Spatial Anchors management functional model is based on SEAL layer as specified in 3GPP TS 23.434 [r23434].

The spatial anchors management functional entities on the UE and in the network are respectively the spatial anchors management (SAnM) client and spatial anchors management (SAnM) server. The VAL client and VAL server are application client and server respectively to provide an application service to the user. The SAnM client and SAnM server provides client and server side functionalities respectively for managing spatial anchors for the application. The SAnM client is a SEAL client providing spatial anchors management service to VAL client. The SAnM server is a SEAL server providing spatial anchors management service to VAL server and UE. The interface between SAnM Client and SAnM Server is SAnM-UU. The SAnM client interacts with the VAL client using a SAnM-C interface. The SAnM server interacts with the VAL server using a SAnM-S. The SAnM server communicates with the underlying 3GPP network system using 3GPP interfaces specified by the 3GPP network system (e.g. T8, N33).

#### 8.2.2.1	Functional Elements

##### 8.2.2.1.1	Spatial anchors management (SAnM) server

The SAnM server acts as a SEAL client and it is used to provide services to create/update/delete/discover spatial anchors for the VAL application (i.e. metaverse application). It can communicate with other SAnM server in the distributed SEAL deployments.

##### 8.2.2.1.2	Spatial anchors management (SAnM) client

The SAnM client acts as a SEAL client for spatial anchors management function. It supports client side functionalities for creating/updating/deleting/discovering/retrieving spatial anchors for VAL application (i.e. metaverse application).

##### 8.2.2.1.3	VAL server

The VAL server is specified in clause 6.4.2.3 of 3GPP TS 23.434 [r23434]. The VAL server provides the server side functionalities corresponding to the vertical applications (i.e. metaverse applications) to the client of a VAL user based on a service contract.

##### 8.2.2.1.4	VAL client

The VAL client is specified in clause 6.4.2.2 of 3GPP TS 23.434 [r23434]. The VAL client provides a metaverse services to the user. It uses the SAnM client to manage spatial anchors for the application.

#### 8.2.2.2	Reference Points

##### 8.2.2.2.1	SAnM-UU

SAnM-UU reference point enables interactions between a SAnM client and SAnM server. It is used to create/update/delete/discover/retrieve spatial anchors for the VAL applications (i.e. metaverse applications).

##### 8.2.2.2.2	SAnM-S

SAnM-S reference point enables interactions between a SAnM server and VAL servers (i.e. metaverse application servers). It can be used to create/update/delete/get/retrieve spatial anchors for the VAL applications (i.e. metaverse applications).

##### 8.2.2.2.3	SAnM-E

SAnM-E reference point enables interactions between SAnM server instances.

##### 8.2.2.2.4	SAnM-C

SAnM-C reference point enables interactions between a SAnM client and VAL clients (i.e. metaverse application clients). It is used by the VAL client to share information regarding spatial anchors related information to the SAnM client and also to get spatial anchors related information from the SAnM client.

### 8.2.3	Service based architecture

Figure 8.2.3-1 illustrates the service-based SEAL architecture to support spatial anchors management services.

Figure 8.2.3-1: service based architecture for spatial anchors management

The SAnM server exposes the APIs over the Ssanm interface to the VAL server and SAnM client. The VAL servers consume the services by interacting on the Ssanm interface of the SAnM server. The CAPIF can be used by VAL functions to discover the services of the SAnM server.

* * * End of Change * * * *