3GPP TSG-SA WG6 Meeting #62	S6-243149

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Samsung

Title:	Pseudo-CR on Spatial anchor create and update operation

Spec:	3GPP TS 23.437 (v0.1.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR provides spatial anchor create and update operation procedures.

2. Reason for Change

KI #1 in TR 23.700-21 is concluded as follows:

“for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.”

It is requried to provide normative work for the solution of KI #1.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (v0.1.0).

* * * First Change * * * *

### 8.3.x	Managing spatial anchor

#### 8.3.x.1	Procedures

##### 8.3.x.1.1	Creating spatial anchor

Figure 8.3.x.1.1-1 depicts the procedure for creating spatial anchor associated with location for metaverse applications.

Figure 8.3.x.1.1-1: Create spatial anchor

1)	The VAL server (or SAnM client) sends a request message to the SAnM server to create spatial anchor associated with the location. The request includes VAL server identity, position of the anchor and application service identifier, security credentials, spatial anchor service area. The request may include service information of the product to associate it with the spatial anchor, access control rules defining which entities are permitted to discover and access the spatial anchor, spatial anchor specific information. The request also includes spatial anchor discoverable visibility information for shared spatial anchor.

NOTE:	The spatial anchor specific information is out of scope of standardization. For example, the spatial anchor specific information may contain application specific service information like a product name, a URL, a media data, a product type, a manufacturing company, a date, an expiry date, a ratings, and customer premise information (e.g. a residence, office or shop), etc. Such information may be stored in key-value pair and is transparent to the mobile metaverse enabler.

2)	The SAnM server authorizes VAL server (or SAnM client). If the requestor is authorized, then the SAnM server creates the association between spatial anchor and the location, and stores the information as received in the request message. The server creates identity for the spatial anchor. The SAnM server sends the response back to the VAL server (or SAnM client) including identity for the spatial anchor for a successful case.

###### 8.3.x.1.1.1	Information flows for creating spatial anchor

Table 8.3.x.1.1.1-1 shows the request sent by a VAL server (or SAnM client) to a SAnM server for the creating spatial anchor.

Table 8.3.x.1.1.1-1: Create spatial anchor request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL UE and VAL user) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL service id | M | Application service identifier |
| Position | M | Position of the anchor in 3D (x, y and z coordinates) |
| Service area | O | Service area where the spatial anchor is accessible |
| Access control rules | O | Access control for the spatial anchor |
| > list of VAL service ids | O | List of application services which can access the spatial anchors |
| Visibility | O | If present, indicates that the spatial anchor is shared (or any application can access the spatial anchor) |
| spatial anchor specific information | O | A container to contain application specific information. |

Table 8.3.x.1.1.1-2 shows the response sent by the SAnM server to the requester for a create spatial anchor request.

Table 8.3.x.1.1.1-2: Create spatial anchor response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail) |
| Spatial anchor ID | O | Identifier of the spatial anchor. This IE is included in case of success response |
| Failure cause | O | Reason for the failure. This IE may be included in case of failure response. |

* * * Next Change * * * *

##### 8.3.x.1.2	Updating spatial anchor

Figure 8.3.x.1.2-1 depicts the procedure for updating spatial anchor associated with location for metaverse applications.

Figure 8.3.x.1.2-1: Update spatial anchor

1)	The VAL server (or SAnM client) sends a request message to the SAnM server to update spatial anchor associated with the location. The request includes identify of the spatial anchor, VAL server identity, and application service identifier and security credentials. The request may include service information of the product to associate it with the spatial anchor, access control rules defining which entities are permitted to discover and access the spatial anchor, spatial anchor specific information. The request also includes spatial anchor discoverable visibility information for shared spatial anchor.

2)	The SAnM server authorizes VAL server (or SAnM client). If the requestor is authorized, then the SAnM server checks whether the spatial anchor as identified by the identity of the spatial anchor exists or not. If the spatial anchor does not exists then the SAnM server sends failure response. Otherwise, the SAnM server updates the spatial anchor details. The SAnM server sends the response back to the VAL server (or SAnM client) including reason for failure for failure case.

###### 8.3.x.1.2.1	Information flows for updating spatial anchor

Table 8.3.x.1.2.1-1 shows the request sent by a VAL server (or SAnM client) to a SAnM server for the updating spatial anchor.

Table 8.3.x.1.2.1-1: Update spatial anchor request

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor ID | M | Identifier of the spatial anchor |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL UE and VAL user) |
| Requestor security credentials | M | The security credentials of the requestor. |
| VAL service id | O | Application service identifier |
| Position | O | Position of the anchor in 3D (x, y and z coordinates) |
| Service area | O | Service area where the spatial anchor is accessible |
| Access control rules | O | Access control for the spatial anchor |
| > list of VAL service ids | O | List of application services which can access the spatial anchors |
| Visibility | O | If present, indicates that the spatial anchor is shared (or any application can access the spatial anchor) |
| spatial anchor specific information | O | A container to contain application specific information. |

Table 8.3.x.1.2.1-2 shows the response sent by the SAnM server to the requester for a update spatial anchor request.

Table 8.3.x.1.2.1-2: Update spatial anchor response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail) |
| Spatial anchor ID | O | Identifier of the spatial anchor. This IE is included in case of success response |
| Failure cause | O | Reason for the failure. This IE may be included in case of failure response. |

##### 8.3.x.1.3	Getting spatial anchor

Editor’s Note: This clause provides procedure for getting Spatial Anchor.

##### 8.3.x.1.4	Deleting spatial anchor

Editor’s Note: This clause provides procedure for deleting Spatial Anchor.

* * * Next Change * * * *

#### 8.3.x.2	Service

##### 8.4.x.2.1	General

Service description: This service enables an VAL server (or SAnM client) manage spatial anchors on SAnM server.

##### 8.4.x.2.2	Create operation

Description: The consumer requests to create spatial anchors.

Inputs: See Table 8.3.x.1-1.

Outputs: See Table 8.3.x.1-2.

See clause 8.3.x for details of usage of this operation.

##### 8.4.x.2.3	Update operation

Description: The consumer requests to update spatial anchors.

Inputs: See Table 8.3.x.2.1-1.

Outputs: See Table 8.3.x.2.1-2.

See clause 8.3.x.2 for details of usage of this operation.

* * * End of Change * * * *