3GPP TSG-SA WG6 Meeting #62	S6-243150

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Samsung

Title:	Pseudo-CR on spatial map management architecture

Spec:	3GPP TS 23.437 (v0.1.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR provides architecture for Spatial mapping management.

2. Reason for Change

In FS_Metaverse_App study, KI#4 was studied to provide support for managing spatial mapping.

Based on the conclusion: “Solution #8 can be adopted in the normative work by defining new SEAL server for spatial map management.”

- A new SEAL server is required to support mangement of spatial mapping.

- It is required to define architecture for the new SEAL server which is based on SEAL archtiecture defined in TS 23.434.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (v0.1.0)

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[r23434]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to TR 21.905 be the first in the list.

* * * Next Change * * * *

## 9.2	Functional model for spatial map management

### 9.2.1	General

This clause provides architecture for spatial mapping management (SM) service. The on-network functional model is specified in clause 9.2.2, and service based architecture is specified in clause 9.2.3.

### 9.2.2	On-network functional model description

Figure 9.2.2-1 illustrates an on-network functional model for spatial mapping (SM) management service.

Figure 9.2.2-1: On-network spatial mapping management functional model

The Spatial mapping management functional entities are grouped into the vertical application layer and the Service Enabler Architecture Layer (SEAL) layer. The Spatial mapping management offers the capabilities to manage spatial anchors to the vertical application layer. The Spatial mapping management functional model is based on SEAL layer as specified in 3GPP TS 23.434 [r23434].

The Spatial mapping management functional entities on the UE and in the network are respectively the spatial mapping management (SMM) client and spatial mapping management (SMM) server. The VAL client and VAL server are application client and server respectively to provide an application service to the user. The SMM client and SMM server provides client and server side functionalities respectively for managing spatial anchors for the application. The SMM client is a SEAL client providing spatial anchors management service to VAL client. The SMM server is a SEAL server providing spatial anchors management service to VAL server and UE. The interface between SMM Client and SMM Server is SMM-UU. The SMM client interacts with the VAL client using a SMM-C interface. The SMM server interacts with the VAL server using a SMM-S. The SMM server communicates with the underlying 3GPP network system using 3GPP interfaces specified by the 3GPP network system (e.g. T8, N33).

#### 9.2.2.1	Functional Elements

##### 9.2.2.1.1	Spatial mapping management (SMM) server

The SMM server acts as a SEAL client and it is used to provide services to create/update/delete/discover spatial map for the VAL application (i.e. metaverse application) and also provides spatial localization service. It can communicate with other SMM server in the distributed SEAL deployments.

##### 9.2.2.1.2	Spatial mapping management (SMM) client

The SMM client acts as a SEAL client for spatial mapping management function. It supports client side functionalities for creating/updating/deleting/discovering/retrieving spatial map for VAL application (i.e. metaverse application) and also provides spatial localization service.

##### 9.2.2.1.3	VAL server

The VAL server is specified in clause 6.4.2.3 of 3GPP TS 23.434 [r23434]. The VAL server provides the server side functionalities corresponding to the vertical applications (i.e. metaverse applications) to the client of a VAL user based on a service contract.

##### 9.2.2.1.4	VAL client

The VAL client is specified in clause 6.4.2.2 of 3GPP TS 23.434 [r23434]. The VAL client provides a metaverse services to the user. It uses the SMM client to manage spatial maps for the application.

#### 9.2.2.2	Reference Points

##### 9.2.2.2.1	SMM-UU

SMM-UU reference point enables interactions between a SMM client and SMM server. It is used to create/update/delete/discover/retrieve spatial map for the VAL applications (i.e. metaverse applications).

##### 9.2.2.2.2	SMM-S

SMM-S reference point enables interactions between a SMM server and VAL servers (i.e. metaverse application servers). It can be used to create/update/delete/get/retrieve spatial map for the VAL applications (i.e. metaverse applications).

##### 9.2.2.2.3	SMM-E

SMM-E reference point enables interactions between SMM server instances.

##### 9.2.2.2.4	SMM-C

SMM-C reference point enables interactions between a SMM client and VAL clients (i.e. metaverse application clients). It is used by the VAL client to share information regarding spatial maps related information to the SMM client and also to get spatial maps related information from the SMM client.

### 9.2.3	Service based architecture

Figure 9.2.3-1 illustrates the service-based SEAL architecture to support Spatial mapping management services.

Figure 9.2.3-1: Service based architecture for spatial mapping management

The SMM server exposes the APIs over the Ssmm interface to the VAL server and SMM client. The VAL servers consume the services by interacting on the Ssmm interface of the SMM server. The CAPIF can be used by VAL functions to discover the services of the SMM server.

* * * End of Change * * * *