3GPP TSG-SA WG6 Meeting #62	S6-243154

# Maastricht, NL, 19 - 23 August, 2024

Source:	Ericsson, KPN N.V.

Title:	AIMLE policies for AIMLE Client selection and discovery

Spec:	3GPP TR 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

1. Introduction

Solution#14 that addresses AIMLE policies provisioning and management as a part of AIMLE Client Discovery and Selection API.

2. Reason for Change

This CR implements the Solution#14 from 3GPP TR 23.700-82. Based on the conclusions of KI#7:

-	Solution #6 and Solution #14 are complementing each other and can be considered for the normative work. It is recommended to specify Solution #6 and Solution #14 as a part the AIMLE Client Discovery and Selection API.

NOTE:	This functionality is a part of AIMLE Client Discovery and Selection API. Please, implement this CR in this API.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.X	AIMLE policies provisioning and management

### 8.X.1	General

This clause describes procedures, information flows for provisioning and managing the AIMLE policies for the AI/ML Enablement services.

### 8.X.2	Procedures

#### 8.X.2.1	AIMLE policies provisioning and management

Figure 8.14.2-1: AI/ML policies provisioning and management procedure

1.	The VAL server sends an AI/ML service request with policies provisioning and management information as defined in clause 8.X.3.1.

2.	Upon receiving the request, the AIMLE Server performs an authorization check of the VAL server.

3.	If the VAL server is authorised, the AIMLE Server stores/updates/deletes the AIMLE policies and applies to ongoing and further the AI/ML operations (e.g., ML model training). The AI/ML application QoS adjustment policies are applied for the selected AI/ML members as per the member selection and re-selection policy after obtaining the identifiers of the AI/ML members that are selected as a result of the enforcement of the policy. The AIMLE server can use the SEALDD or the NEF services (AFSessionWithQoS) to configure the QoS. When the AI/ML members are de-selected, the QoS adjustment is reversed as they no longer participate in the AI/ML operation.

4.	AIMLE Server provides the response to the VAL server with a status via AIMLE member selection policies provisioning and management response defined in clause 8.X.3.2.

NOTE:	The service operation defined in clause 8.X.2.1 can be utilized for AI/ML member selection policies management (i.e., update/delete) functionality.

### 8.X.3	Information flows

#### 8.X.3.1	AIMLE policy provisioning request

Table 8.X.3.1-1 describes the information flow from the VAL server to the AIMLE Server as a request that contains the AIMLE policies provisioning and management information.

Table 8.X.3.1-1: AIMLE policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| List of member selection and re-selection polices | O (NOTE) | Identifies the list of member selection and re-selection policies, e.g., the AI/ML member shall be re-selected when the PLR is above the given value or leaves the certain area. See details in Table 8.X.3.1-2. |
| List of AI/ML traffic QoS adjustment policies | O (NOTE) | Identifies the list of QoS adjustment policies, e.g., the AI/ML traffic QoS shall be adjusted when the conditions are met. See details in Table 8.X.3.1-3. |
| AI/ML data transfer policy for AI/ML member re-selection | O (NOTE) | Identifies the AI/ML data transfer policy that shall be applied for the AI/ML data transfer from de-selected to the selected AI/ML member during the re-selection procedure. See details in Table 8.X.3.1-4. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Table 8.X.3.1-2: Member selection and re-selection policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., select member, not select member, or re-select member. |
| List of AI/ML traffic QoS conditions | O (NOTE) | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |
| List of AI/ML member mobility conditions | O (NOTE) | Provides the list of mobility conditions for the AI/ML member associated with the Action, e.g., low mobility, high mobility, stationary. |
| List of AI/ML member participation conditions | O (NOTE) | Provides the list of the AI/ML member participation conditions associated with the Action. The AI/ML member participation configurations provided by the AI/ML members are matched with the provided conditions detailed in Table 8.X.3.1-5 for member selection and re-selection. |
| NOTE: At least one of these IEs shall be provided. |  |  |

Table 8.X.3.1-3: AIMLE traffic QoS adjustment policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., adjust AIML traffic QoS. |
| List of the required AI/ML traffic QoS values | M | Represents the required QoS values that shall be fulfilled when the conditions are met. |
| List of AI/ML traffic QoS conditions | M | Provides the list of AI/ML traffic QoS conditions associated with the Action, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). |

Table 8.X.3.1-4: AIMLE data transfer policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., initiate background AI/ML data transfer or configure a temporary AI/ML data transfer with QoS during AI/ML member re-selection procedure. |
| List of the required AI/ML traffic QoS values | O (NOTE) | Represents the required network QoS values that shall be enforced for the AI/ML data transfer with QoS. This IE is applicable for the "configure a temporary AI/ML data transfer with QoS". |
| Background data transmission details and policy guidance | O (NOTE) | Provides the background data transmission details and policy guidance as described in clause 14.3.2.58 of 3GPP TS 23.434 [7]. This IE is applicable for the "initiate background AI/ML data transfer" action. |
| NOTE: Only one of these information elements shall be provided. |  |  |

Table 8.X.3.1-5: List of AIMLE member participation conditions

| Information element | Status | Description |
| --- | --- | --- |
| List of conditions | M | List of conditions for AI/ML member selection and re-selection. |
| >VAL service ID | O | The VAL service ID required to be supported by the AI/ML member. |
| >List of time-schedule conditions | O | Provides the required list of time-schedule conditions, e.g., the AI/ML member is required to be available to participate in the AI/ML operations in the given time slot(s) and/or day(s) of the week. |
| >List of location-based conditions | O | Provides the required list of location-based conditions, e.g., the AI/ML member is required to be available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| >Mode of member participation | O | Indicates the required AI/ML member availability for the participation based on the modes like training (e.g., FL, DML, ML, and/or Split Learning), aggregator, coordinator and/or inference. |
| >Service exposure level | O | Indicates required level of service exposed by the AI/ML member for the corresponding VAL service ID e.g., premium resource usage, limited resource usage, raw data, processed data. |

#### 8.X.3.2	AIMLE policy provisioning response

Table 8.X.3.2-1 describes the information flow from the AIMLE Server to the VAL server as a response that contains the AIMLE policies provisioning and management result.

Table 8.X.3.2-1: AI/ML member selection policies provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

* * * Next change * * * *

### 9.Y.A	Create_AIMLE_Policies

API operation name: Create_AIMLE_Policies

Description: The consumer provisions the AIMLE Policies.

Inputs: See clause 8.X.3.1.

Outputs: See clause 8.X.3.2.

See clause 8.X.2.1 for details of usage of this operation.

### 9.Y.B	Update_AIMLE_Policies

API operation name: Update_AIMLE_Policies

Description: The consumer updates the AIMLE Policies.

Inputs: See clause 8.X.3.1.

Outputs: See clause 8.X.3.2.

See clause 8.X.2.1 for details of usage of this operation.

### 9.Y.C	Delete_AIMLE_Policies

API operation name: Delete_AIMLE_Policies

Description: The consumer deletes the AIMLE Policies.

Inputs: See clause 8.X.3.1.

Outputs: See clause 8.X.3.2.

See clause 8.X.2.1 for details of usage of this operation.

* * * End of changes * * * *