3GPP TSG-SA WG6 Meeting #62	S6-243156

# Maastricht, NL, 19 - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0305 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment, correction and clarification of the location-based group creation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2024-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Location-based group creation procedure defined in clause 9.3.10 has non-specified steps 2, 3, and 4, i.e., it is not specified what API can be used by the SEAL Group Management server to retrieve the related VAL UEs/VAL Users from the Location Management server. Thus, these steps require specification. In this CR, it is proposed to re-use SS_LocationAreaInfoRetrieval API and Obtain_UEs_Info operation for the VAL UEs/VAL Users retrieval from the Location Management server. The Obtain_UEs_Info operation procedure and information flows are updated to satisfy the location-based group creation operation requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction, clarification and alignment of the location-based group creation and obtaining UE(s) information at a location service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Location-based group creation service operation is not fully specified that may lead to interoperability issues between group management and location management servers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.9, 9.3.2.10, 9.3.10, 10.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 9.3.2.9	Get UE(s) information request

Table 9.3.2.9-1 describes the information flow for a VAL server to get UE(s) information at the LM server.

Table 9.3.2.9-1: Get UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL service IDs | O | Identifies the list of VAL service IDs associated with that request. |
| Location information | M | Location information around which the UE(s) information is requested (e.g. geographic area or VAL service area ID). |
| Application defined proximity range information | M | Description of the range information over which the UE(s) information is required. |

* * * Next change * * * *

#### 9.3.2.10	Get UE(s) information response

Table 9.3.2.10-1 describes the information flow for a LM server to respond with UE(s) information to the VAL server.

Table 9.3.2.10-1: Get UE(s) information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the VAE server in response to subscription request indicating success or failure |
| List of UEs information (see NOTE) | O | The information of the UEs which were detected in the application defined proximity range. The list can be empty. |
| >UE ID | M | The identifier of VAL UE or VAL user. |
| >Location information | M | Location information of UE within the application defined proximity range |
| > VAL service ID | O | Indicates the VAL service identified associatied with UE ID information flow. |
| NOTE: This IE shall be included when the Result indicates success. |  |  |

* * * Next change * * * *

### 9.3.10	Obtaining UE(s) information at a location

Figure 9.3.10-1 describes the procedure for obtaining UE(s) information at a location.

Pre-condition:

-	The VAL server has a jurisdiction over a geographical area for which the location management server is configured to operate.

-	The UE(s) in the geographical area have provided its location information to the location management server.

Figure 9.3.10-1: Obtaining UE(s) information at a location

1.	The consumer (VAL server or group management server) sends get UE information request to the location management server. The request contains a location information or VAL service area identified by the VAL service area ID and application defined proximity range and may include a list of VAL service identifiers.

2.	The location management server determines the VAL UE(s) or VAL User(s) whose location are within the application defined proximity range of the location information or VAL service area identified by the VAL service area ID provided in step 1. The location management server may provide filtering of the VAL UE(s) or VAL User(s) based on VAL service identifier provided in step 1.

3.	The location management server sends get UE information response to the VAL server with a list of VAL UE(s) or VAL user(s) and its corresponding location information as determined in step 1.

* * * Next change * * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator or VAL server is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message and the location criteria may be the VAL service area identified by the VAL service area ID.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information or the VAL service area identified by the VAL service area ID as defined in clause 9.3.2.9.

3.	The location management server composes the list of users or UEs within the requested location as defined in clause 9.3.10.

4.	The group management server receives the composed list of users or UEs from the location management server as described in clause 9.3.2.10. The location management server may include VAL service IDs associated with the list of users or UEs.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group's configuration information. If VAL service ID list was received in step 1 as part of group formation criteria and not in step 4 for the composed list of users or UEs, the group management server shall retrieve the VAL service data from the configuration management server as described in clause 11.3.2.13 and form the group with relevant users or UEs.

NOTE:	The exact policies are out of scope of the present document.

6.	The group management server provides a location-based group creation response to the group management client or the VAL server.

* * * End of changes * * * *