3GPP TSG-SA WG6 Meeting #62	S6-243159

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	ZTE Corporation

Title:	Discussion on the purpose of data processing management

Agenda Item:	8.7

Contact:	Yang Li (li.yang1226@zte.com.cn)

Abstract: This paper provides an analysis for how to capture the purpose of data processing and where will it be stored.

- Background

According to clause Annex V.2 in TS 33.501, the user consent parameters shall be bound to the purpose of data processing. Any NF that is deemed an enforcement point for user consent (e.g., AEF) shall determine the purpose of data processing prior to the data processing. If the purpose of data processing is not obtained from the service request, the AEF shall request it or otherwise deny the service.

V.2 Requirements The UDM shall support the following services related to the user consent. - Retrieval of user consent parameters. - Notification of user consent parameters change. The user consent parameters shall be stored in the UDM/UDR as subscription data. The user consent parameters shall be bound to a SUPI/GPSI. The user consent parameters shall be bound to the purpose of data processing. The user consent parameters shall include whether the user consent is granted or not. The user consent shall be effective only after the point in time that user consent was given.The user consent shall be effective until revoked. It means that there is no expiry/validity timer for the user consent parameters stored in the subscription data. NOTE: UDM does not provide user consent revocation service, it only provides notifications of user consent parameter changes. V.3 User consent check Any NF that is deemed an enforcement point for user consent shall support to retrieve the user consent parameters from the UDM. Any NF that is deemed an enforcement point for user consent shall not accept any services or requests for data processing unless user consent is granted. Any NF that is deemed an enforcement point for user consent shall determine the purpose of data processing prior to the data processing. If the purpose of data processing is not implicitly known from the service request, the user consent enforcement point shall request it or otherwise deny the service. NFs obtaining or checking the user consent parameters shall consider the user consent parameters as effective until revoked.

In this discussion paper, an analysis for how to capture the purpose of data processing and where will it be stored.

- Discussion

- Generally, the purpose of data processing depends on business requirements which is provided by the API invoker, and shall comply with local regulations. Therefore, the purpose of data processing can be considered to be provided by the API invoker. To ensure that the purpose of data processing complies with local regulations, consideration should be given to conducting a legality check on the purpose of data processing provided by the API invoker.

- Observation	1: The purpose of data processing depends on business requirements, and shall comply with local regulations.

- Proposal	1: The purpose of data processing can be considered to be provided by the API invoker.

- To assist the AEF in obtaining the purpose of data processing before data processing commences, several methods for acquiring the purpose of data processing can be considered.

- Option 1: The API invoker sends the purpose of data processing to the CCF for check and storage. When the AEF receives a service invocation request, if there is no stored purpose of data processing, it requests relevant information from the CCF.

- Option 2: The API invoker includes the purpose of data processing in the service invocation request and sends it to the AEF.

- Option 3: The API invoker initially sends the purpose of data processing to the CCF for check and storage. Subsequently, when sending service invocation requests, it also transmits the purpose of data processing to the AEF. The AEF initiates a verification process with the CCF to ensure that the purpose of data processing is consistent with the stored information.

- Compared to AEF, the CCF, as a central trusted node, is more suitable to perform the verification work. Thus, option 2 is less suitable. Option 3, combining the interaction processes of the previous two options, can reduce the potential risk of information tampering, thereby enhancing the security of the information.

- Proposal 	2: The CCF is used to verify whether the purpose of data processing complies with local regulations.

- Proposal 	3: Considering the characteristics of the purpose of data processing information, Option 1 is more suitable; however, if there are concerns about security, Option 3 can also be considered.

3 Conclusion

- In this discussion paper, we first clarified the characteristics of the purpose of data processing information and who should provide it. Secondly, we outlined possible processes for acquiring and storing the purpose of data processing. Finally, based on the information characteristics, we provided process recommendations.

- Observation	1: The purpose of data processing depends on business requirements, and shall comply with local regulations.

- Proposal	1: The purpose of data processing can be considered to be provided by the API invoker.

- Proposal 	2: The CCF is used to verify whether the purpose of data processing complies with local regulations.

- Proposal 	3: Considering the characteristics of the purpose of data processing information, Option 1 is more suitable; however, if there are concerns about security, Option 3 can also be considered.