3GPP TSG-SA WG6 Meeting #62	S6-243175

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Lenovo

Title:	Support for FL member registration

Spec:	3GPP TS 23.482 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper provides the new capability on supporting FL member registration based on the concluded Solution #9 of the TR 23.700-82.

2. Reason for Change

This paper provides the new capability on supporting FL member registration based on the concluded Solution #9 of the TR 23.700-82.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.1.0

* * * First Change * * * *

## 6.x	Support for FL member registration

This functionality covers the registration and registration update of the candidate FL member to the ML repository which is keeping the FL member registrations. Such candidate member can be a VAL server functionality or a VAL UE functionality or an enabler layer functionality (e.g. AIMLE) which is registering to the ML repository / registry to act as FL member for a given application event (analytics event or event triggered by a VAL layer application server or client).

* * * Next Change * * * *

## 8.x	Support for FL member registration

### 8.x.1	General

This clause provides the procedures for the registration and registration update of candidate FL members in the ML repository which serves as service registry for the FL members undertaking a task related to the ML model lifecycle, such as ML model local training. Candidate FL members can be application layer entities (e.g., VAL server, EAS) at the server or VAL UE side, which can potentially be selected as FL clients or FL server, FL server/aggregator for a particular ASP /vertical requirement.

### 8.x.2	Procedure on FL member registration

Figure 8.x.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.x.2-1 Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server, EAS) onboards to the platform (cloud or edge) hosting the AIMLE Server.

2.	The AIMLE Server sends a registration request to the ML repository for registering the FL members to the ML repository which acts as the AIML service registry.

3.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request.

4.	The ML repository sends the generated information in the registration response message to the AIMLE Server.

5.	The AIMLE Server configures the received information to the individual candidate FL members.

### 8.x.3	Procedure on FL member registration update

Figure 8.x.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.x.3-1: Procedure for registration update

1.	The candidate FL member (e.g. VAL server, EAS) either decides to offboard or optionally change the capabilities or a trigger is initiated based on the condition changes (e.g. VAL UE moves to an area with limited coverage) and provides this information to the AIMLE Server. Such update can be also update of the location of the FL member in different DN/EDN, or the temporary unavailability of the FL member which requires to update the registration.

2.	The AIMLE Server (or the candidate FL member) sends a registration update request to the ML repository acting as AIML service registry for updating the registration of the corresponding FL member(s) based on step 1.

3.	The ML repository validates the received update request and generates the identity and other security related information for all the FL members listed in the registration update request.

4.	The ML repository sends the generated information in the registration update response message to the AIMLE Server (or to the candidate FL member directly).

### 8.x.4	Information flows

#### 8.x.4.1	General

The following information flows are specified for supporting the FL member registration and registration update based on 8.x.2 and 8.x.3.

#### 8.x.4.2	FL member registration request

Table 8.x.4.2-1 describes information elements for the FL member registration request from the AIMLE server or the candidate FL member (VAL server, EAS) to the ML repository / registry.

Table 8.x.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration request |
| List of FL member IDs | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the VAL UE ID based on which entity is the candidate FL member. |
| > FL member supported types | M | The supported types of FL membership which can be FL server, FL client, FL aggregator. |
| > FL member capabilities | O (NOTE) | The processing and communication capabilities of the candidate FL member to be registered. |
| > FL member availability indication | O (NOTE) | The availability in certain area and time for a certain FL member. |
| > FL member activation criteria | O (NOTE) | The criteria under which the FL member will be activated/instantiated or utilized by a certain application |
| > Supported AIML services | M | The list of AIML services for which the FL member can be used |
| > Supported ML model list | O | The list of ML models for which the FL member can be used |
| > Energy/Load constraints | O | Any energy/load constraints related to the use as FL member. |
| > Charging information | O | The charging requirement for the use as FL member. |
| Area of Interest | O | The area of interest for which the registration applies |
| Time validity | O | The time validity for the registration. |
| NOTE: At least one of these shall be present |  |  |

#### 8.x.4.3	FL member registration response

Table 8.x.4.3-1 describes information elements for the FL member registration response to the AIMLE server or the candidate FL member (VAL server, EAS) from the ML repository / registry.

Table 8.x.4.3-1: FL member registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration request (positive or negative acknowledgement). |
| Registration ID | M | The generated ID for the registration. |

#### 8.x.4.4	FL member registration update request

Table 8.x.4.4-1 describes information elements for the FL member registration update request from the AIMLE server or the candidate FL member (VAL server, EAS) to the ML repository / registry.

Table 8.x.4.4-1: FL member registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration update request |
| List of FL member IDs | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the VAL UE ID based on which entity is the candidate FL member. |
| > Updated registration parameters | M | The updates of the registration parameters. |
| >> FL member supported types | O | The new or modified supported types of FL membership which can be FL server, FL client, FL aggregator. |
| >> FL member capability or availabity indication | O | New or modified capabilities for the FL member or modification of the FL member availability for the given area and time. |
| >> supported AIML services | O | The updated list of AIML services for which the FL member can be used |
| >> supported ML models | O | The updated list of ML models for which the FL member can be used |
| > area of interest | O | The updated area of interest for the registration update. |
| > time validity | O | The update time validity for the registration update. |

#### 8.x.4.5	FL member registration update response

Table 8.x.4.5-1 describes information elements for the FL member registration update response to the AIMLE server or the candidate FL member (VAL server, EAS) from the ML repository / registry.

Table 8.x.4.5-1: FL member registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration update request (positive or negative acknowledgement). |
| Registration ID | O | The generated ID for the new registration based on the update (if needed). |

* * * Next Change * * * *