3GPP TSG-SA WG6 Meeting #62	S6-243176

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	Lenovo

Title:	Support for FL event notification

Spec:	3GPP TS 23.482 v0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This contributions introduces a new ML repository capability for supporting FL related event subscriptions and notificaitons.

2. Reason for Change

This contributions introduces a new ML repository capability for supporting FL related event subscriptions and notificaitons. This is based on concluded Solution #16 of TR 23.700-82.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.1.0.

* * * First Change * * * *

## 6.x	Support for FL events notification

This functionality enables a subscriber/consumer (who can be the coordinator FL server or API invoker entity) to subscribe for AI/ML model related events and getting notified on changes on the availability of the FL members which are to be used for ML model training. This capability at the ML repository acting as an AIML service registry supports the subscription for events related to FL members by a subscriber and its notification in case of changes. This solution assumes that such FL members have previously registered to this registry their availability and capabilities (based on the capability in 6.y and 8.y).

* * * Next Change * * * *

## 8.x	Support for FL events subscription and notification

### 8.x.1	General

This clause consists of the following procedures 1) the subscription for the FL related events in clause 8.x.2 and 2) the event notification procedure in clause 8.x.3. Also, clause 8.x.4 describes a list of the FL events.

This capability mainly targets the enhancements to a global ML repository acting as AIML service registry which monitors the status and changes to the availability / capabilities of the FL members. The subscribing entity can be the AIMLE server or VAL server which requires to get notified on the FL member availability, since this information is vital for enabling the subscriber to perform the FL-related AIML operations (e.g. AIMLE server selecting FL members for a certain task).

### 8.x.2	Procedure on subscription for FL related events

This procedure, as illustrated in Figure 8.x.2-1, describes the subscription for events related to FL member availability.

Pre-conditions:

- The subscribing entity has the authorization to subscribe for the FL-related events (events described in 8.x.4).

Figure 8.x.2-1: Procedure for FL-related event subscription

1.	The subscribing entity sends an event subscription request to the ML repository in order to receive notification of events related to the ML model lifecycle changes and in particular the availability of FL member for a target service or area or UEs (such events are described in clause 8.x.4).

2.	Upon receiving the event subscription request from the subscribing entity, the ML repository checks for the relevant authorization for the event subscription.

3.	If the authorization is successful, the ML repository stores the subscription information.

4.	The ML repository sends an event subscription response indicating successful operation.

### 8.x.3	Procedure on FL related event notification

This procedure describes the event notification procedure for the FL member availability. This can be triggered based on a change at the availability / capabilities of the candidate or selected FL members.

Pre-conditions:

- The subscription procedure in 8.x.2 was performed by the subscribing entity.

- The FL member has registered its capabilities / availability to the ML repository based on the support for FL registration capability as in clause 8.y.3.

Figure 8.x.3-1: Procedure for FL-related event notifications

1.	The ML repository detects a change related to the availability of a Candidate or Selected FL member, who has already registered at the ML repository. This detection can be in form of a trigger report from the candidate /selected FL member or a new / updated registration of a new FL member at the ML repository as described in clause 8.y.3 and 8.y.4.

2.	The ML repository generates events to be consumed by the subscribing entity(s), based on the trigger notification, or based on a received registration from a new FL member.

3.	For the generated event, the ML repository retrieves the list of corresponding subscriptions.

4.	The ML repository sends FL-related event notifications to all the subscribing entity(s) that have subscribed for the event matching the criteria. If a notification reception information is available as part of the subscribing entity event subscription, then the notification reception information is used by the ML repository to send event notifications to the subscribing entity.

5.	The subscribing entity sends an event notification acknowledgement to the ML repository.

### 8.x.4	List of FL-related Events

The list of the FL related events is provided in Table 8.x.4-1.

Table 8.x.4-1: List of FL related events

| Events | Events Description |
| --- | --- |
| Availability/Unavailability of candidate FL member(s) | The event type relates to the availability or unavailability indication of a candidate FL member or a set of members, which can be a registered entity as a candidate FL server or FL client. Such availability can be for a given service area, or VAL UE, or for a given ML model ID/profile. |
| Availability changes of selected FL member | The event type relates to the availability change of a selected FL member, which can be a selected entity as a FL server or FL client. Such availability change can be for a given service area, or VAL UE, or for a given ML model ID/profile. |
| FL service API update | The event relates to the API update related to the FL service. This can be due to the relocation of an FL member to a different DN/EDN, and the need to provide the updated northbound API. |
| Monitoring FL member load | This event relates to the monitoring of the computational load for the requested FL member. Such load can be the estimated (based on measurements) or expected/predicted. |
| Monitoring FL member energy | This event relates to the monitoring of the energy consumption for the requested FL member. Such energy consumption can be the estimated (based on measurements) or expected/predicted. |
| FL model monitoring info | Time schedule and latency for the FL training. Latency considering the target latency, i.e., when the FL model training shall be completed. |
| System related events | Alarm events providing fault information for the FL service. |
| Performance related events | Events related to system load conditions. |

### 8.x.5	Information flows

#### 8.x.5.1	General

The following information flows are specified for supporting the FL events subscription and events notification based on 8.x.2 and 8.x.3.

#### 8.x.5.2	FL-related event subscription request

Table 8.x.5.2-1 describes information elements for FL-related event subscription request from the Subscriber (AIMLE server or VAL server) to the ML repository.

Table 8.x.5.2-1: FL-related event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | The information to determine the identity of the subscribing entity. This can be the VAL server ID or AIME server ID. |
| ML model ID or profile | M | The identity of the ML model for which the subscription is needed. This identity can be also in terms of a profile for the ML model. |
| Target area information | M | The target service area for which the subscription applies. This area can be in form of topological area or set of DNAI/DNN or geographical coordinates for the coverage of the subscription. |
| Time of validity | M | The time validity for the subscription. |
| FL member type | O (NOTE) | The type of FL member which can be an FL server or FL client. |
| FL member ID | O (NOTE) | The ID of the entity serving as FL member (if known). This can be used for events where the FL member is known and possibly selected, and the availability needs to be checked. |
| List of UE IDs | O (NOTE) | The list of UEs for which the subscription for event applies. |
| Service ID | O (NOTE) | The VAL service or AIMLE service identity or profile for which the event subscription applies. |
| Event type | M | The event type requested, based on the List of FL related events (shown in Table 8.x.4-1). |
| Notification reception information | M | The information of the subscribing entity for receiving the notifications for the event. |
| ML model / vendor interoperability information | O | The ML model or vendor interoperability indication. |
| NOTE: At least one of these shall be present |  |  |

#### 8.x.5.3	FL-related event subscription response

Table 8.x.5.3-1 describes information elements for the FL-related event subscription response to the Subscriber (e.g. AIMLE server or the VAL server / EAS) from the ML repository.

Table 8.x.5.3-1: FL-related event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription operation |
| Subscription Identifier | M | The unique identifier for the event subscription. |

#### 8.x.5.4	FL related event notification

Table 8.x.5.4-1 describes information elements for the FL-related event notificaton from the ML repository to the Subscriber (e.g. AIMLE server or the VAL server, EAS).

Table 8.x.5.4-1: FL-related event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the event subscription |
| Event identifier | M | The unique identifier for the event. For the list of events, refer to the Table 8.x.4-1 with the list of FL related events. |
| Event related information | M | The event related information (e.g., time at which the event originated or is planned to be executed, location of event). |
| Event content | M | The content of the event information. |
| > List of FL member ID /address | O | The list of the FL members and their addresses. |
| >> FL member enter / leave | O | The indication of an FL member entering or leaving the available list. |
| >> FL member updated capability | O | The update of capabilities for the FL members. |
| >> FL member load status | O | The FL member load information if the event is about the load monitoring. |
| >> FL member energy status | O | The FL member energy consumption information if the FL event is about the energy monitoring. |
| >>Time availability of FL member | O | The time period for which the FL member is available. |
| >> Area of interest | O | The area of interest or exact / predicted location for which the event applies. |

#### 8.x.5.5	FL-related event notification acknowledgement

Table 8.x.5.5-1 describes information elements for the FL-related event notification acknowledgement from the subscriber (VAL server, AIMLE server, EAS) to the ML repository.

Table 8.x.5.5-1: FL-related event notification acknowledgement

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative acknowledgement of the notification reception. |

* * * Next Change * * * *