3GPP TSG-SA WG6 Meeting #62	S6-243178

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	China Mobile

Title:	pCR on Solution on configuration of application layer caller information on MMTel Enabler server

Spec:	3GPP TS 23.700-92 v1.1.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes the solution on configuration of application layer caller information on MMTel Enabler server.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-92 v1.1.0.

* * * First Change * * * *

8.x	Solution #x: configuration of application layer caller information on MMTel Enabler server

8.x.1	Description

This solution provides the procedure on configuration of application layer caller information on MMTel Enabler server. The configured application layer caller information will be shown to the callee by using the procedure in solution#10.

NOTE:	In this release,application layer caller information only applies to the call between MMTel call between application with IMS capability and application without IMS capability, i.e. the mmtel call which goes through the MMTel Enabler Server.

Editor's note:	 Whether the application layer caller information of the application without IMS capability should be configured in HSS repository data via the NEF or not is FFS and may require coordination with SA2.

8.4.2	Procedures

The procedure of onfiguration of application layer caller information on MMTel Enabler server is illustrated in figure 8.x.2-1.

Pre-conditions:

1)	The Application Server has connected to the serving network successfully.

2).	Both the Application Serverand MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.x.2-1: configuration of application layer caller information

1.	The Application Server sends a application layer caller information configuration request to the MMTel Enabler Server. The request message includes information elements as specified in Table 8.x.2-1.

Table 8.4.2-1: Information elements in application layer caller information configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Caller ID | M | The identity of the Application which the application layer caller information |
| Security credentials | O | Security information required by the eMMTel Enabler Server. |
| Application layer caller information data | M | The application layer information of this caller. The application layer information data may include the detailed information elements defined in OMA CAB Personal Contact Card (PCC) document [CAB_TS] or in Rich Call Data (RCD) defined in draft-ietf-sipcore-callinfo-rcd-10 [RCD10]. |

2.	Upon receiving the request, the MMTel Enabler Server initiates authentication procedures with the Requester Identity and Caller ID. The MMTel Enabler Server also authorizes the Application Server if needed. If the authentication and authorization are succeed, the MMTel Enabler Server creates and stores the application layer caller information.

3.	The MMTel Enabler Server sends a application layer caller information configuration response to the Application Server. The response message includes information elements as specified in Table 8.x.2-2.

Table 8.4.2-3: Information elements for application layer caller information configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Configuration result | M | Indicates the success or failure of the configuration of this application layer caller information. |
| Reason | O | Indicates the reason for failure. |

8.4.3	Impacts on Existing Nodes and Functionality

This solution detailed specified the procedures of configuration of application layer caller information. If the application layer caller information is not needed to be configured in HSS repository data via the NEF, there is no impacts on existing  nodes and functionality. Otherwise, the capabilities provided by HSS, e.g. capabilities described in Conclusions of KI #4 of 3GPP TR 23.700-77 [19] are needed and coordination with SA2 is needed.