3GPP TSG-SA WG6 Meeting #62	S6-243181

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	China Mobile

Title:	Remove ENs in 8.10.2

Spec:	3GPP TS 23.700-92 v1.1.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR is proposed to solve ENs in clause 8.10.2. This pCR also adds some clarification on the solution#10.

2. Reason for Change

Solve ENs in clause 8.10.2.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-92 v1.1.0.

* * * First Change * * * *

### 8.9.3	Impacts on Existing Nodes and Functionality

In this solution, the procedures use the basic capability exposure by IMS core which is under studied by SA2.

8.10	Solution #10: providing application layer caller information to callee

8.10.1	Description

This solution resolves Key Issue #4 about support of providing application layer caller information to callee. This solution is a application solution which will not impact the IMS/5GC.

NOTE1:	In this release, this solution only applies to the call between MMTel call between application with IMS capability and application without IMS capability, i.e. the mmtel call which goes through the MMTel Enabler Server. The IMS user will see the application layer caller information of the application without IMS capability by using this solution.

NOTE 2	In this release, this solution only works for a single PLMN operator domain or the PLMN operator domains which served by the same MMTel Enabler server. This solution does not apply for interworking scenarios.

Editor's note:	coordination with SA2 is required on how IMS DC supports a solution for obtaining Caller information by DCMTSI client using IMS DC procedures.

8.10.2	Procedures

Upon receiving an incoming mmtel session, the eMMTel Client in the UE sends an application request messages to eMMTel Enabler Server to fetch the caller information from eMMTel Enabler Server via eMMTel-1 reference point. The procedure is illustrated in Figure 8.10.2-1.

Pre-condition:

a.	The application layer caller information of the application without IMS capability hss been configured to the MMTel Enabler Server.

Figure 8.10.2-1: providing application layer caller information to callee

1.	The UE receives an incoming mmtel session.

2.	The eMMTel Client in the UE decides to fetch the application layer caller information based on the configuration and sends a Get application layer caller information request to eMMTel Enabler Server. The request message includes information elements as specified in Table 8.10.2-1.

NOTE 1:	Whether fetching the application layer caller information is needed can be decided by default or based on configuration provided by the MMTel service provider by using the enhancement of e.g. OMA-DM with the management objects specified in 3GPP TS 24.275 [24275] or procedures specified in GSMA PRD RCC.15 [RCC15].

Table 8.10.2-1: Information elements in Get application layer caller information request

| Information element | Status | Description |
| --- | --- | --- |
| Caller ID (see NOTE) | M | The identifier of the call originator |
| Callee ID | M | The identifier of the call terminator, i.e. the UE. |
| Additional Callee information | O | Additional information of callee, e.g. the location of callee etc. |
| NOTE: If the caller is an application, the Caller ID is the routable ID, which is converted by eMMTel Enabler server in the call between application and UE, in the IMS domain. |  |  |

3.	The eMMTel Enabler Server checks whether the callee is authorized to get the application layer caller information.

4.	The eMMTel Enabler Server finds the  application layer caller information based on the Caller ID. It includes the application layer caller information in the Get application layer caller information response and sends it to the eMMTel Client in UE. The response message includes information elements as specified in Table 8.10.2-2.

Table 8.10.2-2: Information elements for Get application layer caller information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of Get application layer caller information. |
| Failure Cause (see NOTE 1) | O | The reason for failure, e.g. the application layer caller information is not existing. |
| application layer caller information | O | The application layer caller information of the caller, e.g. the caller is verified by MNO and belonged to a specific organization, the verified business card of the organization, location of the organization, etc. The detailed information of this IE is implementation specific and out of scope of the present document. |
| NOTE 1: This IE shall only be present when the Get application layer caller information is failure. NOTE 2: This IE shall only be present when the Get application layer caller information is success. |  |  |

5.	The eMMTel Client in the UE handles the application layer caller information.

NOTE:	E.g. if the Get application layer caller information is success, the eMMTel Client in the UE may gives the information to the DCMTSI client and display it to the user, if the Get application layer caller information is failure, the eMMTel Client in the UE may request the DCMTSI client to ignore the incoming mmtel session..The handling of application layer caller information on UE is out of scope of the present document.

8.10.3	Impacts on Existing Nodes and Functionality

This solution depends on the interaction between eMMTel Client in the UE and the DCMTSI client in the same UE. This interaction is internal interaction in the dialler application on the UE and will not impact the existing 5GC/IMS.

Editor's note:	whether DCMTSI client can be utilized for the eMMTel operations is FFS.