3GPP TSG-SA WG6 Meeting #62	S6-243183

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

Source:	China Mobile

Title:	Remove EN2 in 7.2.6.1.2

Spec:	3GPP TS 23.700-01 v1.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR is proposed to solve EN2 in 7.2.6.1.2.

2. Reason for Change

Solve EN2 in 7.2.6.1.2.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-01 v1.1.0.

* * * First Change * * * *

7.2.6	Solution #AE6: Enhancing store and forward mechanism of MSGin5G service

7.2.6.1	Solution description

As specified in clause 8.3.6 (of 3GPP TS 23.554 [14]), SA6 defined MSGin5G service supports application enabler level store and forward mechanism. As per the defined mechanism, the MSGin5G server stores the message if recipient UE is not available. Further, as specified in KI #7, SA2 is studying S&F events like expected delivery time (including Feeder-Link Availability to deliver data, Service-Link Availability to deliver data, etc).

There can be scenario where recipient UE is connected to satellite in store and forward mode as shown in Figure 7.2.6.1.1-1.

Figure 7.2.6.1.1-1: Store and Forward scenario for MSGin5G service

In such case, MSGin5G server can subscribe to S&F events to know when feeder link to recipient UE is available and also what is the expected time of the delivery.

Based on this information, the message originator can decide to do one of the following:

1)	If the expected delivery time is after the message expiry time, and the message is important to be delivered according to the originating user, the message originator can request the MSGin5G server to increase the store and forward parameters; Otherwise, the message originator can request the MSGin5G server to discard the stored message.

7.2.6.1.2	Enhancement to MSGin5G Store and Forward procedure

Following enhancement are done in clause 8.3.6 of 3GPP TS 23.554 [14]:

-	The MSGin5G server subscribes to receive S&F events:

-	If the recipient MSGin5G Client knows that satellite connection may be used, e.g. based on the configuration, it may include the satellite connection used indicator in its registration request. The related registration enhancement is specified in clause 7.2.6.1.4. In this case, MSGin5G server subscribes the S&F events after the procedure of MSGin5G Client registration.

-	Otherwise, for each received Message towards to the recipient MSGin5G Client which is connected to satellite, after step-2 in clause 8.3.6 of 3GPP TS 23.554 [14], the MSGin5G server subscribes to receive S&F events (e.g. feeder link availability, expected delivery time) as per clause 6.38.2 of 3GPP TR 23.700-29 [15].

-	Upon receiving S&F events to the MSGin5G server from the NEF, the MSGin5G server sends a delivery status message to the MSGin5G client. The message includes the message ID, recipient UE identity and expected delivery time to deliver the message. The user of the UE or the MSGin5G client considers this information and may decide to update the store and forward parameters (as specified in clause 7.2.6.1.3) or may decide to discard the stored message (as specified in clause 7.2.6.1.4). If neither the update the store and forward parameters request nor the MSGin5G client discard the stored message request is received by the MSGin5G server, the MSGIn5G server handled the MSGin5G message as per the procedures specified in clause 8.3.6 of 3GPP TS 23.554 [14].

Editor's note:	The solution depends on progress of SA2 study.

7.2.6.1.2	Procedure for MSGin5G client to update the store and forward parameters

Figure 7.2.6.1.2-1 shows the procedure for MSGin5G client to update the store and forward parameters.

Pre-conditions:

1.	The MSGin5G Client has originated a message to a recipient UE and it is stored to the MSGin5G Server as the recipient UE is not available (as specified in clause 8.3.6 of 3GPP TS 23.554 [14]).

2.	User has received notification from the MSGin5G server about Feeder Link Available event and expected delivery time.

Figure 7.2.6.1.2-1: MSGin5G update stored message parameters

0.	The user or the application client decides to update the store and forward parameters based on the received expected delivery time event about the recipient UE. The application client informs MSGin5G client to update the store and forward parameters.

1.	The MSGin5G client sends a request message to update the store and forward parameters to the MSGin5G server. The message includes the message ID for which store and forward parameters needs to be updated, and new/updated store and forward parameters (e.g. Message expiration time) as specified in Table 8.3.2-2 3GPP TS 23.554 [14].

2.	Upon receiving the request, the MSGin5G server checks whether the message is stored based on received message ID. If message is stored, then it updates the store and forward parameters and sends response back to the MSGin5G client. If the MSGin5G client is not authorized to update the parameters or the message does not exists, then the response message includes failure cause.

7.2.6.1.3	Procedure for MSGin5G client to discard the stored message

Figure 7.2.6.1.3-1 shows the procedure for MSGin5G client to discard the stored messages.

Pre-conditions:

1.	The MSGin5G Client has originated a message to a recipient UE and it is stored to the MSGin5G Server as the recipient UE is not available (as specified in clause 8.3.6 of 3GPP TS 23.554 [14]).

2.	User has received notification from the MSGin5G server about Feeder Link Available event and expected delivery time of the recipient UE.

Figure 7.2.6.1.3-1: MSGin5G discard stored message

0.	The user or the application client decides to discard the stored message based on the received expected delivery time event about the recipient UE. The application client informs MSGin5G client discard the stored message.

1.	The MSGin5G client sends a request message to discard the stored message to the MSGin5G server. The message includes the message ID which needs to be discarded. The request message also includes reason for discarding the stored message.

2.	Upon receiving the request, the MSGin5G server checks whether the message is stored based on received message ID. If message is stored, then it discards the stored message and sends response back to the MSGin5G client. If the MSGin5G client is not authorized to update the parameters or the message does not exists, then the response message includes failure cause.

7.2.6.1.4	Enhancement to MSGin5G UE Registration

If the recipient MSGin5G Client knows that satellite connection may be used, e.g. based on the configuration, it may include the satellite connection used indicator in its registration request. This indicator is included in the MSGin5G Client Communication Availability specified in Table 8.2.1-3 of 3GPP TS 23.554 [14].

Editor's note:	Whether the enhancement of MSGin5G UE Registration needs to be applied to 3GPP TS 23.554 [14] is based on the the normative work.

Table 8.2.1-3 of 3GPP TS 23.554 [14]: MSGin5G Client Communication Availability

| Information element | Status | Description |
| --- | --- | --- |
| Scheduled communication time | M | Time when the UE becomes available for communication. |
| Communication duration time | M | Duration time of periodic communication. |
| Periodic communication indicator | O | Identifies whether the client communicates periodically or not, e.g., on demand. |
| Periodic communication interval | O | Interval Time of periodic communication. This IE is mandatory if the Periodic communication indicator indicates periodic communications. |
| Data size indication | O | Indicates the expected data size to be exchanged during the communication duration. |
| Store and forward option | O | Indicates opting out of store and forward services for incoming MSGin5G requests. The MSGin5G Server uses the information to determine whether Store and Forward procedure applies as specified in clause 8.3.6. |
| Satellite connection used | O | Indicates whether satellite connection will be used by this MSGin5G Client. |

7.2.6.2	Architecture Impacts

No architecture impact is identified for MSGin5G architecture as defined in 3GPP TS 23.554 [14].

7.2.6.3	Corresponding APIs

Table 7.2.6.3-1 specifies information elements for MSGin5G update stored message parameter request message.

Table 7.2.6.3-1: MSGin5G update stored message parameter request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client or the sending Application Server. |
| Message ID | M | Unique identifier of the stored message for which parameters needs to be updated |
| Store and forward parameters | M | Updated parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2 of 3GPP TS 23.554 [14]. |

Table 7.2.6.3-2 specifies information elements for MSGin5G update stored message parameter response message.

Table 7.2.6.3-2: MSGin5G update stored message parameter response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indication if the procedure is success or failure |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the result is Failure. |  |  |

Table 7.2.6.3-3 specifies information elements for MSGin5G discard stored message request message.

Table 7.2.6.3-3: MSGin5G discard stored message request

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client or the sending Application Server. |
| Message ID | M | Unique identifier of the stored message for which parameters needs to be updated |

Table 7.2.6.3-4 specifies information elements for MSGin5G discard stored message response message.

Table 7.2.6.3-4: MSGin5G discard stored message response

| Information element | Status | Description |
| --- | --- | --- |
| result | M | Indication if the procedure is success or failure |
| Failure Cause (see NOTE) | O | The reason for failure |
| NOTE: This IE shall only be present when the result is Failure. |  |  |

7.2.6.4	Solution evaluation

This solution addresses key issue #7 by utilizing store and forward events from SA2, and deciding whether to update the stored and forward parameters or the MSGin5G message or discard the stored message. Solution #AE6 will not be considered for KI #7 in normative phase due to SA2 only support S&F for EPS in Rel-19.

* * * Next Change * * * *

10.4.6	Key issue #7:

Solution #AE4, Solution #AE6 and Solution #AE8 addresses key issue #7.

Solution #AE4, Solution #AE6 and Solution #AE8 resolves open issue to obtain the S&F events information and to utilize S&F events information to optimise application enablement behaviour.

Solution #AE4 proposes how SEAL (NRM, SEALDD) server obtains UE S&F events information and then how such information is used by the SEAL (NRM, SEALDD) server to minimize the service interruption.

Solution #AE6 proposes to utilize S&F events for MSGin5G service over 5G and optimizes the behaviour by deciding to update or discard already stored message. Solution #AE6 will not be considered for KI #7 in normative phase since SA2 supports S&F only for EPC in Rel-19.

Solution #AE8 proposes how SEAL (SEALDD) server obtains UE S&F events information from EPC and then how such information is used by the SEAL (SEALDD) server to minimize the service interruption.

Solution #AE4 and Solution #AE8 can be considered for normative work which are for considering different approaches on how SEAL (NRM, SEALDD) server obtains UE S&F events information and then how such information is used by the SEAL (SEALDD) server to minimize the service interruption.