3GPP TSG-SA WG6 Meeting #62	S6-243184

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0214 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23554 Remove ENs in 8.7.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An EN in clause 8.7.4.2 is Editor’s note: The detailed procedure of Controlling AS registers the specific role in Application Server Registration procedure is FFS. The procedures of Controlling AS registers the specific role in Application Server Registration are already specified in clause 8.2.5, 8.2.6 and 9.1.2.3, therefore the EN in clause 8.7.4.2 can be removed without more action. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed remove EN clause 8.7.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN exists in the TS.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.7.4.2	Message delivery from UE to group

Figure 8.7.4.2-1 shows the MSGin5G Group messaging procedure in which MSGin5G Client or Application Server sends a message to a group. One or multiple Controlling AS may be present in this group. The Controlling AS is not considered as a participant of this group.

Pre-conditions:

1.	An MSGin5G Group is created by following group management SEAL service procedures as specified in 3GPP TS 23.434 [5].

2.	All participants in the MSGin5G Group may get the Group information i.e. the Group Service ID.

3.	The MSGin5G Server has a copy of the group profile with all the group members by using Group information query specified in 3GPP TS 23.434 [5].

Figure 8.7.4.2-1: Group messaging in MSGin5G Service

1.	A MSGin5G message request is sent to a group as specified in clause 8.3.2. This MSGin5G message request includes the information elements from Table 8.3.2-1. This message may be sent by:

a)	The MSGin5G Client-1; or

b)	The Application Server-1.

2.	Upon receiving the MSGin5G message request to send the group message:

a)	if the MSGin5G Client-1 or Application Server-1 is authorized to send the group message, step 3 is to be processed by the MSGin5G Server;

b)	otherwise the MSGin5G Server sends MSGin5G message response with delivery status set as Reject to the  MSGin5G Client-1 or the Application Server-1, i.e. the message is rejected by the MSGin5G service, as specified in clause 8.3.2.

3.	The MSGin5G Server checks the whether one or multiple Controlling present in this MSGin5G group by checking the stored Expected MSGin5G group list to be controlled included in the Application Server registration,

a)	if a Controlling AS(s) is present, the MSGin5G Server sends the message to the Controlling AS (e.g. to log application specific message or for analytics) based on Group service ID present in the received MSGin5G message request , i.e. this group is included in one or multiple Application Server’s Expected MSGin5G group list to be controlled , and holds the message on MSGin5G Server until the Controlling AS agreed to send this message, i.e. after step 4

i)	Upon receiving the MSGin5G message request, the Controlling ASApplication Server checks whether the message is allowed to be sent based on the application specific logic. If the message is not allowed to be sent, the Controlling AS sends MSGin5G message response with delivery status set as Reject to the MSGin5G Server.

The MSGin5G Server sends the MSGin5G message response with delivery status set as reject to the MSGin5G Client-1, or to the Application Server-1 as specified in clause 8.3.2. The information elements defined in Table 8.3.2-3 are included in the response. Following procedures will be skipped, i.e. the message is rejected by the application.

ii)	Otherwise, go to step 4.

b)	if a Controlling AS(s) is not present, the MSGin5G Server skips step 4 and continues the procedure from step 5.

4.	The Controlling AS finishes to process the application specific logic of the message, e.g. log application specific message, or analyzes the information in the message, and sends the MSGin5G message request back to the MSGin5G Server based on different application specific logic of the Controlling AS.

5.	The MSGin5G Server resolves the group ID to determine the members of that group, based on the information from the group management server as specified in 3GPP TS 23.434 [5]. The MSGin5G Server sends the message to all participants of the group by their UE Service ID. The MSGin5G message request includes the information elements from Table 8.3.3-1. The MSGin5G Server routes, using the procedures in clause 8.3.3, the message to:

a)	a MSGin5G UE;

b)	a Legacy 3GPP UE;

c)	a Non-3GPP UE ; or

d)	an Application Server.

NOTE 1:	If the originating UE is member of the group, the originating UE is not included as recipient of the group message.

NOTE 2:	Steps 5 a), 5 b), 5 c) and 5d) can happen in parallel and in any order.

6.	Upon receiving the group message, if message delivery status report is requested and if supported by target message client, the MSGin5G Client or Legacy 3GPP UE or Non-3GPP UE or Application Server sends the message delivery status report to originator MSGin5G Client 1 as specified in clause 8.2.4 and 8.3.5.