3GPP TSG-SA WG6 Meeting #62	S6-243190

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0308 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add dynamic Geofencing procedure and information flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the TR 23.700-72 v1.1.1, the KI#1 for Geofencing has been concluded as follows: Solution #2 and Solution#4 addressing the KI#1 can be considered in the normative work to realize the different functions. The related function and procedure will be specified in 3GPP TS 23.434 to support the two solutions, and the detailed APIs and information flows can be discussed in the normative phase. So it’s needed to specify the functions, procedures and information flows for Geofencing in TS 23.434 for the normative work. This CR focuses on the Sol#4 and tries to add the related dynamic Geofencing procedure and information flow in TS 23.434. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add dynamic Geofencing function in current procedure of Monitoring Location Deviation procedure. - Add Geofencing related parameters in current information flow of Monitor Location Subscription Request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dynamic Geofencing will not be realized for the vertical application services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.11.2a(new), 9.3.2.11, 9.3.2.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.11.2a	Monitoring Location Deviation procedure for Geofencing service

If the VAL server requests the monitoring location deviation for the Geofencing service, the procedure of 9.3.11.2 can be utilized to support it.

Figure 9.3.11.2a-1 describes the procedure for monitoring the VAL UE's location in a Geofencing defined location area.

Figure 9.3.11.2a-1: Monitoring VAL UE's location in a Geofencing location area

1.	Same with step 1 of clause 9.3.11.2. The difference is for AoI related to the target UE(s), a reference UE and its proximity range are included. Besides, the VAL server may include a temporal condition(s) (e.g. time of day and day of week) and/or a spatial condition(s) (e.g. location area(s)) for the reference UE. The target UE information under monitoring can be a list of UEs.

2.	The LM server requests 3GPP CN (e.g. NEF service for location, access type and roaming status monitoring as specified in 3GPP TS 23.502 [11]) to monitor the reference UE location change, access type change and roaming status change, in order to determine spatial dynamicity in AoI related to the target UE(s). If spatial condition is received, the LM server also requests 3GPP CN (i.e. NEF service for AoI monitoring as specified in 3GPP TS 23.502 [11]) to know whether the reference UE is in the VAL server's predetermined area. The LM server will process the Geofencing monitoring considering the received temporal, connectivity and/or spatial condition(s) for the reference UE.

NOTE 1:	For temporal condition with time of day and day of week, the LM server supervises the time since it is not supported by UE location service in CN.

3.	In order to know UE status (e.g. purged or detached) in 3GPP CN, the LM server requests 3GPP CN (i.e. NEF service for loss of connectivity as specified in 3GPP TS 23.502 [11]) for both target UE(s) and reference UE.

4a-4c.	Same as step 2, 3 and 4 of clause 9.3.11.2. If multiple target UEs are received from the VAL server, step 4a-4c are performed for each target UE.

5.	Same as step 5 of clause 9.3.11.2.

6a. Same as step 6a of clause 9.3.11.2.

6b.	If the obtained location information is matching as a result of step 6a, the LM server checks if the VAL UE's current location is within the proximity range of the reference UE.

7.	Same as step 7 of clause 9.3.11.2.

8.	If the location information is matching as a result of step 6a, and the target UE is not within the proximity range of the reference UE, the LM server considers the target UE is outside from the AoI related to the target UE(s) and notifies the VAL server that the target UE's current location is outside of AoI and target VAL UE ID in "Notify Absence" message.

9.	If the location information is matching as a result of step 6a, and the target UE is within the proximity range of the reference UE, then the LM server sends "Notify Presence" message to the VAL server periodically, according to the "Notify_Interval" value specified in clause 9.3.2.11, indicating that the target UE is within the AoI, along with the target UE's current location information.

10.	If the LM server is notified by the 3GPP CN about loss of connectivity for the reference UE or target UE(s), the LM server sends "Notify Unknown" message to the VAL server indicating that the LM server does not know whether the target UE is "Present" or “Absence", therefore the Geofencing monitoring for the impacted target UE(s) is suspended.

NOTE 2:	Upon "Notify Unknown", the VAL server can remind end user to check status for the reference/target UE, which is out of 3GPP scope. If the reference UE loses connectivity, the LM server can stop event monitoring for the target UE. If the target UE loses connectivity, the LM server can stop event monitoring for the reference UE. When the connectivity issue is solved (e.g. UE registered in 3GPP CN), the Geofencing monitoring process resumes (with resumed event monitoring for reference UE or target UE) and the VAL server will be notified accordingly with "Present" or “Absence".

NOTE 3:	Notifications in step 7-10 can be subject to explicit notification indication from the VAL server, for instance, the VAL server can be interested to know only "Notify Absence" and "Notify Unknown" in Geofencing.

* * * Next Change * * * *

9.3.2.11	Monitor Location Subscription Request

Table 9.3.2.11-1 describes the information flow from the VAL server to the LM server for monitoring the location deviation of the VAL UE(s) or for updating an existing monitor location subscription.

Table 9.3.2.11-1: Monitor Location Subscription Request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identifier of the VAL users or VAL UE whose location monitoring is requested to be monitored in a given location (see NOTE). |
| Area of Interest | M | Geographic area location information where the VAL server wishes to monitor the VAL UE's location adherence. This could be geographical co-ordinates or VAL service area identified by the VAL service area ID. For Geofencing service, a reference UE and its proximity range shall are included. And a temporal condition(s) and/or a spatial condition(s) for the reference UE may also be contained. |
| Notify Interval | M | Periodic time interval in which the LM server needs to notify the VAL UE's location information to the VAL server. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL UE's location. |
| Timeout | O | A timeout period when subscription response is not received. |
| NOTE: This information element shall not be updated via monitor location subscription update procedure in clause 9.3.2.12a. |  |  |

* * * Next Change * * * *

9.3.2.13	Notify Location Monitoring Event

Table 9.3.2.13-1 describes the information flow from LM server to the VAL server for notification of location monitoring events.

Table 9.3.2.13-1: Notify Location Monitoring Event message

| Information element | Status | Description |
| --- | --- | --- |
| Event | M | Information of the event to be reported. The event shall be one of the following: "Notify_Mismatch_Location" – When the location information of the VAL UE, from the location management client and the core network do not match, or when the LM server determined area of interest result based on LM client location and the core network reported area of interest result do not match. "Notify_Absence" – When the VAL UE's current location is deviating from the VAL server's area of interest information. "Notify_Presence" – When the VAL UE's current location is within the VAL server's area of interest information. "Notify_Unknow" – When the LM server does not know whether the target UE is "Present" or “Absence" within the VAL server's area of interest information |
| Identity | M | Identifier of the VAL UE whose location information is reported. |
| Location | M | Current location of the VAL UE. |

* * * End of Changes * * * *